EQ-CH146A

| Description | Specifications |
| --- | --- |
| New Thickness/MeasuredAt | 4.972-4.980" (126.289-126.492 MM) |
| Minimum Thickness | 4.964" (126.086 MM) |
| Warp Spec. | .004" OVERALL (.102 MM) |
| Chamber Volume (cc) |  |
| Surface Finish |  |
| OHC Camshaft Bore Diameter | 1.1024-1.1033" (28.001-28.024 MM) |
| Injector Tip Protrusion |  |
| Lifter Bore Diameter | 1.3380-1.3390" (33.985-34.011 MM) |
| Comments | * 1.654-1.674" REPORTED INSTALLED HEIGHT. STEM END REFACE LIMIT .008 I & E |

Valve

Collapse

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 1.654-1.674" (42.012-42.520 MM) * | 1.654-1.674" (42.012-42.520 MM) * |
| Stem Diameter | .2742-.2748" (6.965-6.98 MM) | .2734-.2740" (6.944-6.96 MM) |
| Total Length | 3.9772-4.0008" (101.021-101.62 MM) | 3.8787-3.9260" (98.519-99.72 MM) |
| Head Diameter | 1.437-1.445" (36.5-36.703 MM) | 1.228-1.236" (31.191-31.394 MM) |
| Angle | 45.5° | 45.5° |
| Margin | .020" MIN. (.508 MM) | .020" MIN. (.508 MM) |
| Recession |  |  |
| Valve Guide Information | Intake | Exhaust |
| Guide Height | .524-.547" (13.31-13.894 MM) | .524-.547" (13.31-13.894 MM) |
| Guide Clearance | .0008-.0021" (.02-.053 MM) | .0016-.0029" (.041-.074 MM) |
| Guide OD |  |  |
| Valve Seal Type | POSITIVE | POSITIVE |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .058-.064" (1.473-1.626 MM) | .071-.079" (1.803-2.007 MM) |
| Seat Angle | 45° | 45° |
| Seat Outside Diameter |  |  |
| Seat Inside Diameter |  |  |
| Seat Insert Height |  |  |
| Seat Insert Bore Diameter | 1.4961-1.4967" (38.001-38.016 MM) | 1.2874-1.2880" (32.700-32.715 MM) |
| Seat Insert Bore Depth |  |  |

Valve Spring

Collapse

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length | 1.756" (44.602 MM) or 1.980" (50.3mm) | 1.756" (44.602 MM) OR 1.980" (50.3MM) |
| Installed Height |  |  |
| Open Test Press | 123# @ 1.024 Or 88-94 #s @ 1.150" | 123# @ 1.024 Or 88-94 #s @ 1.150" |
| Close Test Press |  |  |
| Inner Spring | Intake | Exhaust |
| Free Length |  |  |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |

Rocker Shaft

Collapse

| Description | Specifications |
| --- | --- |
| Shaft Diameter |  |
| Bore Diameter |  |
| Oil Clearance |  |
| Rock Arm Ratio |  |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash | .012-.015" Hot (.304-.381 MM) for checking, .0146" Cold (.370 MM) for ADJUSTING |
| Exhaust Valve Lash | .013-.016" Hot (.330-.406 MM) for checking, .0146" Cold (.370 MM) for ADJUSTING |
| Lash Adjustment Method | ADJUSTING SHIMS |
| Precup Torque Spec. |  |
| Cyl Head Torque | 22, 58 FT/LBS, LOOSEN, 22, 54-61 FT/LBS |
| Camshaft Cap Torque | 1ST 16.8 INCH/LBS, 2ND 80.4-104.4 INCH/LBS |