EQ-CH146E

Cylinder Head

Casting

Collapse

| Description | Specifications |
| --- | --- |
| New Thickness/MeasuredAt | 5.0820" (129.083 MM) NEW |
| Minimum Thickness | 5.0700" (128.900 MM) |
| Warp Spec. | .004" (.100 MM) |
| Chamber Volume (cc) |  |
| Surface Finish |  |
| OHC Camshaft Bore Diameter |  |
| Injector Tip Protrusion |  |
| Lifter Bore Diameter | .4730-.4739" (12.013-12.037 MM) |
| Comments |  |

Valve

Collapse

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 1.595-1.605" (40.513-40.767 MM) | 1.595-1.605" (40.513-40.767 MM) |
| Stem Diameter | .2344-.2355" (5.955-5.970 MM) | .2337-.2343" (5.935-5.950 MM) |
| Total Length |  |  |
| Head Diameter | 1.3760-1.4154" (34.950-35.250 MM) | 1.1791-1.1909" (29.950-30.250 MM) |
| Angle | 45° | 45° |
| Margin | .041" (1.053 MM) | .044" (1.117 MM) |
| Recession |  |  |
| Valve Guide Information | Intake | Exhaust |
| Guide Height |  |  |
| Guide Clearance | .0012-.0022" (.030-.057 MM) | .0020-.0026" (.050-.077 MM) |
| Guide OD |  |  |
| Valve Seal Type |  |  |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .0472" (1.200 MM) | .0630" (1.600 MM) |
| Seat Angle | 45° | 45° |
| Seat Outside Diameter |  |  |
| Seat Inside Diameter |  |  |
| Seat Insert Height |  |  |
| Seat Insert Bore Diameter |  |  |
| Seat Insert Bore Depth |  |  |

Valve Spring

Collapse

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length | 1.6299-1.7402" (41.400-44.200 MM) | 1.6299-1.7402" (41.400-44.200 MM) |
| Installed Height | 1.2795" | 1.2795" |
| Open Test Press | 118-129 LBS @ .8858" | 118-129 LBS @ .8858" |
| Close Test Press | 55-61 LBS @ 1.2795" | 55-61 LBS @ 1.2795" |
| Inner Spring | Intake | Exhaust |
| Free Length |  |  |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |

Rocker Shaft

Collapse

| Description | Specifications |
| --- | --- |
| Shaft Diameter |  |
| Bore Diameter |  |
| Oil Clearance |  |
| Rock Arm Ratio | 1.68:1 |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash | HYDRAULIC |
| Exhaust Valve Lash | HYDRAULIC |
| Lash Adjustment Method | NONE |
| Precup Torque Spec. |  |
| Cyl Head Torque | 22 FT/LBS + 155° |
| Camshaft Cap Torque | TIGHTEN IN THREE STEPS TO 89 INCH/LBS |