Cylinder Head

| Description | Specifications |
| --- | --- |
| New Thickness/Measured At | 3.539-3.547" (89.900-90.100 MM) |
| Minimum Thickness | 3.5230" (89.484 MM) |
| Warp Spec. | .008" OVERALL (.203 MM) |
| Chamber Volume (cc) | 75.2 CC |
| Surface Finish |  |
| OHC Camshaft Bore Diameter | 1.3390" (34.011 MM) |
| Injector Tip Protrusion |  |
| Lifter Bore Diameter |  |
| Comments | * VALVE SPG FREE LENGTH FROM 5/84->90 = 1.961" AUX VALVE ASSEMBLY PART #MD009440 |

Valve

Collapse

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 1.690-1.710" (42.926-43.434 MM) | 1.680-1.700" (42.672-43.18 MM) |
| Stem Diameter | .3150" (8.001 MM) | .3150" (8.001 MM) |
| Total Length | 4.330" (109.982 MM) | 4.250" (107.95 MM) |
| Head Diameter | 1.800" (45.72 MM) | 1.500" (38.1 MM) |
| Angle | 45° | 45° |
| Margin | .028-.047" (.711-1.194 MM) | .039-.079" (.991-2.007 MM) |
| Recession |  |  |
| Valve Guide Information | Intake | Exhaust |
| Guide Height | .460" (11.684 MM) | .460" (11.684 MM) |
| Guide Clearance | .0012-.0024" (.03-.061 MM) | .0020-.0035" (.051-.089 MM) |
| Guide OD |  |  |
| Valve Seal Type | POSITIVE | POSITIVE |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .035-.051" (.889-1.295 MM) | .035-.051" (.889-1.295 MM) |
| Seat Angle | 45° | 45° |
| Seat Outside Diameter |  |  |
| Seat Inside Diameter |  |  |
| Seat Insert Height |  |  |
| Seat Insert Bore Diameter |  |  |
| Seat Insert Bore Depth |  |  |

Valve Spring

Collapse

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length | '79-4/84:1.875" (2006.6 MM) * | '79-4/84:1.875" (2006.6 MM) * |
| Installed Height | 1.590-1.629" (40.386-41.377 MM) | 1.590-1.629" (40.386-41.377 MM) |
| Open Test Press |  |  |
| Close Test Press | 61# @ 1.591 | 61# @ 1.591 |
| Inner Spring | Intake | Exhaust |
| Free Length |  |  |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |

Rocker Shaft

Collapse

| Description | Specifications |
| --- | --- |
| Shaft Diameter |  |
| Bore Diameter |  |
| Oil Clearance |  |
| Rock Arm Ratio |  |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash | .006" HOT (.152 MM) |
| Exhaust Valve Lash | .010" HOT & AUX (.254 MM) |
| Lash Adjustment Method | AUX VLV ASSMBLY PART #MD009440 |
| Precup Torque Spec. |  |
| Cyl Head Torque | 65-72 FT/LBS COLD, 73-79 FT/LBS HOT |