Cylinder Head

Casting

Collapse

| Description | Specifications |
| --- | --- |
| New Thickness/Measured At | 3.6673-3.6712" (93.149-93.248 MM) |
| Minimum Thickness | 3.6513" (92.743 MM) |
| Warp Spec. | .008" OVERALL (.203 MM) |
| Chamber Volume (cc) |  |
| Surface Finish |  |
| OHC Camshaft Bore Diameter | 1.3420-1.3437" (34.087-34.13 MM) |
| Injector Tip Protrusion |  |
| Lifter Bore Diameter |  |
| Comments |  |

Valve

Collapse

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 1.7200-1.7500" (43.688-44.45 MM) | 1.7200-1.7500" (43.688-44.45 MM) |
| Stem Diameter | .3129-.3134" (7.948-7.96 MM) | .3118-.3124" (7.92-7.935 MM) |
| Total Length |  |  |
| Head Diameter | 1.740" (44.196 MM) | 1.450" (36.83 MM) |
| Angle | 45° | 45° |
| Margin | .032" MIN. (.813 MM) | .040" MIN. (1.016 MM) |
| Recession | .040-.067" V/REC (1.016-1.702 MM) | .040-.067" V/REC (1.016-1.702 MM) |
| Valve Guide Information | Intake | Exhaust |
| Guide Height | .634-.642" (16.104-16.307 MM) | .634-.642" (16.104-16.307 MM) |
| Guide Clearance | .0009-.0022" (.023-.056 MM) | .0015-.0028" (.038-.071 MM) |
| Guide OD |  |  |
| Valve Seal Type | POSITIVE | POSITIVE |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .063-.077" (1.6-1.956 MM) | .063-.077" (1.6-1.956 MM) |
| Seat Angle | 45° | 45° |
| Seat Outside Diameter |  |  |
| Seat Inside Diameter |  |  |
| Seat Insert Height |  |  |
| Seat Insert Bore Diameter |  |  |
| Seat Insert Bore Depth |  |  |

Valve Spring

Collapse

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length | 1.895, 1.832MIN (NM) | 1.895, 1.832MIN (NM) |
| Installed Height | 1.615" (41.021 MM) | 1.615" (41.021 MM) |
| Open Test Press |  |  |
| Close Test Press | 48-56# @ 1.615 | 48-56# @ 1.615 |
| Inner Spring | Intake | Exhaust |
| Free Length |  |  |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |

Rocker Shaft

Collapse

| Description | Specifications |
| --- | --- |
| Shaft Diameter |  |
| Bore Diameter |  |
| Oil Clearance |  |
| Rock Arm Ratio |  |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash | .008" COLD (.203 MM) |
| Exhaust Valve Lash | .008" COLD (.203 MM) |
| Lash Adjustment Method | ADJUSTING SCREW, 9.4 FT/LBS |
| Precup Torque Spec. |  |
| Cyl Head Torque | 58-72 FT/LBS |