Cylinder Head

| Description | Specifications |
| --- | --- |
| New Thickness/Measured At | 3.5413-3.5452" (89.949-90.048 MM) |
| Minimum Thickness | 3.533" (89.492 MM) |
| Warp Spec. | .006" OVERALL (.152 MM) |
| Chamber Volume (cc) |  |
| Surface Finish |  |
| OHC Camshaft Bore Diameter | 1.1811-1.1821" (30.-30.025 MM) |
| Injector Tip Protrusion |  |
| Lifter Bore Diameter |  |
| Comments |  |

Valve

Collapse

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 1.9290-1.9490" (48.997-49.505 MM) | 1.9290-1.9490" (48.997-49.505 MM) |
| Stem Diameter | .2744-.2750" (6.97-6.985 MM) | .2742-.2748" (6.965-6.98 MM) |
| Total Length | 4.4209-4.4367" (112.291-112.692 MM) | 4.4653-4.4811" (113.419-113.82 MM) |
| Head Diameter |  |  |
| Angle | 45° | 45° |
| Margin | .039" (.991 MM) | .059" (1.499 MM) |
| Recession |  |  |
| Valve Guide Information | Intake | Exhaust |
| Guide Height | .925-.953" (23.495-24.206 MM) | .925-.953" (23.495-24.206 MM) |
| Guide Clearance | .0010-.0024" (.025-.061 MM) | .0012-.0026" (.03-.066 MM) |
| Guide OD |  |  |
| Valve Seal Type | POSITIVE | POSITIVE |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .047-.063" (1.194-1.6 MM) | .047-.063" (1.194-1.6 MM) |
| Seat Angle | 45° | 45° |
| Seat Outside Diameter |  |  |
| Seat Inside Diameter |  |  |
| Seat Insert Height |  |  |
| Seat Insert Bore Diameter |  |  |
| Seat Insert Bore Depth |  |  |

Valve Spring

Collapse

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length | 1.960-1.970" (49.784-50.038 MM) | 1.960-1.970" (49.784-50.038 MM) |
| Installed Height | 1.693" (43.002 MM) | 1.693" (43.002 MM) |
| Open Test Press |  |  |
| Close Test Press | 44-50# @ 1.693 | 44-50# @ 1.693 |
| Inner Spring | Intake | Exhaust |
| Free Length |  |  |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |

Rocker Shaft

Collapse

| Description | Specifications |
| --- | --- |
| Shaft Diameter | .8252-.8260" (20.96-20.98 MM) |
| Bore Diameter | .8268-.8281" (21.001-21.034 MM) |
| Oil Clearance | .0008-.0029" (.02-.074 MM) |
| Rock Arm Ratio |  |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash | HYD |
| Exhaust Valve Lash | HYD |
| Lash Adjustment Method | NONE |
| Precup Torque Spec. |  |
| Cyl Head Torque | LARGE BOLTS: 59-64 FT/LBS, SMALL BOLTS: 12-17 FT/LBS |