CYLINDER HEAD

| Description | Specifications |
| --- | --- |
| New Thickness/Measured At | 5.3150" (135.000 MM) |
| Minimum Thickness | NOT SUPPLIED BY THE OE |
| Warp Spec. | .004" OVERALL (.102 MM) |
| Chamber Volume (cc) | 58.2 |
| Surface Finish |  |
| OHC Camshaft Bore Diameter | OIL CLEARANCE .001-.0024", MAX .0031" |
| Injector Tip Protrusion |  |
| Lifter Bore Diameter | 1.2191-1.2195" (30.965-30.975 MM) |
| Comments | * GUIDE R&R GRADUALLY HEAT THE HEAD TO 176-212° F (80-100° C) ** REPORTED 1.560" |

Valve

Collapse

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 1.545-1.560" (39.243-39.624 MM) ** | 1.545-1.560" (39.243-39.624 MM) ** |
| Stem Diameter | .2350-.2356" (5.969-5.984 MM) | .2348-.2354" (5.964-5.979 MM) |
| Total Length | 4.0531" MIN (102.949 MM) | 4.0590" MIN (103.099 MM) |
| Head Diameter |  |  |
| Angle | 44.5° | 44.5° |
| Margin | .039" (.990 MM), MIN .020 (.5 MM) | .039" (.990 MM), MIN .020 (.5 MM) |
| Recession |  |  |
| Valve Guide Information | Intake | Exhaust |
| Guide Height | .323-.339" (8.204-8.611 MM) * | .323-.339" (8.204-8.611 MM) * |
| Guide Clearance | .0010-.0024" (.025-.061 MM), MAX .0031" | .0012-.0026" (.03-.066 MM), MAX .0039" |
| Guide OD |  |  |
| Valve Seal Type | POSITIVE | POSITIVE |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .039-.055" (.991-1.397 MM) | .039-.055" (.991-1.397 MM) |
| Seat Angle | 30° / 45° / 60° | 30° / 45° / 60° |
| Seat Outside Diameter | CONTACT AREA MIDDLE OF VALVE | CONTACT AREA MIDDLE OF VALVE |
| Seat Inside Diameter |  |  |
| Seat Insert Height |  |  |
| Seat Insert Bore Diameter |  |  |
| Seat Insert Bore Depth |  |  |

Valve Spring

Collapse

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length |  |  |
| Installed Height | 1.406" (35.712 MM) | 1.406" (35.712 MM) |
| Open Test Press |  |  |
| Close Test Press | 39.7-45.9 # @ 1.406 | 3.9.7-45.9 # @ 1.406 |
| Inner Spring | Intake | Exhaust |
| Free Length |  |  |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |

Rocker Shaft

Collapse

| Description | Specifications |
| --- | --- |
| Shaft Diameter |  |
| Bore Diameter |  |
| Oil Clearance |  |
| Rock Arm Ratio |  |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash | .006-.010" COLD (.152-.254 MM) |
| Exhaust Valve Lash | .010-.014" COLD (.254-.356 MM) |
| Lash Adjustment Method | ADJUSTING SHIMS |
| Precup Torque Spec. |  |
| Cyl Head Torque | 29 FT/LBS, + 90°, + 90°, BOLTS 4.094" MIN IN THREADS OR REPL |
| Camshaft Cap Torque | 12FT/LBS |