Casting

| Description | Specifications |
| --- | --- |
| New Thickness/Measured At | 3.169" (80.493 MM) reported |
| Minimum Thickness |  |
| Warp Spec. | .004" OVERALL (.102 MM) |
| Chamber Volume (cc) |  |
| Surface Finish |  |
| OHC Camshaft Bore Diameter |  |
| Injector Tip Protrusion |  |
| Lifter Bore Diameter |  |
| Comments | * CASTING # 14066620 = 2.040-2.050" FOR BOTH INTAKE & EXHAUST ** ROCKER RATIO 1.75:1 |

Valve

Collapse

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 1.9380-1.9680" (49.225-49.987 MM)* | 1.9380-1.9680" (49.225-49.987 MM)* |
| Stem Diameter |  |  |
| Total Length |  |  |
| Head Diameter | 1.730" (43.942 MM) | 1.500" (38.100 MM) |
| Angle | 46° | 46° |
| Margin | .031" (.787 MM) | .031" (.787 MM) |
| Recession |  |  |
| Valve Guide Information | Intake | Exhaust |
| Guide Height |  |  |
| Guide Clearance |  |  |
| Guide OD |  |  |
| Valve Seal Type |  |  |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .032-.063" (.813-1.6 MM) | .032-.063" (.813-1.6 MM) |
| Seat Angle | 45° | 45° |
| Seat Outside Diameter |  |  |
| Seat Inside Diameter |  |  |
| Seat Insert Height |  |  |
| Seat Insert Bore Diameter |  |  |
| Seat Insert Bore Depth |  |  |

Valve Spring

Collapse

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length | 2.080" (52.832 MM) | 2.080" (52.832 MM) |
| Installed Height | 1.660" (42.164 MM) | 1.660" (42.164 MM) |
| Open Test Press | 175# @ 1.260 | 175# @ 1.260 |
| Close Test Press | 83# @ 1.656" CHECK CASTING #'S | 83# @ 1.656" CHECK CASTING #'S |
| Inner Spring | Intake | Exhaust |
| Free Length |  |  |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |

Rocker Shaft

Collapse

| Description | Specifications |
| --- | --- |
| Shaft Diameter | RATIO: 1.75:1 |
| Bore Diameter |  |
| Oil Clearance |  |
| Rock Arm Ratio |  |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash | 3/4-1 TRN FRM 0 ** |
| Exhaust Valve Lash | 3/4-1 TRN FRM 0 ** |
| Lash Adjustment Method | ADJUSTING SCREW |
| Precup Torque Spec. |  |
| Cyl Head Torque | 93 FT/LBS |