Casting

| Description | Specifications |
| --- | --- |
| New Thickness/Measured At | 4.5850-4.5920" (116.459-116.637 MM) |
| Minimum Thickness | OEM CALLS NEW THICKNESS MINIMUM But .010" has been Safe |
| Warp Spec. | .002" OVERALL (.051 MM) |
| Chamber Volume (cc) | 64 CC s |
| Surface Finish |  |
| OHC Camshaft Bore Diameter | 1.0245-1.0252" (26.022-26.040 MM) |
| Injector Tip Protrusion |  |
| Lifter Bore Diameter |  |
| Comments | * 2005-06 INTAKE HEAD DIAMETER=1.9103-1.9205" & INTAKE VALVE LENGTH=4.4666-4.4965". |

Valve

Collapse

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 2.0400-2.0550" (51.816 52.197 MM) | 2.0950-2.1100" (53.213-53.594 MM) |
| Stem Diameter | .2729-.2739" (6.931-6.957 MM) | .2717-.2728" (6.901-6.929 MM) |
| Total Length | * 4.4539-4.4839" (113.129-113.891 MM) | 4.5244-4.5543" (114.92-115.679 MM) |
| Head Diameter | * 1.8846-1.8949" (47.869-48.13 MM) | 1.4516-1.4618" (36.871-37.13 MM) |
| Angle | 45-45.5° | 45-45.5° |
| Margin |  |  |
| Recession |  |  |
| Valve Guide Information | Intake | Exhaust |
| Guide Height | .525" (13.335 MM) | .525" (13.335 MM) |
| Guide Clearance | .0008-.0028" (.018-.071 MM) | .0019-.0039" (.047-.098 MM) |
| Guide OD |  |  |
| Valve Seal Type |  |  |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .0698-.0928" (1.773-2.357 MM) | .0673-.0911" (1.709-2.314 MM) |
| Seat Angle | 44.5-45° | 44.5-45° |
| Seat Outside Diameter |  |  |
| Seat Inside Diameter |  |  |
| Seat Insert Height |  |  |
| Seat Insert Bore Diameter |  |  |
| Seat Insert Bore Depth |  |  |

Valve Spring

Collapse

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length | 1.929" (49.000 MM) | 1.929" (49.000 MM) |
| Installed Height | 1.580" (40.120 MM) | 1.580" (40.120 MM) |
| Open Test Press | 174-196# @ 1.137" | 174-196# @ 1.137" |
| Close Test Press | 70-80# @ 1.580" | 70-80# @ 1.580" |
| Inner Spring | Intake | Exhaust |
| Free Length |  |  |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |

Rocker Shaft

Collapse

| Description | Specifications |
| --- | --- |
| Shaft Diameter |  |
| Bore Diameter |  |
| Oil Clearance |  |
| Rock Arm Ratio |  |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash | .018-.033" (.457-.838 MM) Dry Lash |
| Exhaust Valve Lash | .018-.033" (.457-.838 MM) Dry Lash |
| Lash Adjustment Method | HLA |
| Precup Torque Spec. |  |
| Cyl Head Torque |  |
| Camshaft Cap Torque | 100 INCH/LB |