Casting

| Description | Specifications |
| --- | --- |
| New Thickness/Measured At | 3.7000" (93.98 MM) |
| Minimum Thickness | NOT PROVIDED -- MUST CHECK CC OF HEAD |
| Warp Spec. | .007" OVERALL (.178 MM) |
| Chamber Volume (cc) | 68CC MIN |
| Surface Finish | 60-150 RMS |
| OHC Camshaft Bore Diameter | NONE |
| Injector Tip Protrusion | NONE |
| Lifter Bore Diameter | .8740-.8745" (22.2-22.212 MM) |
| Comments | * ENGINES WITH ROCKER BOLTS ** ENGINES WITH TORQUE-YIELD BOLTS BUILT 1993-> AFTER SEPT. 1 USE 45-55 FT/ LBS. + 80-100° OF ROTATION. |

Valve

Collapse

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 1.8310-1.8610" (46.507-47.269 MM) * | 1.8310-1.8610" (46.507-47.269 MM) * |
| Stem Diameter | .3416-.3423" (8.677-8.694 MM) | .3416-.3423" (8.677-8.694 MM) |
| Total Length | 4.750" (120.65 MM) | 4.750" (120.65 MM) |
| Head Diameter | 1.769-1.793" (44.933-45.542 MM) | 1.551-1.569" (39.395-39.853 MM) |
| Angle | 44° | 44° |
| Margin | .032" MIN. (.813 MM) | .032" MIN. (.813 MM) |
| Recession |  |  |
| Valve Guide Information | Intake | Exhaust |
| Guide Height | INTEGRAL | INTEGRAL |
| Guide Clearance | .0010-.0027" (.025-.069 MM) | .0010-.0027" (.025-.069 MM) |
| Guide OD |  |  |
| Valve Seal Type | UMBRELLA | UMBRELLA |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .060-.080" (1.524-2.032 MM) | .070-.090" (1.778-2.286 MM) |
| Seat Angle | 45° | 45° |
| Seat Outside Diameter | 1.812" (46.025 MM) | 1.687" (42.85 MM) |
| Seat Inside Diameter | 1.406" (35.712 MM) | 1.500" (38.1 MM) |
| Seat Insert Height | .219" (5.563 MM) | .219" (5.563 MM) |
| Seat Insert Bore Diameter | 1.804" (45.822 MM) | 1.679" (42.647 MM) |
| Seat Insert Bore Depth | .219" (5.563 MM) | .219" (5.563 MM) |

Valve Spring

Collapse

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length | 1.960" (49.784 MM) | 1.780" (45.212 MM) |
| Installed Height | 1.610-1.670" (40.894-42.418 MM) | 1.440-1.500" (36.576-38.1 MM) |
| Open Test Press | 166-184# @1.240 | 166-184# @1.070 |
| Close Test Press | 66-74# @ 1.640 | 66-74# @ 1.470 |
| Inner Spring | Intake | Exhaust |
| Free Length |  |  |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |

Rocker Shaft

Collapse

| Description | Specifications |
| --- | --- |
| Shaft Diameter |  |
| Bore Diameter |  |
| Oil Clearance |  |
| Rock Arm Ratio |  |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash | .125-.175" COLD (3.175-4.445 MM) |
| Exhaust Valve Lash | .125-.175" COLD (3.175-4.445 MM) |
| Lash Adjustment Method | COLLAPSED LIFTER, DRY LASH |
| Precup Torque Spec. |  |
| Cyl Head Torque | 1ST 50-55, 2ND 60-65, 3RD 70-85 FT/LBS ** |