Casting

| Description | Specifications |
| --- | --- |
| New Thickness/ Measured At |  |
| Minimum Thickness |  |
| Warp Spec. | .003" OVERALL (.076 MM) |
| Chamber Volume (cc) | 55 OR 65 CC |
| Surface Finish |  |
| OHC Camshaft Bore Diameter |  |
| Injector Tip Protrusion |  |
| Lifter Bore Diameter |  |
| Comments | * 1ST ALL BOLTS IN SEQUENCE 22 FT/LBS, 2ND BOLTS IN FINAL PASS SEQUENCE SHORT 55°, MEDIUM 65°, LONG 75° DEGREES. * UPPER 44,88 INCH/LBS |

Valve

Collapse

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 1.990-2.000" (50.546-50.800 MM) | 1.990-2.000" (50.546-50.800 MM) |
| Stem Diameter | .3410-.3417" (8.661-8.679 MM) | .3410-.3417" (8.661-8.679 MM) |
| Total Length | 4.9120" (124.765 MM) | 4.9280" (125.171 MM) |
| Head Diameter | 1.840" (46.736 MM) | 1.500" (38.100 MM) |
| Angle | 45° | 45° |
| Margin | .031" (.787 MM) | .031" (.787 MM) |
| Recession |  |  |
| Valve Guide Information | Intake | Exhaust |
| Guide Height |  |  |
| Guide Clearance | .0010-.0030" (.025-.076 MM) | .0010-.0030" (.025-.076 MM) |
| Guide OD |  |  |
| Valve Seal Type |  |  |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .045-.070" (1.143-1.778 MM) | .065-.098" (1.651-2.489 MM) |
| Seat Angle | 46° | 46° |
| Seat Outside Diameter |  |  |
| Seat Inside Diameter |  |  |
| Seat Insert Height |  |  |
| Seat Insert Bore Diameter |  |  |
| Seat Insert Bore Depth |  |  |

Valve Spring

Collapse

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length | 2.020" (51.308 MM) | 2.020" (51.308 MM) |
| Installed Height | 1.690-1.710" (42.926-43.434 MM) | 1.690-1.710" (42.926-43.434 MM) |
| Open Test Press | 187-203# @1.270 | 187-203# @1.270 |
| Close Test Press | 76-84# @ 1.700 | 76-84# @ 1.700 |
| Inner Spring | Intake | Exhaust |
| Free Length |  |  |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |

Rocker Shaft

Collapse

| Description | Specifications |
| --- | --- |
| Shaft Diameter |  |
| Bore Diameter |  |
| Oil Clearance |  |
| Rock Arm Ratio |  |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash | ZERO + 1 TURN DOWN |
| Exhaust Valve Lash | ZERO + 1 TURN DOWN |
| Lash Adjustment Method |  |
| Precup Torque Spec. |  |
| Cyl Head Torque |  |