Chevrolet V8 350 Cylinder Head – CH350C

Intake runner 170CC 
Combustion Chamber 64CC Vortec style
1996 - 2006 Intake manifold bolt pattern
Center bolt down valve cover holes

CH350C

Specifications

INTAKE 	EXHAUST

Port Volume: 	170cc	67cc

Port Dimension: 	1.200” 2.025”	1.350” X 1.400”

Port Location: 	Stock

Valves:	1.940” X .3414” X 4.88” 2G	1.500” X .3412” X 4.92”, 2G

Valve Springs:	1.25 OD X .87 ID X 80 LB @ 1.7	1.25 OD X .87 ID X 80 LB @ 1.7

Valve Seals:	.343 X 530 X VITON	.343 X 530 X VITON

Chamber Volume: 	64cc

Spark Plug Location: 	Stock - straight

Spark Plug Type:	14 mm

Intake Manifold Bolt pattern:	1996 - 2006

Valve Cover mounting:	Center bolt down

Deck Surface RA:

Weight:	51 lbs

•Highest grade of material used in manufacturing

•Thicker decks for greater strength

•Superior port finishes for increased flow

•Improved flow design to resist cracking

•Redesigned water flow for greater thermal conductivity

•Hardened exhaust seats

Flat Milling:	 .006" per cc

Angle Milling: 	.0075" per cc (excessive milling can cause intake manifold misalignment)

Head Bolt Torque: 	Refer to cylinder head bolt manufacturer's specifications

Rocker Stud Torque: 	45 lb/ft

Intake Manifold Torque: 	35 lb/ft

Spring Pocket Machining: 	Can be machined to 1.550" maximum diameter, cannot be machined deeper

Pushrod Length: 	Measuring with length checker (COMP Cams® #7705) required to verify pushrod geometry

Piston to Valve Clearance: 	Must be verified -minimum clearance is .080" intake and .100" exhaust

Retainer to Guide Clearance:	 Must always be measured when using high lift cams

Chamber to Dome Clearance: 	Must always be checked with domed pistons, clearance may be necessary

Chamber Volume: 	Varies due to valve margins and dishes, should always be verified before installation

Valve Guide Clearance: 	Must be checked due to the variance in valve stem diameters between manufacturers

Recommended clearance: 	intake .0015"-.0016", exhaust .002"-.0025"

Water jackets require flushing prior to cylinder head installation

Header/Spark Plug Clearance: 	Spark plug/ wire boot clearance verification is required prior to header installation.

Minor modifications may be necessary in order to achieve proper clearance.

Spark Plugs: 	ZEX™ #82070-8 Street/strip 14mm, .708" reach, 5/8" hex, premium spark plug set

ZEX™ #82209-8 Race: Champion S57YC, 14mm, .708" reach, 5/8" hex, spark plug set

Rocker Arms:	COMP Cams® #1301-16* Pro Magnum ™ 1.52 ratio rocker arm set, 3/8" stud diameter

Intake Gaskets:	RHS™ #1707 Intake manifold gasket,2.09" X 1.28" port, .060" thick

Exhaust Gaskets:	RHS™ #1708 Header gasket, 1.500" X 1.500" port size, graphite

Head Gaskets:	RHS™ #1700 Cylinder head gasket, 4.060" bore X .041" compressed thickness

RHS™ #1701 Cylinder head gasket, 4.165" bore X .041" compressed thickness

Head Bolts:	HB350N (5/8” hex head) HB350P (1/2” head) EngineQuest or  ARP #134-3701 HP head bolt set, 265-400 SBC

Head Studs:	ARP #234-4301 8740 head stud set, 265-400 SBC