Chevrolet V8 350 Cylinder Head – CH350CA

Intake runner 170CC 
Combustion Chamber 64CC Vortec style
1996 - 2006 Intake manifold bolt pattern
Center bolt down valve cover holes

CH350CA

Specifications

INTAKE 	EXHAUST

Port Volume: 	170cc	67cc

Port Dimension: 	1.200” 2.025”	1.350” X 1.400”

Port Location: 	Stock

Valves:	1.940” X .3414” X 4.88” 2G	1.500” X .3412” X 4.92”, 2G

Valve Springs:	1.25 OD X .87 ID X 80 LB @ 1.7	1.25 OD X .87 ID X 80 LB @ 1.7

Valve Seals:	.343 X 530 X VITON	.343 X 530 X VITON

Chamber Volume: 	64cc

Spark Plug Location: 	Stock - straight

Spark Plug Type:	14 mm

Intake Manifold Bolt pattern:	1996 - 2006

Valve Cover mounting:	Center bolt down

Weight:	51 lbs

•Highest grade of material used in manufacturing

•Thicker decks for greater strength

•Superior port finishes for increased flow

•Improved flow design to resist cracking

•Redesigned water flow for greater thermal conductivity

•Hardened exhaust seats

•CNC machined guides and seats

•25 point quality check on every head

•Assembled heads use OE quality stainless steel valve, springs, retainers, keepers and viton seals.