National Research and Development Institute for Industrial Ecology – ECOIND

Pollution Control Department

BIOASSAY – BIOLOGICAL ANALYSIS LABORATORY

BIOASSAYS FOR CHEMICALS /  CONTAMINATED SAMPLES / MICROPLASTICS

| AQUATIC TOXICITY TESTS |  |  |  |
| --- | --- | --- | --- |
| Aquatic organism | Reference document | Array | Parameter / Final Result |
| Freshwater fish Cyprinus carpio (carp) Carassius auratus gibelio (caras) | Acute toxicity OECD 203 / SR EN ISO 7346-1,2:2004 [*] | Chemicals Wastewater / surface / Sediment | Mortality / acute lethal conc. (LC50-96h) |
|  | Chronic toxicity Internal procedure | Chemicals | Biological and biochemical indices / maximum allowed conc. in water (MATC) – minimum 3 months |
|  | Bioaccumulation / OECD 305 | Chemicals | Bioconcentration Factor (BCF) – minimum 3 months |
| Plankton crustaceans |  |  |  |
| Daphnia magna | Acute toxicity OECD 202/ SR EN ISO 6341 :2013 [*] | Chemicals Wastewater / groundwater / surface/ leachate /waste | Mortality /immobilization / effective conc. for 50% of organisms after 48h (EC50-48h) |
| Ciliated protozoa |  |  |  |
| Tetrahymena thermophilea | Chronic toxicity Protoxkit F | Chemicals Wastewater / groundwater / surface/ sediments | Reproductive inhibition/ effective conc. for 50% of organisms after 24h (EC50-24h) |
| Rotifers |  |  |  |
| Branchhionus calyciflorus | Acute toxicity ASTM Standard E1440-9 | Chemicals Wastewater / groundwater / surface/ sediments | Mortality / lethal conc. for 50% of organisms after 24h (LC50-24h) |
| Aquatic plants |  |  |  |
| Spirodela duckweed (duckweed) | Acute toxicity ISO 20227:2017 | Chemicals Wastewater / groundwater / surface/ sediments | Rate of inhibition of growth / effective conc. for 50% of organisms after 72 h (EC50-72h) |
| Selenastrum capricornutum (algae) | Acute/chronic toxicity OECD 201/ SR EN ISO 8692 :2012 [*] | Chemicals Wastewater / groundwater / surface/ leachate /waste | Growth rate/biomass/ effective conc. for 50% of organisms after 72h (EC50-72h) |
| Benthic crustaceans (ostracods) |  |  |  |
| Heterocypris incongruens | Chronic toxicity ISO 14371:2012 | Chemicals Waste water / surface/ leachate / mud/ soil/ sediment | Mortality/ Inhibition of growth/ effective conc. for 50% of organisms after 6 days (EC50-6 days) |

Ecotoxicological information will help for: completing Technical Security Files (Ecological Information section), obtaining authorization/marketing product approvals, environmental opinions, evacuation notices, risk assessment studies, environmental impact assessment studies, obtaining the eco label, etc.

| AQUATIC TOXICITY TESTS |  |  |  |
| --- | --- | --- | --- |
| Aquatic organism | Reference document | Array | Parameter / Final Result |
| Bacteria |  |  |  |
| Aliivibrio fischeri | Acute toxicity SR EN ISO 11348-3:2009 | Chemicals Waste water/ groundwater | Inhibition of bioluminescence / conc. inhibitors (IC50) |
| Escherichia coli | Genotoxicity ISO 13829:2000 | Chemicals Waste water / groundwater / surface / leachate / waste / mud / | Enzymatic activity (β-galactosidase and alkaline phosphatase) / Induction factor (SOSIF) |
| Microbacterium sp. Brovundimonas diminuta, Citrobacter freundii, Comamonastestosterroni, Enterococcus casseliflavus, Deltftia acidovorans, Kurthia gibsonii, Staphilococcus warnerii, Pseudomonas aurantiaca, Serratia rubidae, Pichia anomala | Chronic multi-species test - MARA (Microbial Array for Toxicity Risk Assessment) / Internal procedure | Chemicals / wastewater | Inhibition of microbial growth / microbial toxic conc. (MTC) |
| Fish cell line RT gill-W1 (in collaboration with University of Bucharest, Faculty of Biology) | Acute toxicity OECD 249 / ISO 2115:20219 | Chemicals Water sample | Inhibition of cell viability (EC50) |

| TERESTRIAL TOXICITY TESTS |  |  |  |
| --- | --- | --- | --- |
| Terrestrial plants |  |  |  |
| Lepidium sativum (cress) Sinapis alba (mustard) Sorghum saccharatum (sorghum) | Phytotoxicity SR EN ISO 11269-1,2:2013 / OECD 208 | Chemicals / waste water irrigation/ sludge/ soil/ waste/ leachate | Inhibition of germination and root growth / EC50-72h |
| Earthworms |  |  |  |
| Eisenia andrei | Acute toxicity / SR EN ISO 11268-1:2016 OECD 207 | soil, compost, sludge, waste, chemicals | Determination of acute lethal concentration LC 50-14 days |

| BIOCIDE TESTS (ALGICIDES/DISINFECTANTS) |  |  |  |
| --- | --- | --- | --- |
| Organism | Reference document | Array | Parameter / Final Result |
| Aquatic plants |  |  |  |
| Selenastrum capricornutum (algae) | Acute/chronic toxicity / Algaecide efficacy OECD 201/ SR EN ISO 8692 :2012 [*] | Algicides / Products for Swimming Pools | Growth rate/biomass/ effective conc. for 50% of organisms after 72h (EC50-72h). Efficacy at the recommended dose / Recommended effective dose |
| Spirodela duckweed (duckweed) | Acute toxicity ISO 20227:2017 |  |  |
| Bacteria |  |  |  |
| Pseudomonas aeruginosa Escherichia coli Legionella pneumophila Staphylococus aureus | Internal standard | Disinfectants | Bactericidal efficiency tests |

| ULTIMATE AEROBIC BIODEGRADABILITY |  |  |  |
| --- | --- | --- | --- |
| Test | Reference document | Array | Parameter / Final Result |
| Static test for the evaluation of "inherent" aerobic biodegradability under action of aerobic microorganisms – Zahn-Wellens method | SR EN ISO 9888:2004 / OECD 302B/ EC-C.9 method [*] | Chemicals/ Waste water / surface water | % abiotic degradation >20% COD / COD removal – primary biodegradation ≥70% COD / COD removal – inherent biodegradation |
| Watery environmental assessment of "rapid" aerobic biodegradability by experiments in biological installations with active sludge, conducted in a discontinuous system (batch) | Dissolved organic carbon (COD) analysis method (SR EN ISO 7827:2013 / OECD 301A / EC-C.4-A method) | Chemicals/ Waste water / surface water | ≥70% COD removal in 28 days |
|  | Method by analysis of biochemical oxygen consumption (BOD5) - Closed container test [*] (SR EN ISO 10707:2001 / OECD 301D / EC-C.4-E method) |  | Specific BOD5 (mg oxygen /mg tested compound) / ≥60% theoretical oxygen consumption (CTO) in 28 days |
|  | Test to assess the inhibitory effect on oxygen consumption of active sludge microorganisms with oxidation of carbon and ammonium (SR EN ISO 8192:2007 / OECD 209 / EC-C.11 method) |  | Inhibition of total oxygen consumption (I%); which inhibits 50% oxygen consumption (EC50) in 30-180 min. |
| Determination of elimination and biodegradability in a continuous testing system that simulates the biological process with activated sludge | Simulation test in aerobic treatment plants with activated sludge (simulates a wastewater treatment plant) (SR EN ISO 11733:2005/ OECD303A/EC-C.10 method) | Water-soluble organic compounds | Determine daily COD/COD in influence and effluent. Test duration: 12 weeks >80% COD/COD removal in 28 days |
| Determination of the biodegradability of surface agents | Reference method described in Annex VIII.1 to Reg.EC 648/2004 OECD Confirmation Test 303A [*] | Detergents / cleaning products | % remove COD / COD at time t % removal surfactant at time t >80% COD /COD removal for 2 weeks |
| Assessment of degradation capacity by determining global indicators: COD and BOD5 | SR ISO 6060:1996 (validated standard) [*] SR EN ISO 5815-1:2020 [*] SR EN 1899-2:2002 [*] | Chemicals/ Waste water | BOD5/COD report >0.5 |

| BIOLOGICAL ANALYZES |  |  |  |
| --- | --- | --- | --- |
| Test | Reference document | Array | Parameter / Final Result |
| Spectrometric determination of "chlorophyll a" content | SR ISO 10260:1996 [*] | Surface water, algal solutions, groundwater, mineral water, wastewater | "Chlorophyll a" / concentration expressed in µg/L |
| Microscopic analysis (qualitative detection of biotic communities) | Internal standard | Surface water (lotic and lentic aquatic ecosystems), sediment and groundwater | Taxonomic composition |
| Qualitative and quantitative detection of benthic invertebrates | SR EN ISO 8689/1,2:2003 SR EN 16150:2012 SR EN ISO 10870:2012 | Lotic and lentic aquatic ecosystems | Taxonomic composition and structure |
| Bacteria at 22°C and 37°C Total coliforms Fecal coliforms Escherichia coli Salmonella spp. Legionella sp. Clostridium perfringens | SR EN ISO / EPA standards / Internal standards according to RENAR Accreditation Certificate LI 941 dated 15.12.2023 (Annex 1) | Drinking water, mineral water, bathing water, groundwater, wastewater, surface water, soil, sediments, waste, sludge | Number of bacteria in analyzed volume / Present or Absent |
| Sanitation tests | Internal standards | Surfaces and objects | Total number of colonies at 37°C in 48 h / No. of germs per cm² |
| Aeromicroflora tests |  | Indoor air | Total number of bacteria at 37°C (no./plate) / No. of germs per m³ of air |
| Mold tests |  | Products (paints) | No. of germs per cm² at 22°C in 48 h |
| Identification of microorganisms (especially bacteria) using the Biolog Omnilog system |  | All types of water | Bacterial species |
| SAMPLING |  |  |  |
| Water / sediment sampling | According to RENAR Accreditation Certificate LI 941 dated 15.12.2023 (Annex 1) | All types of water, sediments, sludge | Sampling for physico-chemical and biological analyses |
| On-site analyses: pH, dissolved oxygen, oxygen saturation, temperature, free residual chlorine | According to sampling standards | Water samples | multiparameter devices and analysis kits |

[*] accredited method SR EN ISO/CEI 17025:2018; the rest of the methods are applied in a SR EN ISO 9001 and SR EN ISO 14001 certified infrastructure

Request the recommendations of INCD ECOIND specialists for the selection of test methods according to your needs.

** For chemicals toxicity classification according to REACH regulation are needed tests on daphnia, algae and fish and ultimate biodegradability test.

| Microplastics analyses |  |  |  |
| --- | --- | --- | --- |
| Test | Reference document | Array | Parameter / Final Result |
| Visual analysis and sample pretreatment (sorting and separating the suspected particles0 | SR EN ISO 24187:2023 ISO/TR 21960:2020 Internal methods Scientific literature https://doi.org/10.3390/w16020233 https://doi.org/10.3390/toxics13040246 https://www.incdecoind.ro/microplastice-efecte-ecotoxicologice-si-mecanisme-de-actiune-in-pesti-microplasfish/ | Surface water, wastewater, drinking water, sediment | Present/ Absent Number per liter Particles dimensions Shape Color Type of polymer Pictures |
| Conventional microscopic analysis (Stereomicroscope Leica M205FA) to highlight the shape, morphology, size, and number of these particles |  |  |  |
| Electron microscopy (SEM QUANTA FEG 250) analysis - morphology of the suspected microplastic particles (less than 100 µm). |  |  |  |
| Qualitative determination of polymer type using micro-RAMAN or FT-IR analyses - collaboration with the National University of Science and Technology Polyethnic Bucharest |  |  |  |

|  | Dr. Alina Roxana BANCIU Head of Biotests and Biological Analysis Lab Dr. Stefania GHEORGHE Ecotoxicological Studies Coordinator Dr. Catalina Stoica Freshwater Ecology Coordinator Control Pollution Department Address: Drumul Podu Dâmboviței 57-73, District 6, Bucharest Mobile: +4.0724004625; Phone: +4.021.410.03.77, int. 156 e-mail: alina.banciu@incdecoind.ro stefania.gheorghe@incdecoind.ro catalina.stoica@incdecoind.ro Website: www.incdecoind.ro |
| --- | --- |