É P I C A   A U D I O

FEEDBACK CHORUS

Classic BBD Chorus

User Manual

The chorus that feeds on itself.

# INTRODUCTION

Feedback Chorus is a classic chorus plug-in developed by Épica Audio, inspired by the legendary sound of the Roland Juno-60. Like the analog circuit that made it famous, Feedback Chorus is built around a bucket-brigade device (BBD) delay line — the same core technology behind the warmth and character of the original hardware.

Feedback Chorus goes a step further with a parameter no other chorus offers: Feedback. Rather than only modulating a fixed delay line, Feedback Chorus routes a portion of the BBD's own output back into its input, deepening and animating the modulation far beyond what the classic circuit was capable of.

DESIGN PHILOSOPHY The chorus that feeds on itself.

# COMPATIBILITY

## Plug-in Formats

- VST

- AAX

- Audio Unit / Component

## Supported Operating Systems

- macOS

- Windows

Feedback Chorus is compatible with any DAW that supports the plug-in formats listed above.

# SIGNAL FLOW

Audio passes through the plug-in in the following order:

Input Signal  →  BBD Delay Line (Rate & Depth)  →  Feedback Loop  →  Output

The Feedback path taps the BBD's output and routes it back into the BBD's input, alongside the incoming dry signal. This creates a compounding modulation effect that classic, non-feedback chorus circuits cannot produce.

# THE BBD CHORUS CIRCUIT

A bucket-brigade device is an analog delay line built from a long chain of capacitors: an audio sample is passed from capacitor to capacitor, “bucket to bucket,” arriving at the output a short time later. Modulating that delay time with a slow LFO — and mixing the delayed signal back with the dry signal — is what produces the lush, detuned movement associated with classic analog chorus units, the Juno-60 among them.

Feedback Chorus models this circuit faithfully, capturing the same warmth and gentle non-linearity of a real BBD chain. Rate and Depth control the LFO that modulates the delay time, exactly as they would on the original hardware.

# FEEDBACK: BEYOND THE JUNO-60

The Juno-60's chorus circuit was fixed: the BBD's output only ever went to the mix, never back into itself. Feedback Chorus adds a signal path the original never had — routing part of the BBD's delayed output back into its own input.

At low settings, this thickens the modulation subtly, adding a sense of depth and dimension beneath the familiar Juno-style sweep. As Feedback increases, the effect compounds on itself: the chorus becomes denser, more resonant, and more animated, eventually taking on metallic, almost flanger-like movement that no traditional chorus circuit — analog or digital — can produce.

# PARAMETERS

## Feedback

| Parameter | Value / Range | Description |
| --- | --- | --- |
| Feedback | 0% – 100% | Controls how much of the BBD's output is fed back into its own input. At 0%, Feedback Chorus behaves like a classic, Juno-inspired chorus. Higher values add resonance and density to the modulation; extreme settings produce metallic, self-generating textures unique to this circuit. |

## Depth

| Parameter | Value / Range | Description |
| --- | --- | --- |
| Depth | Variable | Sets the modulation depth of the BBD delay time. Higher values increase the intensity of the pitch modulation and the resulting stereo movement. |

## Rate

| Parameter | Value / Range | Description |
| --- | --- | --- |
| Rate | 0 – 1.2 Hz | Sets the speed of the internal LFO modulating the BBD delay time. The range is deliberately restricted to slow, sweeping speeds — true to the original Juno-60 — rather than the faster rates found on modulation-effect plug-ins. |

## On/Off

| Parameter | Value / Range | Description |
| --- | --- | --- |
| On/Off | On / Off | Enables or bypasses the entire plug-in. When off, the input signal passes through unaffected. |

# TIPS & BEST PRACTICES

## Classic Juno Character

For an authentic vintage chorus sound, keep Feedback low or off, set Depth to a moderate value, and keep Rate slow. This recreates the smooth, gentle sweep the Juno-60 is known for.

## Pushing Feedback for Movement

Gradually raising Feedback on pads, guitars, or synths adds a lush, animated quality to the modulation that a standard chorus can't reach — useful when a sound needs to feel alive without adding an obvious extra effect.

## Feedback Into Self-Oscillation

At extreme Feedback settings, the circuit begins to feed on itself, producing resonant, metallic, almost flanger-like tones. This is best used deliberately, as a sound-design tool rather than a subtle chorus effect.

## Wide Slow Modulation

Combining a slow Rate with a higher Depth produces a wide, vibrato-like pitch movement well suited to ambient textures, synth pads, and sustained guitar chords.

## Subtle Width on Vocals

For vocals or other sources where the chorus effect should be felt rather than heard, keep both Depth and Feedback low. This adds width and a sense of analog movement without an obvious modulation character.

Experimental Vocal Textures Heavy 8-’s Guitars and Synths

For that unmistakable '80s sheen, set Rate to a medium speed and push Depth high. This combination produces the thick, resonant, almost shimmering modulation heard on classic '80s guitar tones and synth pads — bold and immediately identifiable rather than subtle.

Developed by Épica Audio