Greenhouse and Energy Minimum Standards (Incandescent Lamps for General Lighting Services) Determination 202X

I, Jenny McAllister, Assistant Minister for Climate Change and Energy, make the following determination.

Dated

Jenny McAllister DRAFT ONLY—NOT FOR SIGNATURE

Assistant Minister for Climate Change and Energy

Contents

Part 1 —Preliminary	1

1  Name	1

2  Commencement	1

3  Authority	1

4  Schedules	1

5  This instrument revokes and replaces the old determination	1

6  Definitions—standards referred to in this instrument	2

7  Definitions—other expressions used in this instrument	2

8  Applicable definitions and applicable versions of standards and documents incorporated into standards	3

9  Families of models	4

10  Product category	5

11  Limited grandfathering	5

12  Registrations affected by this instrument	5

Part 2 —Products covered by instrument	6

13  Purpose of Part	6

14  Classes of products that are covered by this instrument	6

15  Classes of products that are not covered by this instrument	6

Part 3 —GEMS level requirements	7

16  Purpose of Part	7

17  GEMS level requirements	7

18  Testing requirements	7

Part 4 —GEMS labelling requirements	9

19  Purpose of Part	9

20  GEMS labelling requirements	9

21  Testing requirements	9

22  Impact of replacement determination	9

Part 5 —Other requirements	10

23  Purpose of Part	10

24  Other GEMS requirements	10

25  Testing requirements	10

Schedule 1—Registrations affected by this instrument	11

Schedule 2—Products not covered by this instrument	12

1  Aeronautical lamps	12

2  Other exclusions	12

Schedule 3—Repeals	14

Greenhouse and Energy Minimum Standards (Incandescent Lamps for General Lighting Services) Determination 2016	14

COPYRIGHT

© 202X Commonwealth of Australia

This instrument includes material from Australian Standards and/or Australian/New Zealand Standards, which are copyright Standards Australia Ltd. Apart from uses permitted under the Copyright Act 1968, Standards Australia Ltd material may not be reproduced without permission or licence.

This instrument also includes material from International Electrical Commission (IEC) Standards, which are copyright IEC. Apart from reproduction for personal and non-commercial use, and uses permitted under the Copyright Act 1968, IEC material may not be reproduced without permission or licence.

With the exception of the Commonwealth Coat of Arms, any trade marks owned by the Commonwealth and any material the subject of third party intellectual property rights, this instrument is licensed under the Creative Commons Attribution-Non-Commercial-ShareAlike 4.0 International Licence. To view a copy of this licence, visit https://creativecommons.org/licenses/by-nc-sa/4.0/.

You are free to copy and communicate this instrument (apart from the excluded material indicated above) so long as you do so for non-commercial purposes, attribute the Commonwealth of Australia, and license any derivative works under the same licence as the original (indicating any modifications), in accordance with CC BY-NC-SA. For clarity, you are permitted to use this instrument in the above manner to comply with your obligations under it.

Complete Australian/New Zealand Standards are available for purchase from Standards Australia Ltd. Requests and inquiries concerning other reproduction and rights pertaining to standards should be directed to SAI Global Limited.

Complete IEC Standards are available for purchase from Standards Australia Ltd. Requests and inquiries concerning other reproduction and rights pertaining to standards should be directed to Standards Australia Ltd via the contact details set out at https://www.standards.org.au/contact.

—Preliminary

Name

This instrument is the Greenhouse and Energy Minimum Standards (Incandescent Lamps for General Lighting Services) Determination 202X.

Commencement

(1)	Each provision of this instrument specified in column 1 of the table commences, or is taken to have commenced, in accordance with column 2 of the table. Any other statement in column 2 has effect according to its terms.

| Commencement information |  |  |
| --- | --- | --- |
| Column 1 | Column 2 | Column 3 |
| Provisions | Commencement | Date/Details |
| 1. The whole of this instrument | TBC | TBC |

(2)	Any information in column 3 of the table is not part of this instrument. Information may be inserted in this column, or information in it may be edited, in any published version of this instrument.

(3)	To avoid doubt, for the purposes of paragraph 34(b) of the Act, this instrument comes into force on the day specified by column 2 of the table.

Authority

This instrument is made under sections 23 and 35 of the Greenhouse and Energy Minimum Standards Act 2012.

Schedules

Each instrument that is specified in a Schedule to this instrument is amended or repealed as set out in the applicable items in the Schedule concerned, and any other item in a Schedule to this instrument has effect according to its terms.

This instrument revokes and replaces the old determination

For the purposes of section 35 of the Act, this instrument revokes and replaces the Greenhouse and Energy Minimum Standards (Incandescent Lamps for General Lighting Services) Determination 2016.

Note:	Subsection 35(2) of the Act provides that, when a GEMS determination is revoked and replaced, the revoked determination ceases to be in force immediately before the replacement determination comes into force.

Definitions—standards referred to in this instrument

Note:	Subsection 8(3) provides that the applicable version of each standard referred to in this section is the version that existed at the date this instrument was made.

In this instrument:

AS/NZS 4934.1:2014 means Australian/New Zealand Standard AS/NZS 4934.1:2014 – Incandescent lamps for general lighting services – Part 1: Test methods – Energy performance.

Note: 	AS/NZS 4934.1:2014 is available from Standards Australia Limited.

AS 4934.2:2021 means Australian Standard 4934.2:2021 – Incandescent lamps for general lighting services, Part 2: Energy performance and marking requirements.

Note:	AS 4934.2:2021 is available from Standards Australia Limited.

CIE standard 015 means International Commission on Illumination standard 015 – Colorimetry.

Note: 	In 2022, the CIE standard 015 was available from https://www.techstreet.com.

Defence Aviation Safety Authority Regulations, (DASR) 139 – Aerodromes means the Defence Aviation Safety Regulation (DASR) 139 – Aerodromes published by the Australian Government Defence Aviation Safety Authority (DASA).

Note:	The Defence Aviation Safety Authority Regulations, (DASR) 139 – Aerodromes could in 2022 be viewed on the Department of Defence website (https://www.defence.gov.au/).

IEC 60061-1 means the International Electrotechnical Commission - Lamp caps and holders together with gauges for the control of interchangeability and safety. Part 1: Lamp caps.

Note 1: 	IEC 60061-1 is available from Standards Australia Limited.

Note 2: 	IEC 60061-1 includes all amendments up to and including IEC 60061-1:1969/AMD62:2022.

IEC 60630 means International Electrotechnical Commission - Maximum lamp outlines for incandescent lamps.

Note 1:	IEC 60630 is available from Standards Australia Limited.

Note 2: 	IEC 60630 includes all amendments up to and including IEC 60630 1994/Amdt 7 2014.

Definitions—other expressions used in this instrument

Note:	A number of expressions used in this instrument are defined in section 5 of the Act, including the following:

●	category A product;

●	covered by;

●	family of models;

●	GEMS;

●	GEMS labelling requirements;

●	GEMS level requirements;

●	model;

●	product classes.

In this instrument:

Act means the Greenhouse and Energy Minimum Standards Act 2012.

Australian/New Zealand Standard means a standard that is jointly published by Standards Australia and Standards New Zealand, is applicable in both countries and denoted by the letters “AS/NZS” and identifying numbers and/or letters.

carbon filament lamp means a vacuum, or inert gas-filled, lamp whose luminous element is a filament of carbon, housed in a glass envelope.

chromaticity coordinates means the x and y coordinates represented on the 1931 CIE colour space as published by the International Commission on Illumination (CIE) in CIE standard 015.

general lighting services means substantially uniform lighting of an area without provision for special local requirements.

Note:	This is the same meaning as in clause 845-29-007 of IEC 60050-845 Ed.2, International electrotechnical vocabulary (IEV) – Part 845: Lighting.

incandescent lamp means a tungsten filament lamp or a carbon filament lamp or a tungsten halogen lamp.

lumen maintenance factor means the ratio of the luminous flux of a lamp at a given time in its operational life to its initial luminous flux, the lamp being operated under specified conditions.

Note:	Specific conditions are described at section 24 of this instrument.

luminaire means an apparatus which distributes, filters or transforms the light transmitted from at least one source of optical radiation and which includes, except the sources themselves, all the parts necessary for fixing and protecting the sources and, where necessary, circuit auxiliaries together with the means for connecting them to the power supply.

median life, for a model of a lamp, means the duration, in operating hours, after which 50% of a representative group of lamps have survived when operated under specified test conditions.

Note:	This is often referred in product catalogues as ‘average lamp life’.

tungsten filament lamp means a vacuum, or inert gas-filled, lamp whose luminous element is a filament of tungsten, housed in a glass envelope.

Note:	This is the same meaning as in subclause 1.3.13 AS/NZS 4934.1:2014.

tungsten halogen lamp means a gas-filled lamp containing halogen or a halogen compound and a tungsten filament, housed in a glass envelope.

Note:	This is the same meaning as in subclause 1.3.14 AS/NZS 4934.1:2014.

Applicable definitions and applicable versions of standards and documents incorporated into standards

Applicable definitions of terms or phrases

If there is inconsistency in the definitions of words or expressions, words or expression will be interpreted in the following order of priority to the extent of any inconsistency:

the Act;

this instrument;

a standard referred to in this instrument, or another standard referred to in such a standard.

A word or expression defined in a standard mentioned specifically in section 6 has the meaning given to it in the relevant standard.

Note: 	Notwithstanding this, for convenience to users, the key terms for ascertaining if a product is covered by this instrument are set out in this instrument.

Applicable version of documents incorporated into standards

For the purposes of this instrument the applicable version of any:

standard; or

other document that:

is referred to in a standard under the heading ‘Referenced Documents’, or under an equivalent heading; and

must be applied to give effect to this instrument or a standard referred to in this instrument;

is the version of the standard or other document that existed at the date this instrument was made.

Example: 	Subclause 5.1.1. of AS 4934.2:2021 requires that a test report in accordance with AS/NZS 4934.1:2014 be submitted by a manufacturer. The applicable version of AS/NZS 4934.1:2014 is the version that existed at the date this instrument was made.

Families of models

For section 28 of the Act, for a particular product class covered by this instrument, two or more models are in the same family of models if:

they are members of a family that has been declared to the GEMS Regulator; and

the requirements of this section are satisfied in relation to the models and the family.

For subsection (1), the models must:

be in the same product class; and

be of a single brand; and

rely on the same test report that sets out the results of testing conducted in accordance with sections 17, 20 and 24; and

for the purposes of complying with sections 17, 20 and 24, have the same:

overall size; and

geometric form factor; and

other dimensions, components or component arrangements that may affect performance; and

wattage; and

median life; and

lumen maintenance factor.

For subsection (1), a model cannot be a member of a family of models if its inclusion in that family would lead to the family consisting of more than 8 models.

Note: 	For paragraph (2)(c), if a single test report is used for a family registration that contains models exhibiting different efficacies (for example, frosted and clear models) the test report should show the results for the model with the lowest efficacy of the family, noting that all models must meet the initial efficacy requirement outlined in section 17.

Product category

For section 29 of the Act, the products covered by this instrument are category A products.

Limited grandfathering

For section 31 of the Act, the limited grandfathering period for product class 1 and 2 in the table at section 14 of this instrument ends at the end of the period of 5 years that begins on the day this instrument comes into force.

Registrations affected by this instrument

For section 36 of the Act, this instrument affects the registration of the models registered against the Greenhouse and Energy Minimum Standards (Incandescent Lamps for General Lighting Services) Determination 2016 that are specified in Schedule 1.

Note 1:	If a model’s registration is not affected, the model is taken to be registered against this instrument. See section 36 of the Act.

Note 2:	If a model’s registration is affected, the model’s registration against the Greenhouse and Energy Minimum Standards (Incandescent Lamps for General Lighting Services) Determination 2016 ceases to be in force. See section 48 of the Act.

—Products covered by instrument

Purpose of Part

For subsections 23(1) and (2) of the Act, this Part specifies:

one or more classes of products that are covered by this instrument; and

one or more classes of products that are not covered by this instrument.

Classes of products that are covered by this instrument

This instrument covers incandescent lamps that:

are designed to produce a visible, optical radiation used in general lighting services; and

are supplied or used as individual lamps or as part of a luminaire; and

are listed in the product classes set out in the following table:

| Product classes |  |  |
| --- | --- | --- |
| Column 1 | Column 2 | Column 3 |
| Product class | Products covered by class | Product class characteristics |
| 1 | Mains voltage incandescent lamps as described at subclause 1.1.2.1 of AS 4934.2:2021 | This product class comprises products with the following attributes: (a) rated voltage >140 V a.c.; (b) lamp burner: tungsten filament, tungsten halogen or carbonised material; (c) cap: E13, E14, E26, E27, BA15d, B15d, B22d, GU10, or GZ10 (as described in IEC 60061-1). |
| 2 | 11-13 volt incandescent lamps as described at subclause 1.1.2.2 of AS 4934.2:2021 | This product class comprises products with the following attributes: (a) rated voltage ≥11V and ≤13V a.c.; (b) lamp burner: tungsten filament, tungsten halogen or carbonised material; (c) cap: G4, GY6.35, GU4, GZ4, GU5.3, GX5.3 or G53 (as described in IEC 60061-1). |

Note 1:	This subsection reflects the scope specified in clause 1.1.2 of AS 4934.2:2021 and clause 1.1 of AS/NZS 4934.1:2014.

Note 2:	Products that are not covered by these classes are not covered by this instrument. In addition, section 15 sets out classes of products that are not covered.

Classes of products that are not covered by this instrument

This instrument does not cover lamps specified in Schedule 2.

—GEMS level requirements

Purpose of Part

For paragraph 24(1)(a) of the Act, this Part specifies GEMS level requirements in accordance with section 25 of the Act for the product classes covered by this instrument.

GEMS level requirements

Energy use and greenhouse gas production

Products in product class 1 must satisfy the following initial efficacy requirement:

Mean value of initial efficacy≥10.24L+0.0103

where:

L is the mean measured initial luminous flux of the products in lumens.

Products in product class 2 must satisfy is the following initial efficacy requirement:

Mean value of initial efficacy≥2.8×lnL-4.0

where:

L is the mean measured initial luminous flux of the products in lumens.

Testing requirements

In order to determine whether products in product class 1 or product class 2 meet the requirements specified in section 17, tests must be conducted in accordance with the requirements mentioned in section 2 of AS/NZS 4934.1:2014.

A test report must show conformance to AS 4934.2:2021 in accordance with AS/NZS 4934.1:2014 for each model tested. The test report must contain the applicable information mentioned in Appendix A of AS 4934.2:2021.

Test measurements must only be recorded following the required duration for ageing, which is 100 ±1 h unless the lamp manufacturer specifies a lesser duration. Lamps that fail prior to 100 h must not be included in the mean and must be replaced with another lamp.

The minimum sample size required for testing is 10.

—GEMS labelling requirements

Purpose of Part

For paragraph 24(1)(b) of the Act, this Part specifies GEMS labelling requirements in accordance with section 26 of the Act for the product classes covered by this instrument.

GEMS labelling requirements

The following information must be marked on all lamp packaging:

light output in lumens: the rated value must be no more than the mean measured initial value times 1.1, rounded to the nearest 10 lumens;

power in watts: the rated value must be no less than the mean measured value times 0.9 rounded to the nearest watt, and no more than the mean measured value times 1.1, rounded to the nearest watt;

median life: the marked value must not be more than the median measured value as calculated from the lifetime test value times 1.1 rounded to the nearest 100 hours.

Testing requirements

In order to rate products in product class 1 and product class 2 against criteria specified in section 20, tests must be conducted in accordance with the requirements mentioned in section 2 of AS/NZS 4934.1:2014.

Impact of replacement determination

A GEMS labelling requirement of this instrument (the revoked requirement) is taken to be complied with if:

this instrument is revoked in accordance with paragraph 35(1)(a) of the Act; and

another GEMS determination (the replacement determination) is made in accordance with paragraph 35(1)(b) of the Act; and

a transitional GEMS labelling requirement (the replacement requirement) of the replacement determination provides that, if the replacement requirement is complied with, the revoked requirement is taken to be complied with.

—Other requirements

Purpose of Part

For subsection 24(2) of the Act, this Part specifies other requirements in accordance with section 27 of the Act for product classes covered by this instrument.

Other GEMS requirements

Lumen maintenance factor

For products in product class 1 and product class 2, when measured at 75% of rated median life, the mean lumen maintenance factor must be at least 80%.

Minimum median life

For products in product class 1 and product class 2, the minimum median life is 2000 hours.

Product performance—product class 2

Products in product class 2 must satisfy the maximum wattage requirements mentioned in clause 2.6 of AS 4934.2:2021.

Testing requirements

In order to determine whether products in product class 1 and product class 2 meet the requirements specified in section 24, tests must be conducted in accordance with the requirements mentioned in section 2 of AS/NZS 4934.1:2014.

For subsection 24(1), lamps that fail prior to 75% of rated median lamp life must not be included in the mean.

The minimum sample sizes required for testing are as follows:

for lumen maintenance factor—10;

for minimum median life—20;

for maximum wattage—10.

Schedule 1—Registrations affected by this instrument

Note:	 See section 12 of this instrument.

- Table 1 – Registrations affected by this instrument

The following table sets out the models whose registration is affected by this instrument.

| Brand | Model Identifier | Registration Number |
| --- | --- | --- |

Schedule 2—Products not covered by this instrument

Note:	 See section 15 of this instrument.

Aeronautical lamps

Aircraft lamps

This instrument does not cover lamps that:

(a)	are designed to operate on an aircraft; and

(b)	satisfy the requirements of:

(i)	the airworthiness standards in subsections 23.001(1), 25.001(1), 27.001(1) and 29.001(1) of the Civil Aviation Safety Regulations 1998; or

(ii)	Section 3, Chapter 7 (Lighting Systems) of the Airworthiness Design Requirements Manual published by the Defence Aviation Safety Authority as in force on the date this instrument was made.

Note 1:	In 2022, the Civil Aviation Safety Regulations 1998 were available at https://www.legislation.gov.au, while the Airworthiness Design Requirements Manual was available at https://www.defence.gov.au.

Note 2:	The provisions of the Civil Aviation Safety Regulations 1998 mentioned in subparagraph (b)(i) refer to the European Aviation Safety Agency certification specifications and US Federal Aviation Administration airworthiness standards for aircraft.

Note 3:	The GEMS Regulator may request evidence that such lamps comply with the approval requirements mentioned in paragraph (b) (among other things).

Aeronautical ground lights

This instrument does not cover lamps that:

(a)	are designed as aeronautical ground lights; and

(b)	meet the requirements that apply to such lights under one or more of the following:

Part 139 (Aerodromes) Manual of Standards 2019 as in force on the date this instrument was made;

the Australian Government Defence Aviation Safety Authority Regulations, DASR 139 – Aerodromes as in force on the date this instrument was made.

Note 1: 	Aeronautical ground lights are also used for aerodromes, heliports and obstacles (objects that project through protected surfaces or are otherwise determined to be a hazard of aircraft safety).

Note 2:	Part 139 (Aerodromes) Manual of Standards 2019 is an instrument made under regulation 139.005 of the Civil Aviation Safety Regulations 1998.

Note 3:	In 2022, the Defence Aviation Safety Authority Regulations were available at https://defence.gov.au/.

Note 4: 	The GEMS Regulator may request evidence that aeronautical ground lights comply with the requirements specified in paragraph (b) (among other things).

In this clause, aeronautical ground light has the meaning given by Part 139 (Aerodromes) Manual of Standards 2019 as in force on the date this instrument was made.

Other exclusions

This instrument does not cover the following:

high temperature lamps;

infrared heat lamps;

low power decorative lamps.

In this clause:

Note 1:	This subclause reflects the exclusions in clause 1.1.3 of AS 4934.2:2021.

Note 2: 	For the lamps listed in this subclause it is recommended that packaging and accompanying product information state clearly and prominently their purpose and that they are not intended for general purpose illumination.

high temperature lamp means a lamp designed to be capable of operating at rated voltage in a 250°C oven for a minimum of 200 h, with no melting of solder and no oxidation on or near the lamp pinch area and no degradation of cement if present.

Example:  Oven lamps.

Note:  	A test report documenting correct operation at these temperatures may provide evidence of this attribute.

infrared heat lamp means lamps with all of the following attributes:

(a)	Infrared lamp designed solely for the purpose of heating;

(b)	Cap: E27 (as described in IEC 60061-1);

(c)	Shape: R63, R75, R125 (as described in IEC 60630);

(d)	Rated power: 50, 100, ≥ 150 W;

(e)	either of the following:

(i)	Correlated Colour Temperature (CCT) that satisfies the following formula:

CCT<2,500

(ii)	chromaticity coordinates that satisfy the following:

-2.31x2+2.3653x-0.1<y<-2.3172x2+2.3653x-0.28

Note 1: 	These excluded lamps must be < 2500 K or otherwise away from black body locus.

low power decorative lamp means a lamp with the following attributes:

(a)	rated voltage >140 V a.c.;

(b)	cap: E13, E14, E26, E27, BA15d, B15d, or B22d (as described in IEC   60061-1);

(c)	shape: round (P), candle (B), pigmy (S), tubular (T), globe (G) or pilot lamps (as described in IEC 60630);

(d)	rated power < 10 W.

Schedule 3—Repeals

Greenhouse and Energy Minimum Standards (Incandescent Lamps for General Lighting Services) Determination 2016

1  The whole of the instrument

Repeal the instrument.