# DRAFT Greenhouse Gas Emissions Estimation and Reporting Guidelines for Agriculture, Fisheries and Forestry

Methodological Guidance

Chapter 4: Manure Management

© Commonwealth of Australia 2025

Ownership of intellectual property rights

Unless otherwise noted, copyright (and any other intellectual property rights) in this publication is owned by the Commonwealth of Australia (referred to as the Commonwealth).

Creative Commons licence

All material in this publication is licensed under a Creative Commons Attribution 4.0 International Licence except content supplied by third parties, logos and the Commonwealth Coat of Arms.

Inquiries about the licence and any use of this document should be submitted via our online contact form.

Cataloguing data

This publication (and any material sourced from it) should be attributed as: DCCEEW 2025, DRAFT Greenhouse Gas Emissions Estimation and Reporting Guidelines for Agriculture, Fisheries and Forestry - Methodological Guidance, Department of Climate Change, Energy, the Environment and Water, Canberra, February. CC BY 4.0.

This publication is available at dcceew.gov.au.

Department of Climate Change, Energy, the Environment and Water

GPO Box 3090 Canberra ACT 2601

Telephone 1800 920 528

Web dcceew.gov.au

Disclaimer

The Australian Government acting through the Department of Climate Change, Energy, the Environment and Water has exercised due care and skill in preparing and compiling the information and data in this publication. Notwithstanding, the Department of Climate Change, Energy, the Environment and Water, its employees and advisers disclaim all liability, including liability for negligence and for any loss, damage, injury, expense or cost incurred by any person as a result of accessing, using or relying on any of the information or data in this publication to the maximum extent permitted by law.

Acknowledgements

We thank Environmental Accounting Services for drafting this document, and the Voluntary Greenhouse Gas Estimation and Reporting Standards Reference Group for their valued contributions.

Acknowledgement of Country

We acknowledge the Traditional Owners of Country throughout Australia and recognise their continuing connection to land, waters and culture. We pay our respects to their Elders past and present.

Contents

4	Scope 1 – Manure Management	5

4.1	Beef - Feedlot	7

4.1.1	Estimation methodology	9

4.1.1.1	Method 1 — Manure Methane Beef Feedlot	9

4.1.1.2	Method 2 — Manure Methane Beef Feedlot	10

4.1.1.3	Method 1 — Manure Direct N2O Beef Feedlot	10

4.1.1.4	Method 2 — Manure Direct N2O Beef Feedlot	12

4.1.1.5	Method 1 — Manure Deposition N2O Beef Feedlot	13

4.1.1.6	Method 2 — Manure Deposition N2O Beef Grazing	13

4.1.1.7	Method 1 — Manure Leaching and Runoff N2O Beef Feedlot	13

4.1.1.8	Manure Applied to Soils	13

4.1.2	Data/Parameters	15

4.1.2.1	Input Data (Required)	15

4.1.2.2	Data (Method 1 and 2 Options)	17

4.1.2.3	Constants	18

4.2	Beef - Pasture, Rangeland and Paddock	22

4.2.1	Estimation methodology	24

4.2.1.1	Method 1 — Manure Methane Beef Grazing	24

4.2.1.2	Method 2 — Manure Methane Beef Grazing	25

4.2.1.3	Method 1 — Soil Direct N2O Beef Grazing	25

4.2.1.4	Method 2 — Soil Direct N2O Beef Grazing	27

4.2.1.5	Method 1 — Soil Deposition N2O Beef Grazing	27

4.2.1.6	Method 2 — Soil Deposition N2O Beef Grazing	28

4.2.1.7	Method 1 — Soil Leaching and Runoff N2O Beef Grazing	28

4.2.1.8	Method 2 — Soil Leaching and Runoff N2O Beef Grazing	28

4.2.2	Data/Parameter tables	29

4.2.2.1	Input data (Required)	29

4.2.2.2	Data (Method 1 and 2 options)	30

4.2.2.3	Constants	33

4.3	Dairy	38

4.3.1	Estimation methodology	40

4.3.1.1	Method 1 — Manure Methane Dairy	40

4.3.1.2	Method 2 — Manure Methane Dairy	41

4.3.1.3	Method 1 — Manure Direct N2O Dairy	41

4.3.1.4	Method 2 — Manure Direct N2O Dairy	44

4.3.1.5	Method 1 — Manure Deposition N2O Dairy	44

4.3.1.6	Method 1 — Manure Leaching and Runoff N2O Dairy	44

4.3.1.7	Method 2 — Manure Leaching and Runoff N2O Dairy	45

4.3.1.8	Manure Applied to Soils	45

4.3.1.9	Method 1 — Soil Direct N2O Dairy	46

4.3.1.10	Method 1 — Soil Deposition N2O Dairy	46

4.3.1.11	Method 2 — Soil Deposition N2O Dairy	47

4.3.1.12	Method 1 — Soil Leaching and Runoff N2O Dairy	47

4.3.1.13	Method 2 — Soil Leaching and Runoff N2O Dairy	48

4.3.2	Data/Parameters	49

4.3.2.1	Input Data (Required)	49

4.3.2.2	Data (Method 1 and 2 Options)	50

4.3.2.3	Constants	54

4.4	Sheep	61

4.4.1	Estimation methodology	63

4.4.1.1	Method 1 — Manure Methane Sheep	63

4.4.1.2	Method 2 — Manure Methane Sheep	64

4.4.1.3	Method 1 — Soil Direct N2O Sheep	64

4.4.1.4	Method 2 — Soil Direct N2O Sheep	67

4.4.1.5	Method 1 — Soil Deposition N2O Sheep	67

4.4.1.6	Method 2 — Soil Deposition N2O Sheep	68

4.4.1.7	Method 1 — Soil Leaching and Runoff N2O Sheep	68

4.4.1.8	Method 2 — Soil Leaching and Runoff N2O Sheep	68

4.4.2	Data/Parameters	69

4.4.2.1	Input Data (Required)	69

4.4.2.2	Data (Method 1 and 2 Options)	70

4.4.2.3	Constants	73

4.5	References	77

Figures

Figure 4.1: Example possible emissions sources from manure for livestock enterprises	6

Figure 4.2: Summary of emissions sources associated with manure management in beef feedlot systems	7

Figure 4.3 Example of the possible different stages of manure management in beef feedlot systems from the Australian National Inventory Report [1].	8

Figure 4.4: Emissions that result from manure of beef cattle on pasture, rangeland and paddock	22

Figure 4.5: Emissions that result from dairy cattle manure	38

Figure 4.6: Emissions that result from sheep manure	61

# Scope 1 – Manure Management

Where appropriate, for each sector this chapter covers the following manure management systems:

Manure Management Systems (MMS)

| Subscript (m) | MMS |
| --- | --- |
| 1 | Anaerobic lagoon |
| 2 | Liquid systems |
| 3 | Daily spread |
| 3a | Sump and dispersal |
| 3b | Drains to paddock |
| 4 | Solid Storage |
| 5 | Drylot |
| 6 | Composting (passive windrow) |
| 7 | Digestor/Covered Lagoon |
| 8 | Deep Litter |
| 9 | Pit storage (<1 month) |
| 10 | Poultry manure with bedding |
| 11 | Poultry manure without bedding |
| 12 | Direct Processing |
| 13 | Direct Application |
| 14 | Pasture range and paddock |

Figure 4.1 demonstrates the relevant emissions calculations outlined in this chapter to each of these MMS. There two main components to methane emission estimates – manure excreted directly onto pasture, and managed manure through systems for housed animals. Managed manure also results in direct and indirect nitrous oxide emissions. For manure excreted to pasture there are no nitrous oxide emissions from manure management, however emissions from the soil occur from the metabolism of the deposited urine and faeces, and indirectly through atmospheric deposition and leaching and runoff. Note that emissions associated with manure from management systems that is subsequently applied to soils are covered in the organic fertiliser section in Chapter 5 Section 5.1 Fertiliser module.

Figure 4.1: Example possible emissions sources from manure for livestock enterprises

## Beef - Feedlot

This module covers the estimation of methane and nitrous oxide that results from beef feedlot cattle manure management. Figure 4.2 summarises the emission sources associated with the manure from beef feedlot cattle. As cattle are housed the pasture, range and paddock MMS is not applicable in this chapter. However, emissions associated with manure from management systems that is subsequently applied to soils are covered in the organic fertiliser section in Chapter 5 Section 5.1 Fertiliser module.

Figure 4.2: Summary of emissions sources associated with manure management in beef feedlot systems

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| i | Climate zone |
| j | Cattle group |
| m | Manure Management System (MMS) |
| T | Treatment Stage |

Emissions are estimated based on groups of cattle with similar intake requirements and lengths of stay on the feed pad and the different stages and systems of manure treatment. The climate zone will affect the selection of appropriate emission factors. The emissions are summed across each group and manure management system within the feedlot operation during the reporting period.

The number of groups per farm within the reporting period will depend on the size of the feedlot operation and the diversity of the intake and length of stay of the groups of cattle on the feed pad. Groups can be labelled as numbers e.g. Group 1, Group 2, Group 3 or given relevant names based on age or breed of group e.g. R2 Wagyu, R3 Angus, R3 Mixed for data entry purposes.

Manure from feedlot cattle may pass through multiple treatment stages. Therefore, volatile solid and nitrogen inputs and losses are estimated at each treatment stage (see Figure 4.3). It is assumed that the primary treatment stage (T=1) of all manure is the feedpad (considered to a drylot system). Manure may then be transferred to a secondary treatment stage (T=2), such as a stock pile, after which all waste is assumed to be applied to soils (either on-site or sold as an organic fertiliser). A fixed amount (2%) of manure is assumed to runoff from the feedpad into an anerobic lagoon, the tertiary treatment stage (T=3), before also being applied to soils.

The manure management system (m) for secondary treatment stage (T=2) will need to be input so that the appropriate constants and emissions factors can be applied. Additionally, the climate zone (i) the feedlot is operating in is needed to inform the methane conversion factor applied.

Figure 4.3 Example of the possible different stages of manure management in beef feedlot systems from the Australian National Inventory Report [1].

Question Reference 4.1.

The Guidance currently assumes that the feed pad (T2), stockpiles, passive windrows and direct application to soils (T2) and uncovered anaerobic lagoons (T3) are the only manure management system (MMS) options aligned with the NIR options.

Are there other MMS in use in the feedlot industry, is so what systems and at what treatment stage do they occur?

### Estimation methodology

#### Method 1 — Manure Methane Beef Feedlot

(1) 	Total annual methane emissions from manure management ECH4 (t CH4) is calculated as:

ECH4= jDj ×Mmj×Nj ×10-3

Where 		Dj = length of stay of each cattle group (days)

Mmj = methane production from manure in each group (kg CH4/head/day)

Nj = numbers of beef cattle in each group (head)

(2)	Methane production from the manure of each group of feedlot beef cattle Mmij (kg CH4/head/day) is calculated as:

Mmj= mTMjmT

Where: 	MjmT = methane production from manure per cattle group, MMS, and treatment stage (kg CH4/head/day)

Treatment stage 1 (primary system)

(3)	Production of methane in treatment stage 1 MMS Mijm=5 T=1 (kg CH4/head/day) is calculated as:

Mijm=5 T=1=VSj × BO×MMSm=5 T=1×MCFim=5×ρ

Where 		VSj  = volatile solid production from feedlot cattle in each group (kg/head/day)

BO = emissions potential (m3 CH4/kg VS)

MMSm=5 T=1 = the fraction of manure in each primary system. Assumed to be 1 for the drylot MMS

MCFim=5 = methane conversion factor for climate zone and system. Under Method 1, the default state climate zone is applied

ρ = density of methane (kg/m3)

Treatment stage 2 (secondary system)

(4)	Production of methane in each treatment stage 2 MMS MijmT=2 (kg CH4/head/day) is calculated as:

MijmT=2=VSj ×0.5×BO× MMSmT=2×MCFim×ρ

Where	0.5 = fraction of volatile solids lost during storage in the primary system, (based on McGahan, et al. (2004) [2] and Wiedemann et al. (2014) [3])

MMSmT=2 = the fraction of manure in each secondary system

MCFim = methane conversion factor for climate zone and MMS. Under Method 1, the default state climate zone is applied

Note: if all manure is directly applied to soils after it is scraped from the feed pad then methane emissions from the secondary stage is assumed to be zero i.e. if MMSjm=13T=2=1  then Mijm=13T=2=0.

Treatment stage 3 (tertiary system)

(5)	Production of methane in treatment stage 3 MMS Mijm=1 T=3 (kg CH4/head/day) is calculated as:

Mijm=1 T=3=VSj × BO× MMSm=1T=3×MCFim=1×ρ

Where	MMSm=1 T=3 = the fraction of manure to the tertiary system. Assumed to be 0.02 to an anaerobic lagoon MMS

MCFim=1 = methane conversion factor for climate zone and anerobic lagoon system. Under Method 1, the default state climate zone is applied

(6)	Volatile solid production VSj (kg/head/day) based on the BeefBal model (McGahan et al. (2004)) [2] is calculated as:

VSj=Ij × 1-DMDj× 1-A

Where		Ij  = dry matter intake for each group (kg DM/head/day)

DMDj = dry matter digestibility for each group (fraction)

A = ash content of manure (fraction)

#### Method 2 — Manure Methane Beef Feedlot

Method 2 is the same as Method 1 except that under equations 4.1.1.1 (3) – (5) farm specific climate zone is required for MCFim=1.

#### Method 1 — Manure Direct N2O Beef Feedlot

(1)	The total annual direct nitrous oxide emissions from manure management systems EN2O,dir (t N2O) is calculated as:

EN2O,dir= jmTMNjmT×EFjm×Cg,N2O ×10-3

Where: 	MNjmT = nitrogen per cattle group, MMS and treatment stage (kg N2O)

EFjm = nitrous oxide emission factor for MMS (kg N2O-N/kg N)

Cg,N2O = 44/28 factor to convert elemental mass of nitrous oxide to molecular mass

Note: exclude MNjm=13T=2 which moves directly to MNsoil calculation (equation 4.1.1.7(1))

Treatment stage 1 (primary system)

(2)	Nitrogen in the treatment stage 1 MMS MNjm=5 T=1 (kg N2O) is calculated as:

MNjm=5 T=1= AEj×MMSjm=5 T=1

Where: 	AEj = total nitrogen excreted by each cattle group (kg N)

MMSjm=5 T=1 = fraction of manure in primary system. Assumed to be 1 for the drylot MMS

(3)	Annual nitrogen excretion AEj (kg N) from each feedlot cattle group is calculated as:

AEj=Nj × NEj×Lj

Where		Nj = number of cattle in each group (head)

NEj = nitrogen excretion (kg N/head/day)

Lj = length of stay of cattle group (days)

(4)	Nitrogen excretion NEj (kg/head/day) is calculated by:

NEj=NIj × 1-NRj

Where		NIj = nitrogen intake (kg/head/day)

NRj = nitrogen retention expressed as a fraction of intake (per cent).

Under Method 1, default Ij and NRj values are applied.

(5)	Nitrogen intake NIj (kg/head/day) of feedlot cattle is calculated by:

NIj=Ij × CPj÷6.25

Where		Ij = dry matter intake as calculated for enteric methane (kg DM/head/day)

CPj = crude protein content of feed (fraction)

6.25 = factor for converting crude protein into nitrogen

Under Method 1, CPj values are applied.

Treatment stage 2 (secondary system)

(6)	Nitrogen in each treatment stage 2 MMS MNjm T=2 (kg N2O) is calculated as:

MNjm T=2= NTjm T=2×MMSjmT=2

Where 	NTjm T=2 = nitrogen transferred to secondary treatment MMS (kg N)

MMSjmT=2 = fraction of manure in each to secondary system.

Note: if direct application occurs at treatment stage 2 the nitrogen (MNjm=13T=2) moves directly to MNsoil calculation (equation 4.1.1.7(1)) and is not included in direct nitrous oxide from manure management equation (Equation 4.1.1.3 (1)).

(7)	Nitrogen transferred to treatment stage 2 MMS NTjmT=2 (kg N) is calculated as:

NTjmT=2=MNjm=5T=1×1-FracGASMm=5-EFm=5-MNjmT=3

Where 		MNjmT=1 = nitrogen in MMS treatment stage 1 (kg N)

FracGASMm=5 = fraction of N volatilised from drylot MMS

MNjmT=3 = nitrogen in MMS treatment stage 3 (kg N)

Treatment stage 3 (tertiary system)

(8)	Nitrogen in treatment stage 3 MMS MNjm=1 T=3 (kg N) is calculated as:

MNjm=1 T=3= AEj×MMSjm=1 T=3

Where	 	AEj = total nitrogen excreted by each cattle group (kg N)

MMSm=1 T=3 = fraction of manure to tertiary system. Assumed to be 0.02 to anaerobic lagoon MMS

#### Method 2 — Manure Direct N2O Beef Feedlot

Method 2 is the same as Method 1 except that under equation 4.1.1.3 (4) - (5) farm specific data for NRj, Ij,and CPj is required.

#### Method 1 — Manure Deposition N2O Beef Feedlot

(1)	Total annual atmospheric deposition emissions from manure management EN2O,ad (t N2O) is calculated as:

EN2O,ad=(MMSATMOS × EFad×Cg,N2O) ×10-3

Where		MMSATMOS = mass of N volatilised from MMS (kg N)

EFad = emission factor for atmospheric deposition (kg N2O-N/kg N)

(2) Volatilised nitrogen from MMSMMSATMOS (kg N) is calculated as:

MMSATMOS=jmTMNjmT×FracGASMmT

Where	MNjmT = nitrogen in each treatment stage MMS (kg N) (excluding MNijm=13T=2)

FracGASMmT = fraction of N volatilised in each MMS ((kg NH3–N + NOx–N)/kg N)

Note: in the case of direct application at treatment stage 2 the nitrogen (MNjm=13T=2) moves directly to MNsoil calculation (equation 4.1.1.7 (1)) and is not included in the atmospheric deposition from manure management calculation.

#### Method 2 — Manure Deposition N2O Beef Grazing

There is no Method 2 quantification option for this emission source.

#### Method 1 — Manure Leaching and Runoff N2O Beef Feedlot

Due to strict environmental controls leaching from feedlot MMS are assumed to be zero. Run-off from MMS is included in the treatment stage 3 of the direct MMS estimates.

#### Manure Applied to Soils

It is assumed that all manure managed through MMS (m=1-13) will eventually be applied to soils. The mass of nitrogen applied to soils is calculated here and then this MNSoil value is transferred to Chapter 5 Section 5.1 to estimate emissions associated with the application of organic fertilisers, including the associated atmospheric deposition, and leaching and run-off emissions.

This manure may be applied to soils within the boundary of the feedlot or may be sent off-site and applied to another farming system or enterprise. Where manure is applied to soils within the boundary of the farming enterprise MNSoilscope1, the emissions from shall be estimated and reported as Scope 1 emissions. Where the manure is used on other farming enterprises MNSoilscope3, the emissions from shall be estimated and reported as Scope 3 emissions under Scope 3 Category 5 – waste generated in operations (see Chapter 7 Section 7.10).

(1)	The mass of nitrogen applied to soils for scope 1 emissions MNSoilscope1 (kg N) is calculated as:

MNSoilscope1=jmTMNjmT=2,3×1-EFmT=2,3-FracGASMmT=2,3 × PF

Where 	MNjmT=2,3 = mass of N in secondary and tertiary treatment MMS stages as calculated for manure management (kg N)

EFmT=2,3 = nitrous oxide emission factor for each MMS (kg N2O-N/kg N)

FracGASMmT=2,3 = fraction of animal waste N volatilised in each MMS for each cattle group ((kg NH3–N + NOx–N)/kg N)

PF = fraction of manure applied to soil within the feedlot boundary

Note:  where direct application occurs at treatment stage 2 (MNjm=13T=2,3), EFjm=13T=2 and FracGASMjm=13T=2 are set to zero.

(2)	The mass of nitrogen applied to soils for scope 3 emissions MNSoilscope3 (kg N) is calculated as:

MNSoilscope3=jmTMNjmT=2,3×1-EFmT=2,3-FracGASMmT=2,3 × (1-PF)

This approach assumes that the manure used off-site within a different farm enterprise is considered a “waste product” from the farm, rather than a co-product.

If the manure is sold for use as an organic fertiliser elsewhere, under the GHG Protocol the manure would be considered a co-product and thus would require some of the total farm emissions allocated to it.

Furthermore, the emissions from the application of manure to soils would then fall into a use-of-sold-products emissions category, which is outside of the cradle-to-gate boundary.

The exact wording used in this section is therefore subject to review and non-specific  until the Common Reporting Framework provides guidance on emission allocation between co-products; and whether manure sold off-site is considered a co-product or waste stream.

### Data/Parameters

#### Input Data (Required)

| Data / Parameter | Dj |
| --- | --- |
| Data unit | days |
| Description | Duration of stay for each cattle group |
| Data source | Farm stock records; system type records or purchase and sales can be evaluated to determine average length of stay |
| Quality assurance / quality control considerations | Ensure average length of stay aligns with duration of stay expected for feedlot cattle class in the National Inventory Report. |

| Data / Parameter | Nj |
| --- | --- |
| Data unit | head |
| Description | Number of beef cattle in each group |
| Data source | Farm stock records e.g. head counts, diary entries, purchase, and sales records |
| Quality assurance / quality control considerations | All animals purchased or sold in the reporting period are assumed to be reported. Number of cattle reported can be cross checked with stocking density allowances for system size. If value is significantly higher or lower than expected stocking density, flagged as possibly erroneous. Check the class of animals on farm align with reported products from farm. |

| Data / Parameter | Ij |
| --- | --- |
| Data unit | kgDM/head/day |
| Description | Average dry matter intake per head of each group of cattle within feedlot system per day. National default data based on length of stay is available if intake of a group is not known. |
| Data source | Farm records relating to dry matter assessment of total mixed ration (TMR), and daily feed and feed waste weights for each group |
| Quality assurance / quality control considerations | Intake can be compared to inventory defaults for the relevant cattle class. If significantly higher or lower than inventory defaults check farm records to ensure correct data entry. |

Question Reference 4.2:

If a feedlot has more than one manure management system (MMS), should specific guidance be provided for calculating the percentage of manure allocated to each system? Are there existing tools or methodologies available to support this calculation?

| Data / Parameter | MMSmT=2 |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of manure at treatment stage 2 for each manure management system i.e. what fraction of manure goes to what MMS once it has been removed from the feed pad. If there is only one available MMS then this should be equal to 1. This may be different for different cattle groups. |
| Data source | Farmer records as to what fraction of manure flows to each system based on the available MMS options (m). This fraction should be based on either weight or volume of manure. If treatment is different for different cattle groups this can also be recorded per cattle group. |
| Quality assurance / quality control considerations | Only those specific MMS options (m) addressed by the guidance should be included as options to assign fractions to. The fraction of manure to each manure management system should sum to one. If only one MMS used to treat manure, then fraction equal to 1. Note: if manure is directly applied to soils after it is scraped from the feedlot then methane and nitrous oxide emissions will be zero for this secondary treatment stage MMS. The nitrogen in the manure (MNjm=13T=2) moves directly to MNsoiljm calculation (equation 4.1.17(1)) (see also Chapter 5 Section 5.1). It is excluded from equation 4.1.1.3 (1) and 4.1.1.4 (2) |

| Data / Parameter | PF |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of manure applied to soil within the feedlot boundary |
| Data source | Farmer records of manure sales or transport weights compared to total manure treated on farm. |
| Quality assurance / quality control considerations | If all of manure is not applied to soil within the feedlot boundary ensure it is appropriately tracked what proportion is sent off-site to other entities. If it is not known what proportion is sent off-site to other entities assume all manure is applied within in the feed lot boundary until better data can be collected. |

#### Data (Method 1 and 2 Options)

| Data / Parameter | DMDi |
| --- | --- |
| Data unit | fraction |
| Description | Dry matter digestibility of diet used within feedlot system |
| Method 1 data source | Table A5.5.3.2, National Inventory Report, Volume 2 [4] |
| Method 1 value | See Appendix Table 12.1.2.3 in Chapter 12 Apply the dry matter digestibility that best represents the duration of stay of each cattle class. |
| Method 2 data source | Farm records or feed receipt stating dry matter digestibility of TMR |
| Quality assurance / quality control considerations | If Method 2 data is used compare this input to inventory defaults for the relevant cattle class. If significantly higher or lower than inventory defaults check farm records to ensure correct data entry. |

Question Reference 4.3:

Is farm-specific data on the crude protein content (CP) of the ration readily available? If so should farm-specific data for CP be required under Method 1?

Are there specific tools or industry guidance that could be referenced to support feedlot producers to calculate CP?

| Data / Parameter | CPj |
| --- | --- |
| Data unit | fraction |
| Description | Average crude protein content of the feed given to each cattle group. National default data based on length of stay is available if crude protein content of TMR for a group is not known. |
| Source of data (T1) | Table A5.5.3.2, National Inventory Report, Volume 2 [4] |
| Default (T1) Value | See Appendix Table 12.1.2.3 in Chapter 12 Apply the crude protein per cent that best represents the duration of stay of each cattle class. |
| Source of optional data (T2) | Farm records or feed receipt stating crude protein of TMR |
| Quality assurance / quality control considerations | If T2 data is used compare this input to inventory defaults for the relevant cattle class. If significantly higher or lower than inventory defaults check farm records to ensure correct data entry. |

#### Constants

| Data / Parameter | B0 |
| --- | --- |
| Data unit | m3 CH4/kg VS |
| Description | Emissions potential. |
| Data source | IPCC (2019), Chapter 10 [5] |
| Value | 0.19 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report. |

| Data / Parameter | MMSm=5T=1 |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of volatile solids to each primary system, assumed all primary systems are drylot. |
| Data source | Table A5.5.3.4, National Inventory Report, Volume 2 [4] |
| Value | 1 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report. |

| Data / Parameter | p |
| --- | --- |
| Data unit | kg/m3 |
| Description | Density of methane. |
| Data source | National Greenhouse and Energy Reporting (Measurement) Determination 2008 [6] |
| Value | 0.6784 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

A Method 2 option for methane conversion factors (MCF) is currently under consideration. This approach would require selection of MCF base on site-specific climate zone. This will be further developed in Tranche 2 as it is also relevant for piggeries and poultry production.

The climate zone specific methane conversion factors for feedlot cattle are also under review and may change in future revisions.

| Data / Parameter | MCFimT |
| --- | --- |
| Data unit | fraction |
| Description | Methane conversion factor based on State, and system at each treatment stage |
| Data source | Table A5.5.3.5, National Inventory Report, Volume 2 [4] |
| Value | See Appendix Table 12.1.2.5 in Chapter 12 Apply the methane conversation factor that best represents the MMS in each treatment stage. |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

| Data / Parameter | MMSm=1T=3 |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of volatile solids to each tertiary system assumed that 2 per cent is run-off from the feed pad. |
| Data source | Table A5.5.3.4, National Inventory Report, Volume 2 [4] |
| Value | 0.02 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

| Data / Parameter | A |
| --- | --- |
| Data unit | per cent |
| Description | Ash content of manure. |
| Data source | National Greenhouse Gas Inventory Report Vol 1. (2023) [1] |
| Value | 0.16 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report. |

| Data / Parameter | Cg,N2O |
| --- | --- |
| Data unit | fraction |
| Description | Factor to convert elemental mass of nitrous oxide to molecular mass. |
| Data source | IPCC Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories. Chapter 5: Agriculture [7] |
| Value | 44/28 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report. |

Question Reference 4.4:

Is farm-specific data on nitrogen retention (NR) readily available? If so should farm-specific data for NR be required under Method 1?

Are there specific tools or industry guidance that could be referenced to support feedlot producers to calculate NR?

| Data / Parameter | NRj |
| --- | --- |
| Data unit | per cent |
| Description | Nitrogen retention expressed as a per cent of intake based on length of stay of each cattle group on the feed pad. |
| Data source | Table A5.5.3.1, National Inventory Report, Volume 2 [4] |
| Value | See Appendix Table 12.1.2.2 in Chapter 12 Apply the nitrogen retention that best represents the duration of stay of each cattle class. |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

| Data / Parameter | EFmT |
| --- | --- |
| Data unit | kg N2O-N/kg N |
| Description | Nitrous oxide emission factor for each MMS. |
| Data source | Table A5.5.3.6, National Inventory Report, Volume 2 [4] |
| Value | See Appendix Table 12.1.2.6 in Chapter 12 Apply the EF that best represents the MMS in each treatment stage. |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

| Data / Parameter | FracGASMmT |
| --- | --- |
| Data unit | (kg NH3–N + NOx–N)/kg N |
| Description | Fraction of N volatilized in each MMS. |
| Data source | Table A5.5.3.7, National Inventory Report, Volume 2 [4] |
| Value | See Appendix Table 12.1.2.7 in Chapter 12 Apply the FracGASM that best represents the MMS in each treatment stage. |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

| Data / Parameter | EFad |
| --- | --- |
| Data unit | kg N2O-N/kg N |
| Description | Emission factor for atmospheric deposition |
| Data source | Table 5.21, National Inventory Report, Volume 1 [1] |
| Value | See Appendix Table 12.2.2.3 in Chapter 12 Apply the atmospheric deposition EF that best represent the production systems immediately surrounding the feedlot. For non-irrigated cropping systems, the emission factor varies depending on whether the agricultural system is in a high rainfall or low rainfall zone. Refer to Chapter 1 Section 1.8 for guidance on determining the relevant rainfall zone. |
| Quality assurance / quality control considerations | Ensure the most recently available emissions factors is used in alignment with the Australian NIR. If farm rainfall records are not available for the reporting year ensure rainfall records from the Australian Bureau of Meteorology are used to ensure rainfall is taken from a credible source. |

## Beef - Pasture, Rangeland and Paddock

This module covers the estimation of methane and nitrous oxide emission that results from grazing beef cattle manure management. Figure 4.4 summarises the emission sources associated with the manure of beef cattle on pasture, rangeland and paddock (PRP). There are two components to methane emission estimates –manure excreted directly onto PRP, and manure that runs-off into natural water sources and farm dams. There are no nitrous oxide emissions from manure management for manure excreted to PRP, however emissions from the soil occur from the metabolism of the deposited urine and faeces, and indirectly through atmospheric deposition and leaching and runoff.

Figure 4.4: Emissions that result from manure of beef cattle on pasture, rangeland and paddock

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| i | Climate zone |
| j | Time-period (e.g season or month) |
| k | Beef cattle class |
| l | Beef cattle subclass |
| m | Manure management system |

Emissions are estimated based on age and sex classes of cattle, time of the year and manure management system. The climate zone will affect the selection of appropriate emissions factors. The emissions are summed across each class (and subclass), manure management system and time-period spent on farm during the reporting period.

The classes of cattle on the farm (‘Beef cattle input class’) will depend on the diversity of the farming operation. These classes need to be mapped back to default categories (‘Beef Cattle Classes (l) and Subclass (n)’) to allow the use of default values under Method 1.

The time-period selected will depend on availability of stock numbers and liveweight and liveweight gain data (see Herd Flow modelling guidance in Section 1.3.7).

| Beef Cattle Classes (l) | Beef Cattle Subclass (n) (a) | Beef Cattle input classes |
| --- | --- | --- |
| 1 = Bulls < 1 year | 1 = Bulls < 1 year | Bulls < 1 year |
| 2 = Bulls > 1 year | 2 = Bulls > 1 year | Bulls > 1 year |
| 3 = Cows < 1 year | 3 = Cows < 1 year | Cows < 1 year |
| 4 = Cows 1-2 years | 4 = Cows 1-2 years | Cows 1-2 years |
| 5 = Cows > 2 years | 5a = Cows 2-3 years | Cows 2-3 years |
|  | 5b = Cows > 3 years | Cows > 3 years |
| 6 = Steers < 1 year | 6 = Steers < 1 year | Steers < 1 year |
| 7 = Steers > 1 year | 7a = Steers 1-2 years | Steers 1-2 years |
|  | 7b = Steers 2-3 years | Steers 2-3 years |
|  | 7c = Steers >3 years | Steers >3 years |

- Only available for cattle in QLD and NT

The only relevant manure management systems (m) for grazing beef are PRP (m=14) and anaerobic lagoons (m=1). Where farmers have unfenced natural water sources and farm dams, anaerobic lagoons are used as a proxy to estimate the runoff of manure into these water sources .

### Estimation methodology

#### Method 1 — Manure Methane Beef Grazing

(1)	Total annual methane production from manure management ECH4 (t CH4) for grazing beef cattle is calculated as:

ECH4= jklm Njkl×Mmjklm× Dj×10-3

Where 	Njkl = number of beef cattle in each time-period, class and sub class (head)

Mmjklm = methane production from manure in each time-period, class, sub class and MMS (kg/head/day)

Dj= number of days in each time-period for each input class (days). This is 91.25 days under Method 1 as the default time-period is a season

(2)	Methane production from the manure of grazing beef cattle Mmjkl (kg/head/day) is calculated as:

Mmjklm=VSjkl × BO×MMSm ×MCFim×ρ

Where	VSjkl  = volatile solid production (kg/day)

BO = emissions potential (m3 CH4/kg VS)

MMSm = fraction of waste in each manure management system

MCFim = Methane conversion factor for climate zone and MMS. Under Method 1, the default state climate zone is applied

ρ = density of methane (kg/m3)

The primary manure management system (MMS) for pasture-fed beef is pasture range and paddock (m=14). Where natural water sources and farm dams are present the anaerobic lagoon (m=1) MMS is used to estimate the run-off of manure into these water sources. Where farmers can demonstrate animals are on bore or reticulated water systems, where water is provided in troughs (i.e., no unfenced natural water sources or farm dams), then the only relevant MMS to be considered shall be pasture range and paddock (m=14).

The assumption regarding % run-off into farm dams is under review and may change. Until new assumptions are confirmed the allocation to MMS for unfenced water sources is aligned with the current NIR approach.

A new method for estimating VS production will be implemented in the next NIR and has been reflected here.

(3)	Volatile solid production from grazing beef VSjkl  (kg/head/day)is calculated by:

VSjkl=Ijkl × 1-DMDjkl+0.04 ×Ijkl× 1-A

Where	Ijkl  = dry matter intake as calculated in Chapter 3 Section 3.2 (kg/head/day)

DMDijk = dry matter digestibility expressed as a fraction. Default values applied under Method 1

A = ash content of feed intake expressed as fraction

#### Method 2 — Manure Methane Beef Grazing

Method 2 is the same as Method 1 except that under equations 4.2.1.1 (3) feed intake Ijkl  is estimated as per section 3.2.1.2 and farm specific data is applied for DMDjkfor the selected time-period (i.e. seasonal, monthly or other) and for MCFim

Under Method 2 the selected time-period and associated Dj used in equation 4.2.1.1 (1) may be a season (Dj= 91.25 days), month (Dj=28-31 day depending on month) or a specific number of days in a month or season if the entry and exit from farm of a specific cohort of animals is being estimated

#### Method 1 — Soil Direct N2O Beef Grazing

(1)	Total annual direct nitrous oxide emissions from agricultural soils from deposition of urine and dung EN2O,dir (t N2O) is calculated as:

EN2O,dir= AF×EFPRP×Cg,N2O+AU ×EFPRP×Cg,N2O×10-3

Where 	AF = total mass of faecal nitrogen excreted on PRP for beef cattle (kg N)

AU = total mass of urinary nitrogen excreted on PRP for beef cattle (kg N)

EFPRP= emission factor for nitrous oxide from urine and dung deposited to soil (kg N2O-N/kg N deposited)

Cg,N2O = factor to convert elemental mass of nitrous oxide to molecular mass

(2)	Total faecal nitrogen excreted to pasture, range and paddock AF (kg N) is calculated as:

AF= jklNjkl ×Fjkl×Dj

Where 		Njkl = number of beef cattle by time-period, class and sub class (head)

Fjkl = nitrogen excreted in faeces in each time-period, class and subclass (kg N/head/day)

Dj= number of days in each time-period (days). This is 91.25 days under Method 1 as the default time-period is a season

(3)	Nitrogen excreted in faeces Fjkl (kg N/head/day) is calculated based on the approach developed by the SCA (1990) [8] and Freer et al. (1997) [9] as:

Fjkl=0.3× Ijkl × CPj× 1-DMDijk+0.1+(0.105×MEjkl × Ijkl × 0.008) + (0.0152 × Ijkl) ÷6.25 + 0.08×0.032 × MCjk=1,3,6 ÷6.38

Where	Ijkl = dry matter intake (as calculated in Section 3.2 ) (kgDM/head/day)

CPj = crude protein content of dry matter intake (fraction)

DMDjk = dry matter digestibility (fraction)

MEjkl = metabolizable energy (MJ/kgDM)

MCjk=1,3,6= milk intake (kg/head/day)

Under Method 1, default CPj  and DMDj values are applied.

(4)	Metabolizable energy MEjkl (MJ/kgDM) is calculated by Minson and McDonald (1987) [10] as:

MEjkl=0.1604×(DMDj×100)-1.037

Where	DMDj = dry matter digestibility (fraction)

(5)	Total urinary nitrogen excreted to pasture, range and paddock AU is calculated as:

AU=jkl(Njkl × Ujkl ×Dj)

Where 	Ujkl = nitrogen excreted in urine in each time-period, class and sub class (kg N/head/day)

Dj= number of days in each time-period (days). This is 91.25 days under Method 1 as the default time-period is a season

(6)	Nitrogen excreted in urine Ujkl (kg N/head/day) is calculated as:

Ujkl= Ijkl × CPj ÷ 6.25+ 0.032 × MCjk=1,3,6 ÷ 6.38- NRjkl - Fjkl-  1.1 × 10-4 ×Wjkl       0.75÷ 6.25

Where		NRjkl = the amount of nitrogen retained by the body (kg N/head/day)

Wjkl = liveweight (kg/head)

Under Method 1, default Wjkl values are applied.

(7)	The amount of nitrogen retained by the body NRjkl (kg N/head/day) is calculated as:

NRjkl=(0.032 ×MPjk=5÷6.38)+0.212-0.008×(Ljkl - 2) - 0.140-0.008×(Ljkl - 2)1+e(-6×(Zjkl - 0.4)) × (LWGijkln × 0.92) ÷ 6.25

Where 	MPjkl=5 = milk production (kg/head/day)

Ljkl = intake relative to that needed for maintenance

Zjkl = relative size

LWGjkl = liveweight gain  (kg/head/day). Under Method 1, default LWGjkl values are applied.

(8)	Milk production for the season of calving and season after calving MPjkl=5 (kg/head/day) is calculated as:

MPjk=5= LCjk=5 ×DMPjk=5

Where 	LCjk=5 = proportion of cows > 2 years (k=5) lactating for the season of calving and season after calving noting that for all other seasons LCjk=5=0

DMPjk=5 = daily milk production (kg/head/day)

For farms with multiple calving seasons, the proportion of cows > 2 years in calf (LCjk=5) should be reported separately for each season.

(9)	Intake relative to that needed for maintenance Lijkln is calculated as:

Lijkln=Iijkln1.185 + 0.00454 ×Wijkln - 0.0000026×Wijkln22

(10)	The relative size of the cattle Zijkln, is calculated as:

Zjkl=WjklSRWk

Where 		SRWk = standard reference weight (kg)

#### Method 2 — Soil Direct N2O Beef Grazing

Method 2 is the same as Method 1 except that under equations 4.2.1.3 (3), (6) and (9) feed intake Ijkl  is estimated as per section 3.2.1.2 and farm specific data is applied for the selected time-period (i.e. seasonal, monthly or other) for CPjkl (Eq 4.2.1.3 (3) and (6)), DMDijk (Eq 4.2.1.3 (3) and (4)), Wjkl (Eq 4.2.1.3 (6), (9) and (10)), LWGjkl (Eq 4.2.1.3 (7)).

Under Method 2 the selected time-period and associated Lj used in equation 4.2.1.3 (2) may be a season (Lj= 91.25 days), month (Lj=28-31 day depending on month) or a specific number of days in a month or season if the entry and exit from farm of a specific cohort of animals is being estimated

#### Method 1 — Soil Deposition N2O Beef Grazing

(1)	Atmospheric deposition emissions from urine and dung deposited on pasture EN2O,ad (t N2O) are calculated as:

EN2O,ad=Mvol×EFad×Cg,N2O×10-3

Where	Mvol = mass of nitrogen volatilised from urine and faeces deposited on pasture (kg N)

EFad = emission factor for atmospheric deposition (kg N2O-N/kg N)

(2)	The mass of nitrogen volatilised Mvol (kg N) is calculated as:

Mvol=AU+AF×FracGASMsoil

Where	FracGASMsoil = fraction of nitrogen volatilised from urine and faeces deposited on pasture ((kg NH3–N + NOx–N)/kg N)

#### Method 2 — Soil Deposition N2O Beef Grazing

There is no Method 2 quantification option for this emission source.

#### Method 1 — Soil Leaching and Runoff N2O Beef Grazing

(1)	Leaching and runoff emissions from urine and dung deposited on pasture EN2O,leach (t N2O) are calculated as:

EN2O,leach=Mleach×EFleach×Cg,N2O×10-3

Where		Mleach = mass of nitrogen lost to leaching and runoff (kg N)

EFleach = emission factor for leaching and runoff (kg N2O-N/kg N)

(2)	The mass of nitrogen lost to leaching and runoff Mleach is calculated as:

Mleach=AU+AF×FracWet× FracLEACH

Where	FracWet = fraction of N available for leaching and runoff in each climate zone.

FracLEACH = fraction of all N that is lost through leaching and runoff

#### Method 2 — Soil Leaching and Runoff N2O Beef Grazing

There is no Method 2 quantification option for this emission source

### Data/Parameter tables

#### Input data (Required)

| Data / Parameter | Njkl |
| --- | --- |
| Data unit | head |
| Description | Number of pasture beef cattle per time-period, class and sub class |
| Data source | Farm stock records and herd flow model see Chapter 1 Section 1.9 |
| Quality assurance / quality control considerations | All animals purchased, sold or born in the reporting period are assumed to be reported. Number of cattle reported can be cross checked with stocking density allowances for farm size. If value is significantly higher or lower than expected stocking density, flag as possibly erroneous. |

| Data / Parameter | LCjk=5 |
| --- | --- |
| Data unit | fraction |
| Data source | Proportion of cows > 2 years in calf in the season of calving. This is used as a proxy for cows >2 lactating |
| Value | Farm records: Proportion of cows > 2 can be based on scanning numbers where available. If scanning results are not available weaning numbers can also be used to approximate the proportion of cows > 2 years lactating. For farms with multiple calving seasons, the proportion of cows > 2 years in calf LCjk=5 should be reported separately for each season or time period. Noting that the calving season should be considered 3 months from the calving month if Method 2 herd flow data is used (see Chapter 1 Section 1.9) and that for all other time periods Ljk=5 = 0 |
| Quality assurance / quality control considerations | Scanning records can be used to check input data. If scanning or weaning results are not recorded proportion of cows > 2 years can be cross checked from number of cows and calves in the reporting period. |

#### Data (Method 1 and 2 options)

| Data / Parameter | Dj |
| --- | --- |
| Data unit | days |
| Description | Duration of stay for each cattle, class and sub class |
| Method 1 data source | National Inventory Report Volume 1 [1] |
| Method 1 value | 91.25 |
| Method 2 data source | Farm stock records; system type records or purchase and sales can be evaluated to determine average duration of each cattle input class (see Chapter 1 Section 1.9 for more details on herd flow modelling) |
| Quality assurance / quality control considerations | Ensure that if animals are on the farm all year round the duration of stay is 365 days Ensure that if animals are only born part way through the reporting period their duration of stay reflect this. If Method 1 is used inputs must be completed seasonally throughout the calculations. |

| Data / Parameter | DMDj |
| --- | --- |
| Data unit | fraction |
| Description | Dry matter digestibility |
| Method 1 data source | Table A5.5.2.3, National Inventory Report, Volume 2 [4] |
| Method 1 value | See Table 12.1.1.5 in Chapter 12 Appendix Select the appropriate default value for location of cattle and season |
| Method 2 data source | Farm source records for seasonally dry matter digestibility of feed. |
| Quality assurance / quality control considerations | If Method 2 data is used compare this input to inventory defaults for the relevant cattle class. If significantly higher or lower than inventory defaults check farm records to ensure correct data entry. If farm data is only available for one (or more) seasons, use data available for season(s) and supplement missing data with inventory data. |

| Data / Parameter | CPj |
| --- | --- |
| Data unit | fraction |
| Description | Crude protein content of feed dry matter |
| Method 1 data source | Table A5.5.2.4, National Inventory Report, Volume 2 [4] |
| Method 1 value | See Table 12.1.1.5 in Chapter 12 Appendix Select the appropriate default value for location of cattle and season |
| Method 2 data source | Farm source records for seasonally crude protein of feed. |
| Quality assurance / quality control considerations | If Method 2 data is used compare this input to inventory defaults for the relevant cattle class. If significantly higher or lower than inventory defaults check farm records to ensure correct data entry. If farm data is only available for one (or more) seasons, use data available for season(s) and supplement missing data with inventory data. |

| Data / Parameter | Wjkl |
| --- | --- |
| Data unit | kg |
| Description | Average liveweight of beef pasture cattle per time-period, class and subclass |
| Method 1 data source | Table A5.5.2.1 National Inventory Report Volume 2 [4] |
| Method 1 value | See Table 12.1.1.1 & 12.1.1.2 in Chapter 12 Appendix Select the appropriate default value for location of cattle, class/subclass and season |
| Method 2 data source | Farm stock records and herd flow model (see Chapter 1 Section 1.9) |
| Quality assurance / quality control considerations | Purchase and sale weight records (invoices) can be used for data assurance and control of entered values. Dated print out, screen shot, or photo of scale unit records can also be used for quality assurance of entered values. It is recommended Method 1 default values are sense checked against production data and system to ensure they are appropriate e.g. if autumn calving ensure default weights reflect this with lowest weigh value applied in autumn for stock <1 year and/or that default weights reflect when stock are expected to be at the lightest and heaviest. |

| Data / Parameter | LWGjkl |
| --- | --- |
| Data unit | kg/head/day |
| Description | Average liveweight gain of beef pasture cattle per time-period, class and sub class |
| Method 1 data source | Table A5.5.2.2 National Inventory Report Volume 2 [4] |
| Method 1 value | See Table 12.1.1.3 & 12.1.1.4 in Chapter 12 Appendix Select the appropriate default value for location of cattle, class/subclass and season |
| Method 2 data source | Farm stock records and herd flow model (see Chapter 1 Section 1.9) |
| Quality assurance / quality control considerations | Purchase and sale weight records (invoices) can be used for data assurance and control of entered values. Dated print out, screen shot, or photo of scale unit records can also be used for quality assurance of entered values. It is recommended Method 1 default values are sense checked against production data and system to ensure they are appropriate e.g. that default liveweight gain reflects when stock are expected to be growing more due to feed quality and availability. |

| Data / Parameter | EFPRP |
| --- | --- |
| Data unit | kg N2O-N/kg N |
| Description | Emission factor for urine and dung deposited on pasture, range or paddock |
| Method 1 data source | IPCC 2019, Volume 4, Chapter 11, Table 11.1 [11] |
| Method 1 value | 0.004 |
| Method 2 data source | A farm-specific approach to EFPRP can be employed by using the IPCC values for EFPRP specific to the climate zone (wet vs dry) in which the farm is located. See Table 12.2.2.4 in Chapter 12 Appendix for these values. Refer to Chapter 1 Section 1.8 for guidance on how to determine the climate zone the enterprise is located in. Wet and dry climates used in this classification are a subset of the temperate and tropical climate zones – this is not to be confused with rainfall zones. |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

#### Constants

| Data / Parameter | B0 |
| --- | --- |
| Data unit | m3 CH4/kg VS |
| Description | Emissions potential |
| Data source | IPCC (2019), Chapter 10 [5] |
| Value | 0.19 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report. |

| Data / Parameter | MMSm |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of waste in each MMS |
| Data source | National Inventory Report, Volume 1 [1] |
| Value | If the farm has unfenced water sources inventory defaults of MMSm=14 = 0.95 and MMSm=1 = 0.05 If animals are on bore or reticulated water systems where water is provided in troughs (i.e. no unfenced natural water sources) then all waste should be allocated to pasture range and paddock and MMSm=14 = 1 and MMSm=1 = 0. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian NIR. High resolution aerial imagery can be used to cross check that water sources are stock excluded. |

| Data / Parameter | MCFim |
| --- | --- |
| Data unit | fraction |
| Description | Methane conversion factor |
| Data source | M14 = National Inventory Report, Volume 1 [1] M1 = IPCC 2019, Volume 4, Chapter 10, Table 10.17 [12] |
| Value | Methane conversion factor is dependent on climate zone. See Table 12.1.1.9 in Chapter 12 Appendix for these climate zone dependent values. Refer to Chapter 1 Section 1.8 on guidance for determining which climate zone the enterprise is located in. |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

| Data / Parameter | ρ |
| --- | --- |
| Data unit | kg/m3 |
| Description | Density of methane |
| Data source | National Greenhouse and Energy Reporting (Measurement) Determination 2008 [6] |
| Value | 0.6784 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

| Data / Parameter | A |
| --- | --- |
| Data unit | fraction |
| Description | Ash content of feed intake |
| Data source | National Greenhouse Gas Inventory Report Vol 1. (2023) |
| Value | 0.08 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

| Data / Parameter | MCjk=1,3,6 |
| --- | --- |
| Data unit | kg/head/day |
| Description | Milk intake |
| Method 1 data source | Table A5.5.2.5, National Inventory Report, Volume 2 [4] |
| Method 1 value | See Table 12.1.1.7 in Chapter 12 Appendix Select the appropriate default value based on the cattle breed. Noting that the calving season should be considered 3 months from the calving month if Method 2 herd flow data is provided (see Chapter 1 Section 1.9) and the season after calving the 3 months after this period. |
| Quality assurance / quality control considerations | Ensure all other stock classes other than cattle <1 year old are defaulted to zero. Ensure alignment with the latest version of the Australian National Inventory Report |

| Data / Parameter | DMPjk=5 |
| --- | --- |
| Data unit | kg/head/day |
| Description | Daily milk production |
| Method 1 data source | Table A5.5.2.5 |
| Method 1 value | See Table 12.1.1.7 in Chapter 12 Appendix Select the appropriate default value based on the cattle breed. Noting that the calving season should be considered 3 months from the calving month if Method 2 herd flow data is provided (see Chapter 1 Section 1.9) and the season after calving the 3 months after this period. |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report. |

| Data / Parameter | SRWk |
| --- | --- |
| Data unit | kg |
| Description | Standard reference weight |
| Method 1 data source | Table A5.5.2.6, National Inventory Report, Volume 2 [4] |
| Method 1 value | See Table 12.1.1.8 in Chapter 12 Appendix. Select the appropriate default value based on cattle class. |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report. |

Question Reference 4.5.

Current guidance provides the option to select the most appropriate emission factor for atmospheric deposition based on grazing system.

Is this necessary? Should this just be assumed to be constant as non-irrigated pasture? Will producers want the option of a weighted average approach?

| Data / Parameter | EFad |
| --- | --- |
| Data unit | kg N2O-N/kg N |
| Description | Emission factor for atmospheric deposition. |
| Data source | National Inventory Report Volume 1, Table 5.21 [1] |
| Value | See Table 12.2.2.3 in Chapter 12 Appendix This is the same emissions factor as used for direct nitrous oxide emissions from inorganic fertilisers. Apply the EF that best represent the production system In some farm systems animals may graze on crops. Apply the inorganic fertiliser EF that best representing how the cattle are grazed for the majority of the year or calculate a weighted average if the number of days on each grazing system is known. For non-irrigated cropping systems, the emission factor varies depending on whether the agricultural system is in a high rainfall or low rainfall zone. Refer to Chapter 1 Section 1.8 for guidance on determining the relevant rainfall zone. |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

| Data / Parameter | FracGASMsoil |
| --- | --- |
| Data unit | (kg NH3–N + NOx–N)/kg N |
| Description | Fraction of nitrogen volatilised from urine and faeces deposited on pasture |
| Method 1 data source | IPCC 2019, Volume 4, Chapter 11, Table 11.3 [11] |
| Method 1 value | 0.21 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian NIR. |

| Data / Parameter | FracWET |
| --- | --- |
| Data unit | Fraction |
| Description | Fraction of nitrogen that is available for leaching and runoff |
| Data source | National Inventory Report Volume 1 [1] |
| Value | To determine whether the entity is located in an area where leaching occurs refer to Chapter 1 Section 1.8 on how to determine whether the enterprise is located in a leaching zone. Where it is determined that leaching does occur, FracWET = 1 Where it is determined that leaching does not occur, FracWET = 0 |
| Quality assurance / quality control considerations | Farm title data can be checked to ensure spatial farm boundaries used to define climate zone are correct. Other inputs such as electricity or fuel of irrigation purposes can be used to highlight if pastures are irrigated and therefore leaching will be occurring. |

| Data / Parameter | FracLEACH |
| --- | --- |
| Data unit | fraction |
| Description | Default fraction of N that is lost through leaching and runoff |
| Method 1 data source | National Greenhouse Gas Inventory Volume 1 [13, p. 1] |
| Method 1 value | 0.24 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

| Data / Parameter | EFleach |
| --- | --- |
| Data unit | kg N2O-N/kg N |
| Description | Emission factor for leaching and runoff |
| Method 1 data source | National Greenhouse Gas Inventory Volume 1 [13, p. 1] |
| Method 1 value | 0.011 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

## Dairy

This module covers the estimation of methane and nitrous oxide emission that results from dairy cattle manure management. Figure 4.5 summarises the emission sources associated with the manure from dairy cattle. There are two components to methane emission estimates – manure excreted directly onto pasture, and managed manure from the milking shed or and feedpad. Managed manure also results in direct and indirect nitrous oxide emissions. For manure excreted to pasture there are no nitrous oxide emissions from manure management, however emissions from the soil occur from the metabolism of the deposited urine and faeces, and indirectly through atmospheric deposition and leaching and runoff. Note that emissions associated with manure from management systems that is subsequently applied to soils are covered in the organic fertiliser section in Chapter 5 Section 5.1 Fertiliser module.

Figure 4.5: Emissions that result from dairy cattle manure

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| i | Climate zone |
| j | Livestock class |
| m | Manure management system |

Emissions are estimated based on age and sex classes of cattle and the manure management system. The climate zone will affect the selection of appropriate emissions factors. The emissions are summed across each class, manure management system and time-period spent on farm during the reporting period.

The classes of cattle on the farm (‘dairy input class’) will depend on the diversity of the farming operation. The number of heifer and bull calves should be reported separately to weaned stock less than 12 months in age to capture emissions from calves sold at weaning. These classes need to be mapped back to default categories (‘dairy cattle class j’) to allow the use of default inventory values under Method 1.

| Dairy Cattle Class j | Possible Dairy Input Class |
| --- | --- |
| 1 = Milking cows | Milking cows |
| 2 = Heifers > 1 year | Heifers > 1 year |
| 3 = Heifers < 1 year | Replacement heifers < 1 year, weaned |
|  | Heifer Calves, <1 year, preweaning |
| 4 = Bulls > 1 year | Mature bulls |
| 5 = Bulls < 1 year | Other weaned stock < 1 (non-replacement heifers, steers or bulls) |
|  | Bull Calves, <1 year, preweaning |

The National Inventory Report and Method 1 defaults for dairy assume that all “other stock” will be sold from the dairy at 12 months or less. In cases where other stock such as steers or bulls are retained beyond 12 months Method 1 default values for liveweight and liveweight gain can be taken from the appropriate beef pasture, range and paddock defaults (see Section 4.2 for more detail).

Nitrous oxide emissions from managed manure are only relevant for milking cows (j=1). The manure management systems (m) will differ for manure excreted while animals are grazing or in the milking shed or feedpad. Manure from other dairy cattle classes (j=2-5) is all assumed to be excreted to pasture (m=14)

Guidance is currently provided for the manure management systems (MMS) aligned with the NIR options. These include pasture, anaerobic lagoons, drains to paddock, sump and dispersal systems, and associated solid separation (considered solid storage). The MMS options available for dairy cattle are under review

### Estimation methodology

#### Method 1 — Manure Methane Dairy

(1)	Total annual methane production from manure management ECH4 (t CH4) is calculated as the sum of methane from manure from dairy cows, bulls, young stock and pre-weaned calves:

ECH4=jmNj=1,2,4×Mmj=1,2,4m×Dj=1,2,4+Nj=3,5×Mmj=3,5,m×Dj=3,5 +Nj=3,5×MPWmanure,j=3,5 ×Dj=3,5 ×10-3

Where 		Nj = number of dairy cattle in each class

Mjm = methane production from manure of weaned dairy cattle in each class in each MMS (kgCH4/head/day)

MPWmanure,j=3,5= methane production from manure of pre-weaned heifer and bulls calves (<1 year) (kg CH4/head/day)

Dj = Duration of stay on the farm (days). Method 1 default values for these time periods are provided in data tables.

(2)	Methane from manure production Mijm (kgCH4/head/day) from weaned dairy cattle is calculated by:

Mmjm=VSj×Bo×MMSjm×MCFim×ρ

Where: 	VSj= daily volatile solids production per head for dairy cattle (kg/head/day)

Bo = emissions potential (m3 CH4/kgVS)

MMSjm= fraction of waste from each class in each management system

MCFim = methane conversion factor for climate zone and MMS. Under Method 1, the default state climate zone is applied

ρ = density of methane (kg/m3)

Guidance for calculating MMSm has been provided in the data tables below based on default times spent on pasture, in the milking shed and on a feedpad including both Method 1 and 2 options.

For weaned dairy cattle volatile solids VSj (kg/head/day) are calculated as:

VSj=Ij×1-DMDj+0.04×Ij×1-A

Where	Ij = dry matter feed intake as calculated in Chapter 3 Section 3.3) (kgDM/head/day)

DMDj = dry matter digestibility. Default values applied under Method 1

A = ash content of feed intake

#### Method 2 — Manure Methane Dairy

Method 2 is the same as Method 1 except that under equations 4.3.1.1 (3) feed intake Ijkl  is estimated as per section 3.3.1.2 and farm specific data is applied for DMDkfor and for MCFim

Under Method 2 the selected time-period and associated Dj used in equation 4.2.1.1 (1) may be a year ,Dj=1,2,4= 365 days, or Dj=3,5=284 or 84 days depending on pre or post weaning or a specific number of days if the entry and exit from farm of a specific cohort of animals is being estimated

#### Method 1 — Manure Direct N2O Dairy

(1)	The total annual direct nitrous oxide emissions from manure management systems EN2O,dir (t N2O) is calculated as:

EN2O,dir= jmFNjm×EFm×Cg,N2O+jmUNjm ×EFm×Cg,N2O×10-3

Where 	FNjm = total faecal nitrogen in each MMS for dairy cattle classes (kg N)

UNjm  = total urinary nitrogen in each MMS for dairy cattle classes(kg N)

EFm= nitrous oxide emission factor for each MMS (kg N2O-N/kg N deposited)

(2)	Total faecal nitrogen in each MMS FNjm (kg N) is calculated as:

FNjm= AFj× MMSjm

Where 	AFj = total annual faecal nitrojgen excreted by each dairy class (kg N)

MMSjm  = fraction of waste in each MMS for each dairy class (kg N)

(3a)	Total annual faecal nitrogen excreted by dairy cattle >1 year old, AFj=1,2,4 (kg N) is calculated as:

AFj=1,2,4=(Nj=1,2,4 × Fj=1,2,4 ×365)

Where: 	Fj=1,2,4= nitrogen excreted in faeces in dairy cattle >1 year old (kg N/head/day)

(3b)	Total annual faecal nitrogen excreted by dairy cattle <1 year old, AFj=3,5 (kg N) is calculated as:

AFj=3,5=Nj=3,5×Fj=3,5×281+Nj=3,5×FPWj=3,5×84

Where: 	Fj=3,5= nitrogen excreted in faeces in weaned dairy cattle <1 year old (kg N/head/day)

FPWj=3,5= nitrogen excreted in faeces in pre-weaned calves (kg N/head/day)

(4)	Nitrogen excreted in faeces of dairy cattle >1 year old and weaned cattle <1 year old Fj is calculated using equations developed by the SCA (1990) [8] and Freer et al. (1997) [9] as:

Fj=0.3×CPIj×1-DMDj+0.1+0.105×MEj×Ij×0.008+0.0152×Ij×16.25

Where 		CPIj = crude protein intake (kg/head/day)

MEj = metabolizable energy (MJ/kgDM)

DMDj = dry matter digestibility (fraction). Default values applied under Method 1

(5)	Metabolizable energy MEj is calculated by Minson and McDonald (1987) [10] as:

MEj=0.1604×(DMDj×100)-1.037

Where	DMDj = dry matter digestibility (fraction). Default values applied under Method 1

(6)	The crude protein intake CPIj (kg/head/day) of dairy cattle is calculated as:

CPIj=Ij×CPj

Where	CPj = crude protein content of feed intake (fraction). Default values applied under Method 1

(8)	Total urinary nitrogen in each MMS UNjm (kg N) is calculated as:

UNjm=AUj× MMSjm

Where 	AUj = total annual urinary nitrogen excreted by each dairy class (kg N)

MMSjm  = fraction of waste in each MMS for each dairy class (kg N)

(9a)	Total annual urinary nitrogen excreted by dairy cattle >1 year old, AUj=1,2,4 (kg N) is calculated as:

AUj=1,2,4=(Nj=1,2,4 × Uj=1,2,4 ×365)

Where: 	Uj=1,2,4= nitrogen excreted in urine in dairy cattle >1 year old (kg N/head/day)

(9b)	Total annual urinary nitrogen excreted by dairy cattle <1 year old, AUj=3,5 (kg N) is calculated as:

AUj=3,5=Nj=3,5×Uj=3,5×281+Nj=3,5×UPWj=3,5×84

Where: 	Uj=3,5= nitrogen excreted in urine in weaned dairy cattle <1 year old (kg N/head/day)

UPWj=3,5= nitrogen excreted in urine in pre-weaned calves (kg N/head/day)

(10)	Nitrogen excreted in urine for of dairy cattle >1 year old and weaned cattle <1 year old Uj (kg N/head/day) is calculated as:

Uj=CPIj6.25-NRj-Fj-1.1×10-4×Wj0.756.25

Where		NRj = amount of nitrogen retained by the body (kg N/head/day)

Wj = liveweight (kg). Default values applied under Method 1

(11)	The amount of nitrogen retained by the body NRj is calculated as:

NRj=0.032 ×MPj×1.036.38+0.212-0.008×(Lj - 2) - 0.140-0.008×(Lj - 2)1+e-6×(Zj - 0.4) × (LWGj × 0.92)×16.25

Where		MPj = milk production (L/head/day)

Lj = intake relative to that needed for maintenance

Zj = relative size

LWGj = liveweight gain (kg/day). Default values applied under Method 1

1.03 = conversion factor for milk in litres to kg

Where daily milk production data is collected in quantities of milk solids rather than litres of milk, refer to Chapter 3 Section 3.3 for converting milk solids to litres.

(12)	Intake relative to maintenance Lj is calculated as:

Lj=Ij1.185+0.00454×Wj-0.0000026×Wj22×MRj+MIj

Where	MRj = increase in metabolic rate when producing milk (kgDM/head/day)

MIj = additional intake required for milk production, as calculated in Chapter 3 Section 3.3 (kgDM/head/day)

(13)	The relative size Zj is calculated as;

Zj=Wj SRWj

Where 		SRWj = standard reference weight (kg)

#### Method 2 — Manure Direct N2O Dairy

Method 2 is the same as Method 1 except that under equations 4.3.1.3 (4), (6) and (12) feed intake Ijkl  is estimated as per section 3.2.1.2 and farm specific data is applied for the selected time-period (i.e. seasonal, monthly or other) for CPjkl (Eq 4.2.1.3 (6)), DMDijk (Eq 4.3.1.3 (4) and (5)), Wjkl (Eq 4.3.1.3 (10), (12) and (13)), LWGjkl (Eq 4.3.1.3 (11)).

#### Method 1 — Manure Deposition N2O Dairy

(1)	Annual atmospheric deposition emissions from MMS EN2O,MMS,ad is calculated as:

EN2O,MMS,ad=MNATMOS × EFN2O×Cg ×GWPN2O×10-3

Where		MNATMOS = mass of N volatilised from MMS (kg N)

EFN2O = inorganic fertiliser emission factor for irrigated pasture (kg N2O-N/kg N)

(2)	Total of nitrogen volatilised from MMS MNATMOS (kg N) is calculated as:

MNATMOS= jm(FNjm×FracGASMm+ UNjm ×FracGASMm)

Where		FracGASMm = fraction of N volatised from MMS (fraction)

#### Method 1 — Manure Leaching and Runoff N2O Dairy

(1)	Annual emissions from leaching and run off from each MMS EN2O,leach (t N2O) is calculated as:

EN2O,leach=jm=1-13(MNLeachjm × EFleach×Cg )×10-3

Where 	MNLEachjm = mass of N lost through leaching and runoff from each MMS (kg N)

MNLeachmis assumed to be zero for anaerobic lagoons (m=1), and for daily spread treatments (sump and dispersal m=3a, and drains to paddock m=3b) is nitrogen is transferred directly to manure applied to soils (see section 4.3.1.8 Manure applied to Soil)

Emissions from leaching and run off are only estimated for the solid storage MMS (m=4). This simplifies the equation to:

EN2O,leach=MNLeachjm=4 × EFleach×Cg ×10-3

This assumption may need to be reconsidered if other MMS are incorporated based on stakeholder feedback.

(2)	The mass of nitrogen lost to leaching and runoff from solid storage systems MNLeachjm=4 (kg N) is calculated as:

MNLeachjm=4=FNjm=4+UNjm=4×FracWETi×FracLEACHMS

Where	FracWETi = fraction of N available for leaching and runoff in climate zone. Under Method 1, the default state climate zone is applied

FracLEACHMS = fraction of N lost through leaching and runoff

#### Method 2 — Manure Leaching and Runoff N2O Dairy

Method 2 is the same as Method 1 except that under equations 4.6.1.6 (2) farm specific climate zone is required for FracWETi.

#### Manure Applied to Soils

It is assumed that all manure managed through an MMS (m=1-13) will eventually be applied to soils. The mass of nitrogen applied to soils is calculated here and then this MNSoil value is transferred to Chapter 5 Section 5.1 to estimate emissions associated with the application of manure to soils as organic fertilisers, including associated atmospheric deposition and leaching and run-off emissions.

The manure may be applied to soils within the boundary of the farm or may be sent off-site and applied to another farming enterprise. Where manure is applied to soils within the boundary of the farming enterprise MNSoilscope1, the emissions from shall be estimated and reported as Scope 1 emissions. Where the manure is used on other farming enterprises MNSoilscope3, the emissions from shall be estimated and reported as Scope 3 emissions under Scope 3 Category 5 – waste generated in operations (see Chapter 7 Section 7.10).

(1)	The mass of nitrogen from manure that is applied to soils for scope 1 emissions MNSoilscope1 (kg N) is calculated as:

MNSoilscope1=jmUNjm=1-13+FNjm=1-13 ×1-EFm=1-13-FracGASMm-1-13- MNLeachjM)× PF

Where 	PFscope = fraction of manure applied to soil within the farm boundary

(2)	The mass of nitrogen from manure that is applied to soils for scope 3 MNSoilscope3 (kg N) is calculated as:

MNSoilscope3=jmUNjm=1-13+FNjm=1-13 ×1-EFm=1-13-FracGASMm-1-13- MNLeachjM)× (1-PF)

This approach assumes that the manure used off-site within a different farm enterprise is considered a “waste product” from the farm, rather than a co-product.

If the manure is sold for use as an organic fertiliser elsewhere, under the GHG Protocol the manure would be considered a co-product and thus would require some of the total farm emissions allocated to it. Furthermore, the emissions from the application of manure to soils would then fall into a use-of-sold-products emissions category, which is outside of the cradle-to-farm gate boundary.

The wording used in this section is therefore still subject to review and non-specific   until the Common Reporting Framework provides guidance on emission allocation between co-products, and whether manure sold off-site is considered a co-product or waste stream.

#### Method 1 — Soil Direct N2O Dairy

(1)	Direct nitrous oxide emissions EN2O,dir (t N2O) on to pasture are calculated as:

EN2O, dir=jFNjm=14×EFPRP×Cg,N2O+AUjm=14 ×EFPRP×Cg,N2O ×10-3

Where 	FNjm=14 = mass of faecal nitrogen excreted on pasture, range and paddock for each dairy cattle class (kg N)

EFPRP= emission factor for nitrous oxide emissions for urine and dung deposited on PRP (kg N2O-N/kg N deposited)

Cg,N2O = factor to convert elemental mass of N2O to molecular mass

UNjm=14  = mass of urine nitrogen excreted pasture, range and paddock for each dairy cattle class (kg N)

#### Method 1 — Soil Deposition N2O Dairy

(1)	Atmospheric deposition emissions from urine and dung deposited on pasture EN2O,ad (t N2O) are calculated as:

EN2O,ad=Mvol,m=14×EFad×Cg,N2O× 10-3

Where	Mvol,m=14 = mass of nitrogen volatilised from urine and faeces deposited on pasture (kg N)

EFad = emission factor for atmospheric deposition (kg N2O-N/kg N)

(2)	The mass of nitrogen volatilised Mvol,PRP (kg N) is calculated as:

Mvol,m=14= jUNjm=14+FNjm=14×FracGASMsoil

Where	FracGASMsoil = fraction of nitrogen volatilised from urine and faeces deposited on pasture ((kg NH3-N + NOx-N)/kg N)

#### Method 2 — Soil Deposition N2O Dairy

There is no Method 2 quantification option for this emission source

#### Method 1 — Soil Leaching and Runoff N2O Dairy

(1)	Leaching and runoff emissions from urine and dung deposited on pasture ESN2O,leach (t N2O) are calculated as:

EN2O,leach=Mleach.m=14×EFleach×Cg,N2O×10-3

Where	Mleach,m=14 = mass of nitrogen lost to leaching and runoff from urine and faeces deposited on pasture (kg N)

EFleach = emission factor for leaching and runoff (kg N2O-N/kg N)

(2)	The mass of nitrogen lost to leaching and runoff Mleach,PRP is calculated as:

Mleach,m=14=jUNjm=14+FNjm=14×FracWet×FracLEACH

Where	FracWet = fraction of N available for leaching and runoff in the climate zone of the entity. Under Method 1, it is assumed the entity is in a leaching climate zone.

FracLEACH = fraction of N that is lost through leaching and runoff

Where 	FracGASMm=1-13 = fraction of N volatilised in each MMS ((kg NH3–N + NOx–N)/kg N)

#### Method 2 — Soil Leaching and Runoff N2O Dairy

Method 2 is the same as Method 1 except that under equations 4.3.1.11 (2) farm specific climate zone is can be calculated FracWet.

### Data/Parameters

#### Input Data (Required)

| Data / Parameter | Nj |
| --- | --- |
| Data unit | head |
| Description | Number of dairy cattle in each class j |
| Data source | Farm stock records and herd flow model see Chapter 1 Section 1.9 Noting that the number of heifer and bull calves should be reported separately to weaned stock less than 12 months in age to capture emissions from calves sold at weaning. |
| Quality assurance / quality control considerations | All animals purchased or sold in the reporting period are assumed to be reported. The number of heifer and bull calves should be reported separately to weaned stock less than 12 months in age to capture emissions from calves sold at weaning. Number of cattle reported can be cross checked with stocking density allowances for system size. If value is significantly higher or lower than expected stocking density, flagged as possibly erroneous. Check the class of animals on farm align with expected enterprise on farm: for example, self-replacing systems vs purchased breeder or trading systems. |

| Data / Parameter | MPj |
| --- | --- |
| Data unit | L/head/day |
| Description | Daily milk production per milking cow |
| Data source | Farm milk sales records and number of milking cows. The average amount of milk sold per day (in litres) can be divided by the average number of milking cows on the farm to calculated the daily milk production per milking cow. If milk production records are in terms of milk solids rather than litres of milk, refer to the section 3.3 for converting milk solids (MSj) into litres (MPj). |
| Quality assurance / quality control considerations | Ensure farm source data isn’t significantly different when compared to NIR state-based values indicating potential data entry error. State-based values can be found in Table 12.1.3.9 in Chapter 12 Appendix If milk production can be verified through milking parlour software and corroborated via milk collection receipts. |

| Data / Parameter | PF |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of manure from MMS applied to soil within the farm boundary |
| Data source | Farm source data When all manure treated in MMS is applied within the farm boundary and none is sold off-site to be used in a different enterprise, then PF = 1. |
| Quality assurance / quality control considerations | Ensure farm source data isn’t significantly different when compared to NIR state-based values indicating potential data entry error. |

#### Data (Method 1 and 2 Options)

| Data / Parameter | Dj |
| --- | --- |
| Data unit | days |
| Description | Duration of stay for each dairy cattle input class |
| Method 1 data source | National Inventory Report Volume 1 [1] |
| Method 1 value | For all manure stock (j = 1,2,4) Dj = 365 For pre-weaned young stock (j=3,5) Dj = 84 For weaned stock <1 year (j = 3,5) Dj = 281 |
| Method 2 data source | Farm stock records; system type records or purchase and sales can be evaluated to determine average duration of each cattle input class (see Chapter 1 Section 1.9 for more details on herd flow modelling). |
| Quality assurance / quality control considerations | Ensure that if animals are on the farm all year round the duration of stay is 365 days Ensure that if animals are only born part way through the reporting period their duration of stay reflect this. |

| Data / Parameter | MMSjm |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of volatile solids voided to each manure management system m |
| Method 1 data source | Annual fraction of time spent on pasture, in the milking shed or on a feedpad is based on the assumptions underlying the defaults for fm used in the National Greenhouse Gas Inventory MMS calculations. User-defined data for fraction of waste from the milking shed and feedpad that goes to each MMS may be based on farm records of MMS quantities (where available), or estimates based on experience and observations made throughout the year. |
| Method 1 value | MMSjm is determined based on the fraction of time dairy cows or heifers spend on pasture, in the milking shed, or on feedpad. The user must select which the feeding system most representative of the dairy operation, to then determine the fraction of time the cows spend in each area. See Table 12.1.3.10 in Chapter 12 Appendix. The fraction of volatile solids voided to pasture (fm=14) is equal to the annual fraction of time spent on pasture in the provided table. For example, for a grazing-only system, fm=14 = 0.89 The annual fraction of time spent in the milking shed and on the feedpad are used to allocate manure to different MMS. The user must indicate what percentage of waste is allocated to each MMS that is used to manage waste from the feedpad and from the milking shed respectively. The following MMS options are available: - Anerobic lagoon (m = 1) - Sump and dispersal system (m = 3a) - Drains to paddocks (m = 3b) - Solid storage (m = 4) fm is calculated as the percentage of waste allocated to the MMS from that area multiplied by the time spent in that area. For example, for a grazing-only system where 50% of the waste from the milking shed is drained to paddocks and the remaining 50% is spread a through sump and dispersal system: fm=3a = 0.11*0.50 = 0.055 fm=3b = 0.11*0.50 =0.055 The user must also indicate if the waste is pre-treated (solid separation) before entering in to sump system of lagoon. The user must specify the percentage of solid waste removed by the pre-treatment process and transferred to solid storage. For example, if the previous example included pre-treatment before sump and dispersal and the use specified that 20% of the waste is removed my pre-treatment, then: fm=3a = 0.055 *0.8 = 0.044 fm=4 = 0.055*0.2 = 0.011 |
| Method 2 data source | If the time spent between pasture, milking shed and feedpad is different to the systems 1-4 offered in the Method 1 option, then the annual fraction of time spent in each area can be estimated from farm records for time split between paddock and housed for each livestock class. Alternatively,the ADCC tool provides a questionnaire-based approach to these calculations, which can be utilised to calculate fm [14]. |
| Quality assurance / quality control considerations | Ensure MMS and time spent in each system align with dairy system recorded on farm, i.e., housed or pasture-based system. If Method 1 is used ensure the most recent estimate for annual fraction of time spent per area is used. |

| Data / Parameter | DMDj |
| --- | --- |
| Data unit | fraction |
| Description | Dry matter digestibility |
| Data source | Table A5.5.1.4, National Inventory Report, Volume 2 [4]. |
| Method 1 value | 0.75 |
| Method 2 data source | If average DMD for the reporting period of different livestock classes is known based on farm records of feed sources and quality, a farm specific DMD value can be used. The ADCC, 2025 [14] provides support on how to calculate DMD from feed sources. For any stock classes where farm records of feed sources and quality are not available, the NIR default shall be applied. |
| Quality assurance / quality control considerations | Compare to inventory defaults, if values are significantly higher or lower data entry error is possible. Where Method 2 data is used conduct checks of farm source data are within expected ranges. The ADCC, 2025 [14] provides ranges of DMD for different forage of non-forage supplements. |

| Data / Parameter | CPj |
| --- | --- |
| Data unit | fraction |
| Description | Crude protein content of feed intake |
| Method 1 data source | Table 5.5.1.4, National Inventory Report, Volume 2 [4] |
| Method 1 value | 0.20 |
| Method 2 data source | Farm records or feed receipt stating crude protein of TMR (total mixed ration) for different livestock classes. The ADCC, 2025 [14] provides support on how to calculate CP from feed sources. For any stock classes where farm records of feed sources and quality are not available, the NIR default shall be applied. |
| Quality assurance / quality control considerations | Compare to inventory defaults, if values are significantly higher or lower data entry error is possible. Where Method 2 data is used conduct checks of farm source data are within expected ranges. The ADCC, 2025 [14] provides ranges of CP for different forage of non-forage supplements |

| Data / Parameter | Wj |
| --- | --- |
| Data unit | kg |
| Description | Liveweight |
| Method 1 data source | Table A5.5.1.1 National Inventory Report, Volume 2 [4] and the ADCC 2025 [14] |
| Method 1 value | See Table 12.1.3.1 and Table 12.1.3.2 in Chapter 12 Appendix Select the appropriate live weight for the input class. |
| Method 2 data source | Farm stock records and herd flow model (see Chapter 1 Section 1.9) |
| Quality assurance / quality control considerations | Ensure farm source data isn’t significantly different when compared to NIR default values indicating potential data entry error. Checking purchase and sale weight records against entered values can be used for data assurance and control. |

| Data / Parameter | LWGj |
| --- | --- |
| Data unit | kg/head/day |
| Description | Liveweight gain |
| Method 1 data source | Table 5.5.1.2 National Inventory Report, Volume 2 [4] |
| Method 1 value | See Table 12.1.3.3 in Chapter 12 Appendix Select the appropriate live weight gain for the input class. |
| Method 2 data source | Farm stock records and herd flow model (see Chapter 1 Section 1.9) |
| Quality assurance / quality control considerations | Ensure farm source data isn’t significantly different when compared to NIR default values indicating potential data entry error. Additional checks of farm source data are within expected ranges. Typical liveweight gain for heifers of various milking breeds are (ADCC, 2025): - smaller breeds, such as Jerseys, approximately 0.45 to 0.5 kg/head/day; - medium breeds, such as Friesians, approximately 0.60 to 0.65 kg/head/day - larger breed, such as Holstein Friesians, approximately 0.7 to 0.75 kg/head/day. Checking purchase and sale weight records against entered values can be used for data assurance and control. |

| Data / Parameter | FracWET |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of nitrogen available for leaching and run-off |
| Method 1 data source | National Inventory Report Volume 2, Table A5.5.10.2 [4] |
| Method 1 value | 1 |
| Method 2 data source | A Method 2 approach can be used to determine whether the farm enterprise is located in an area where leaching occurs (refer to guidance in see Chapter 1 Section 1.8 to determine whether the enterprise is located in a leaching zone). Where it is determined that leaching does occur, FracWET,soil = 1 Where it is determined that leaching does not occur, FracWET,soil = 0 |
| Quality assurance / quality control considerations | Complete an inventory guided calculation and compare. If 10-15% higher or lower than inventory defaults flag as possibly erroneous. |

| Data / Parameter | EFPRP |
| --- | --- |
| Data unit | kg N2O-N/kg N |
| Description | Emission factor for nitrous oxide emissions per amount of urine and dung deposited |
| Method 1 data source | IPCC 2019, Volume 4, Chapter 11, Table 11.1 [11] |
| Method 1 value | 0.004 |
| Method 2 data source | A farm-specific approach to EFPRP can be employed by using the IPCC values for EFPRP specific to the climate zone (wet vs dry). Please see table 12.2.2.4 in Chapter 12 Appendix Refer to Chapter 1 Section 1.8 for guidance on how to determine the climate zone the enterprise is located in. Wet and dry climates used in this classification are a subset of the temperate and tropical climate zones – this is not to be confused with rainfall zones. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian NIR. |

#### Constants

| Data / Parameter | GWPCH4 |
| --- | --- |
| Data unit | fraction |
| Description | Global warming potential of methane to convert tCH4 to tCO2e |
| Data source | IPCC (2014), AR5 [15] |
| Value | 28 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report. |

| Data / Parameter | B0 |
| --- | --- |
| Data unit | m3 CH4/kg VS |
| Description | Emissions potential. |
| Data source | IPCC (2019), Chapter 10 [5] |
| Value | 0.24 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report. |

| Data / Parameter | ρ |
| --- | --- |
| Data unit | kg/m3 |
| Description | Density of methane |
| Data source | National Greenhouse and Energy Reporting (Measurement) Determination 2008 [6] |
| Value | 0.6784 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report. |

| Data / Parameter | A |
| --- | --- |
| Data unit | fraction |
| Description | Ash content of manure |
| Data source | National Inventory Report, Volume 1 [1] |
| Value | 0.08 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report. |

| Data / Parameter | MCFim |
| --- | --- |
| Data unit | Methane conversion factor by MMS |
| Description | fraction |
| Data source | National Inventory Report, Volume 2 [4] |
| Value | See Table 12.1.3.6 in Chapter 12 Appendix Select appropriate value based on MMS and state. |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

| Data / Parameter | VSj=3,5 |
| --- | --- |
| Data unit | kg/head/day |
| Description | Volatile solids production for pre-weaned calves |
| Data source | Table A5.5.1.5, National Inventory Report, Volume 2 [4] |
| Value | See Table 12.1.3.5 in Chapter 12 Appendix Default values for are only provided for pre-weaned calves. For all other classes volatile solids must be calculated as described in Section 4.3.1. |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

| Data / Parameter | Cg,N2O |
| --- | --- |
| Data unit | fraction |
| Description | Factor to convert elemental mass of nitrous oxide to molecular mass |
| Data source | IPCC Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories. Chapter 5: Agriculture [7] |
| Value | 44/28 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

| Data / Parameter | FPWj=7,8 |
| --- | --- |
| Data unit | kg/head/day |
| Description | Nitrogen excreted in faeces in pre-weaned calves |
| Data source | Table A5.5.1.5, National Inventory Report, Volume 2 [4] |
| Value | See Table 12.1.3.5 in Chapter 12 Appendix Default values for are only provided for pre-weaned calves. For all other classes nitrogen excreted in faeces must be calculated as described in Section 4.3.1. |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

| Data / Parameter | UPWj=3,5 |
| --- | --- |
| Data unit | kg/head/day |
| Description | Nitrogen excreted in urine in pre-weaned calves. It is assumed that pre-weaned dairy calves are removed from their mothers and receive milk or milk replacer and feed pellets. |
| Data source | National Inventory Report, Volume 2, A5.5.1.5 [4] |
| Value | See Table 12.1.3.5 in Chapter 12 Appendix Default values for are only provided for pre-weaned calves. For all other classes nitrogen excreted in urine must be calculated as described in Section 4.3.1. |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

| Data / Parameter | MRj |
| --- | --- |
| Data unit | kg DM/head/day |
| Description | Increase in metabolic rate when producing milk |
| Data source | National Inventory Report, Volume 1 [1] |
| Value | See Table 12.1.3.11 in Chapter 12 Appendix Select appropriate value based on livestock class |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian NIR. |

| Data / Parameter | NE |
| --- | --- |
| Data unit | MJ net energy/kg milk |
| Description | Net energy required for milk production |
| Data source | National Inventory Report, Volume 1 [1] |
| Value | 3.054 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian NIR. |

| Data / Parameter | GEC |
| --- | --- |
| Data unit | MJ/kg DM |
| Description | Gross energy content of feed dry matter |
| Data source | National Inventory Report, Volume 1 [1] |
| Value | 18.4 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian NIR. |

| Data / Parameter | k |
| --- | --- |
| Data unit | fraction |
| Description | Efficiency of use of metabolisable energy for milk production |
| Data source | National Inventory Report, Volume 1 [1] |
| Value | 0.60 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian NIR. |

| Data / Parameter | SRWj |
| --- | --- |
| Data unit | kg |
| Description | Standard reference weight |
| Data source | National Inventory Report Volume 2, Table A5.5.1.3 [4] |
| Value | See Table 12.1.3.4 in Chapter 12 Appendix |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian NIR. |

| Data / Parameter | EFad |
| --- | --- |
| Data unit | kg N2O-N/kg N |
| Description | Emission factor for atmospheric deposition |
| Data source | National Inventory Report, Volume 1, Table 5.21 [1] |
| Value | 0.0059 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian NIR. |

| Data / Parameter | FracGASMsoil |
| --- | --- |
| Data unit | (kg NH3-N + NOx-N)/kg N |
| Description | Fraction of nitrogen volatilized from urine and faeces deposited on pasture. |
| Data source | National Inventory Report, Volume 1 [1] |
| Value | 0.21 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian NIR. |

| Data / Parameter | EFleach |
| --- | --- |
| Data unit | kg N2O-N/kg N |
| Description | Emission factor for leaching and runoff |
| Data source | IPCC 2019, Volume 4, Chapter 11, Table 11.3 [10] |
| Value | 0.011 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian NIR. |

| Data / Parameter | FracLEACH |
| --- | --- |
| Data unit | fraction |
| Description | Default fraction of N that is lost through leaching and runoff |
| Data source | IPCC 2019, Volume 4, Chapter 11, Table 11.3 [10] |
| Value | 0.24 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian NIR. |

| Data / Parameter | EFm |
| --- | --- |
| Data unit | kg N2O-N/kg N |
| Description | Emission factor for nitrous oxide emissions per MMS |
| Data source | Table A5.5.1.9, National Inventory Report, Volume 2 [4] |
| Value | See Table 12.1.3.11 in Chapter 12 Appendix Select the appropriate emission factor based on the MMS in place |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian NIR. |

| Data / Parameter | EFN2O |
| --- | --- |
| Data unit | kg N2O-N/kg N |
| Description | Inorganic fertilizer emission factor for irrigated pasture |
| Data source | NIR |
| Value | See Table 12.2.2.5 in Chapter 12 Appendix Refer to Chapter 1 Section 1.8 for guidance on how to determine the rainfall zone the enterprise is located in |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian NIR. |

| Data / Parameter | FracGASMm |
| --- | --- |
| Data unit | (kg NH3–N + NOx–N)/kg N |
| Description | Fraction of nitrous oxide volatilised from each MMS |
| Method 1 data source | Table A5.5.1.9, National Inventory Report, Volume 2 [4] |
| Method 1 value | See Table 12.1.3.7 in Chapter 12 Appendix Select the appropriate value based on the MMS in place |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used. |

| Data / Parameter | FracLEACH_MS |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of nitrogen lost through leaching and runoff for solid storage MMS only |
| Method 1 data source | National Inventory Report Vol 1 [13] |
| Method 1 value | 0.02 |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian NIR. |

## Sheep

This module covers the estimation of methane and nitrous oxide that result from sheep manure management. Figure 4.6 summarises the emission sources from the manure of sheep. There are two components to methane emission estimates –manure excreted directly onto pasture, rangeland and paddock (PRP) , and manure that runs-off into natural water sources and farm dams. There are no nitrous oxide emissions from manure management for manure excreted to PRP, however emissions from the soil occur from the metabolism of the deposited urine and faeces, and indirectly through atmospheric deposition and leaching and runoff.

Figure 4.6: Emissions that result from sheep manure

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| i | Climate zone |
| j | time-period |
| k | Sheep Class |
| m | Manure management system (MMS) |

Emissions are estimated based on age and sex classes of sheep, time of the year and manure management system. The climate zone will affect the selection of appropriate emissions factors. The emissions are summed across each class, manure management system and time-period spent on farm during the reporting period.

The classes of sheep on the farm (‘sheep input class’) will depend on the diversity of the farming operation. These classes need to be mapped back to default categories (‘sheep class k’) to allow the use of default inventory values under Method 1

The time-period selected will depend on availability of stock numbers and liveweight and liveweight gain data (see Herd Flow modelling guidance in Section 1.3.7).

| Sheep Class k | Possible Sheep Input Class |
| --- | --- |
| 1 = Rams | Rams |
| 2 = Wethers | Wethers |
| 3 = Maiden ewes | Maiden ewes (1-2 years) intended for breeding |
| 4 = Breeding ewes | Breeding ewes |
| 5 = Other ewes | Other ewes |
| 6 = Lambs and hoggets | Ewe lambs (<1 year) |
|  | Wether lambs (<1 year) |
|  | Ram lambs (<1 year) |

The only relevant manure management systems (m) for grazing beef are PRP (m=14) and anaerobic lagoons (m=1). Where farmers have unfenced natural water sources and farm dams, anaerobic lagoons are used as a proxy to estimate the runoff of manure into these water sources.

### Estimation methodology

#### Method 1 — Manure Methane Sheep

- Total annual methane production from manure management for sheep ECH4 (kgCH4) is calculated as:

ECH4= jkmNjk ×Mmjkm×Dj

Where 		Njk = number of sheep in each time-period and input class (head)

Mmjkm = daily methane produced from the manure in each time-period, sheep class and MMS (kgCH4/head/day)

Dj= number of days in each time-period (days). This is 91.25 days under Method 1 as the default time-period is a season

(2)	Methane produced from the manure of sheep Mmjk (kgCH4/head/day) is calculated as:

Mmjkm=VSjk × BO×MMSm ×MCFim×ρ

Where	VSijk  = volatile solid production  in each time-period and class(kg/head/day)

BO = emissions potential (m3 CH4/kgVS)

MMSm = fraction of waste in each manure management system

MCFim = methane conversion factor for climate zone and MMS. Under Method 1, the default state climate zone is applied

ρ = density of methane (kg/m3)

The primary manure management system (MMS) for sheep is pasture range and paddock (m=14). Where natural water sources, such as farm dams, are present the anaerobic lagoon (m=1) MMS is used to estimate the run-off of manure into these water sources. Where farmers can demonstrate animals are on bore or reticulated water systems, where water is provided in troughs (i.e., no unfenced natural water sources), then the only relevant MMS to be considered shall be pasture range and paddock (m=14).

The assumption regarding % run-off in to farm dams is under review and may change. Until new assumptions are confirmed the allocation to MMS for unfenced water sources is aligned with the current NIR approach.

A new method for estimating VS production will be implement in the next NIR and has been reflected here.

Volatile solid production from sheep VSijk (kg/head/day) is calculated by:

VSjk=Ijk × 1-DMDjk+0.04 ×Ijk× 1-A

Where	Iijk  = dry matter intake as calculated in Chapter 3 Section 3.4 (kgDM/head/day)

DMDijk = dry matter digestibility expressed as a fraction. Default values applied under Method 1

A = ash content of feed intake

#### Method 2 — Manure Methane Sheep

Method 2 is the same as Method 1 except that under equations 4.2.1.1 (3) feed intake Ijkl  is estimated as per section 3.4.1.2 and farm specific data is applied for DMDjkfor the selected time-period (i.e. seasonal, monthly or other) and for MCFim

Under Method 2 the selected time-period and associated Lj used in equation 4.2.1.1 (1) may be a season (Lj= 91.25 days), month (Lj=28-31 day depending on month) or a specific number of days in a month or season if the entry and exit from farm of a specific cohort of animals is being estimated

#### Method 1 — Soil Direct N2O Sheep

(1)	Total annual direct nitrous oxide emissions from agricultural soils from deposition of urine and dung EN2O,dir (t N2O) are calculated as:

EN2O,dir=AF×EFPRP×Cg,N2O+AU×EFPRP×Cg,N2O ×10-3

Where	 AF = total mass of faecal nitrogen excreted on PRP by sheep (kg N)

AU = total mass of urinary nitrogen excreted on PRP by sheep (kg N)

EFPRP= emission factor for nitrous oxide from urine and dung deposited to soil (kg N2O-N/kg N deposited)

Cg,N2O= factor to convert elemental mass of nitrous oxide to molecular mass

(2)	Total faecal nitrogen excreted to pasture, range and paddock AF (kg N)  is calculated as:

AF= jk (Njk×Fjk×Lj)

Where		Njk = number of sheep in each time-period and class k (head)

Fjk = nitrogen excreted in faeces per sheep per day (kg N/head/day)

Lj= number of days in each time-period (days). This is 91.25 days under Method 1 as the default time-period is a season

(3)	Nitrogen excreted in faeces Fijk (kg N/head/day) is calculated using equations developed by the SCA (1990) [8] and Freer et al. (1997) [9] as:

Fijk=0.3× CPIj  ×1-DMDj +0.1+ 0.105×MEjk × Ijk × 0.008+ 0.08×0.045 × MCjk=6+ 0.0152 × Ijk÷6.25

Where		CPIj = crude protein intake (kg/head/day)

DMDjk = dry matter digestibility (per cent). Under Method 1, default values are applied

Ijk  = dry matter intake (kgDM/head/day) as calculated in Section 3.4

MEjk= metabolizable energy (MJ/kgDM)

MCjk=6 = milk intake (kg/head/day)

(4)	Crude protein intake CPIijk (kg/head/day) of sheep is calculated as:

CPIjk=Ijk ×CPjk+(0.045 × MCjk=6)

Where	CPjk = crude protein content of feed intake expressed as a fraction. Under Method 1, default values are applied

(5)	Milk intake for sheep MCjk=6 (kg/head/day) is calculated as:

MCjk=6=1.6 ×PLjk=6

Where		1.6 = milk production for time-period lambs are born (kg/head/day).

PLjk=6 = proportion of lambs born in the time-period

(6)	Metabolizable energy  MEjk (MJ/kgDM) can be calculated based on Minson and McDonald (1987) [10] as:

MEjk=0.1604×(DMDjk×100)-1.037

(7)	Total urinary nitrogen excreted to pasture, range, and paddock AUi (kg N)  is calculated as:

AU= jk(Nk × Ujk ×Lj)

Where

Ujk = nitrogen excreted in urine in each time-periods and class (kg N/head/day)

Lj= number of days in each time-period (days). This is 91.25 days under Method 1 as the default time-period is a season

(8)	Nitrogen excreted in urine Ujk (kg N/head/day) is calculated as:

Ujk= CPIjk÷6.25 - NRijk - Fijk

Where		NRijk = amount of nitrogen retained by the body (kg N/head/day)

(9)	The amount of nitrogen retained by the body NRjk (kg N/head/day) is calculated as:

NRijk=0.045 ×MPjk=3,4+WPk ×0.84+EBGjk×212-4×EBGjk × 10004 × SRWk   0.75-1-140-4×EBGjk × 10004 × SRWk   0.75-11+e-6(Zjk - 0.4)×10-3÷6.25

Where 		MPjk=3,4= milk production (kg/head/day)

WPk = clean wool production (kg/head/day)

EBGjk  = empty body gain (kg)

SRWk = standard reference weight (kg)

Zjk = relative size

(10)	Milk production of sheep MPjk=3,4(kg/head/day) is calculated as:

MPjk=3,4= LEjk=3,4 ×1.6

Where 	LEjk=3,4 = proportion of ewes lactating each season or time period as calculated in Chapter 3 Section 3.4

1.6 = milk production for the season the lambs are born in (kg/head/day)

For sheep classes other than breeding ewes and maidens, milk production should be set to zero, i.e., MPjk=1,2,5,6= 0 and in non-breeding seasons milk production should be set to zero for all sheep classes.

Question Reference 4.6.

The Guidance assumes producers will be able to provide average greasy fleece weights and average clean wool yield per class. Is this appropriate?

Should defaults be provided for clean wool yield percentage as in the SB-GAF or are there other approaches, such as the Agriculture Victoria Data Request template, that could be used to help producers estimate clean wool production?

(11) Clean wool production WPk (kg/head/day) is calculated as:

WPk =GWk ×Yk365

Where		GWk = greasy wool production p class (kg/head)

Yk = average clean wool yield per sheep class (fraction)

365 = number of days in a year

(12)	The empty body gain EBGjk (kg) is calculated as:

EBGjk=LWGjk ×0.92

Where		LWGjk = liveweight gain (kg). Under Method 1, default values are applied.

(13)	The relative size of the sheep  Zjk is calculated as:

Zjk=Wjk SRWk

Where		Wjk = liveweight (kg). Under Method 1, default values are applied.

#### Method 2 — Soil Direct N2O Sheep

Method 2 is the same as Method 1 except that under equations 4.4.1.3 (3) and (4) feed intake Ijkl  is estimated as per section 3.2.1.2 and farm specific data is applied for the selected time-period (i.e. seasonal, monthly or other) for CPjk (Eq 4.2.1.3 (4) and (6)), DMDjk (Eq 4.2.1.3 (3) and (6)), Wjk (Eq 4.2.1.3 (13)), LWGjk (Eq 4.2.1.3 (12)).

Under Method 2 the selected time-period and associated Lj used in equation 4.2.1.3 (2) may be a season (Lj= 91.25 days), month (Lj=28-31 day depending on month) or a specific number of days in a month or season if the entry and exit from farm of a specific cohort of animals is being estimated

#### Method 1 — Soil Deposition N2O Sheep

(1)	Annual atmospheric deposition emissions from urine and dung deposited on pasture ESN2O,ad (t N2O) is calculated as:

EN2O,ad=Mvol×EFad×Cg,N2O×10-3

Where	Mvol = mass of nitrogen volatilised from urine and faeces deposited on pasture (kg N)

EFad = emission factor for atmospheric deposition (kg N2O-N/kg N)

The mass of nitrogen volatilised Mvol (kg N) is calculated as:

Mvol=AU+AF×FracGASMsoil

Where	FracGASMsoil = fraction of nitrogen volatilised from urine and faeces deposited on pasture ((kg NH3–N + NOx–N)/kg N)

#### Method 2 — Soil Deposition N2O Sheep

There is no Method 2 quantification option for this emission source.

#### Method 1 — Soil Leaching and Runoff N2O Sheep

(1)	Leaching and runoff emissions from urine and dung deposited on pasture ESN2O,leach (t N2O) are calculated as:

ESN2O,leach=Mleach×EFleach×Cg,N2O×10-3

Where		Mleach = mass of nitrogen lost to leaching and runoff (kg N)

EFleach = emission factor for leaching and run off (kg N2O-N/kg N)

(2)	The mass of nitrogen lost to leaching and runoff Mleach (kg N) is calculated as:

Mleach=AU+AF× FracWet×FracLEACH

Where	FracWet = fraction of N that is available for leaching and runoff in each climate zone.

FracLEACH = fraction of all N that is lost through leaching and runoff

#### Method 2 — Soil Leaching and Runoff N2O Sheep

There is no Method 2 quantification option for this emission source.

### Data/Parameters

#### Input Data (Required)

| Data / Parameter | Njk |
| --- | --- |
| Data unit | head |
| Description | Number of sheep in each time-period and class |
| Data source | Farm records and see herd flow model see Chapter 1 Section 1.9 |
| Quality assurance / quality control considerations | All animals purchased or sold in the reporting period are assumed to be reported. Number of sheep reported can be cross checked with stocking density allowances for system size. If value is significantly higher or lower than expected stocking density, flagged as possibly erroneous. Check the class of animals on farm marry up with expected enterprise on farm. For example, self-replacing systems vs purchased breeder or trading systems. |

| Data / Parameter | GWk |
| --- | --- |
| Data unit | kg/head |
| Description | Average greasy wool fleece weight for each sheep class |
| Data source | Farm source data – based on average wool fleece weight for each class. If not known this can be calculated from the total greasy weight sold divided by the number of head in each class shorn. . |
| Quality assurance / quality control considerations | Invoices from wool sales can be used to confirm the weight of wool sold in the reporting period. The average fleece weight can be considered in the context of the farming operation and breed of sheep farmed. Industry averages can be used to sense check average fleece weights for different breeds and age classes to help prevent data entry errors. |

| Data / Parameter | Yk |
| --- | --- |
| Data unit | fraction |
| Description | Average clean wool yield for each sheep class |
| Data source | Farm source data – based on wool testing results at point of sale for the clean wool percentage for each class. If only total greasy and total clean weights are known the fraction of clean can be calculated as: Y =cleanweightgreasyweight For more accurate estimation, it is recommended this value is calculated for ewes and lambs separately and applied to each class appropriately. |
| Quality assurance / quality control considerations | Invoices from wool sales can be used to confirm the percent clean wool sold in the reporting period. The clean wool yield can be considered in the context of the farming operation and breed of sheep farmed. Industry averages can be used to sense check average fleece weights for different breeds and age classes to help prevent data entry errors. |

#### Data (Method 1 and 2 Options)

| Data / Parameter | Dj |
| --- | --- |
| Data unit | days |
| Description | Duration of stay for each sheep input class |
| Method 1 data source | National Inventory Report Volume 1 [1] |
| Method 1 value | 91.25 |
| Method 2 data source | Farm stock records; system type records or purchase and sales can be evaluated to determine average duration of each sheep input class (see Chapter 1 Section 1.9 for more details on herd flow modelling) |
| Quality assurance / quality control considerations | Ensure that if animals are on the farm all year round the duration of stay is 365 days Ensure that if animals are only born part way through the reporting period their duration of stay reflect this. If Method 1 is used inputs must be entered seasonally throughout the calculations. |

| Data / Parameter | DMDjk |
| --- | --- |
| Data unit | fraction |
| Description | Dry matter digestibility of feed |
| Method 1 data source | National Inventory Report Volume 2, Table A5.5.4.2 [4] |
| Method 1 value | See Table 12.1.4.2 in Chapter 12 Appendix Select the appropriate default value for location of sheep, sheep class and season |
| Method 2 data source | Farm records - user input values for each season and class of stock. Noting that if farm specific DMD can be used for some classes or seasons if known and inventory defaults for other unknown classes of stock. |
| Quality assurance / quality control considerations | Compare Method 2 values to NIR default values with entered values to flag potential data entry errors |

| Data / Parameter | CPjk |
| --- | --- |
| Data unit | fraction |
| Description | Crude protein content of feed |
| Method 1 data source | National Inventory Report Volume 2, Table A5.5.4.4 [4] |
| Method 1 value | See Table 12.1.4.4 in Chapter 12 Appendix. Select the appropriate default value for location of sheep, sheep class and season |
| Method 2 data source | Farm records - user input values for each season and class of stock. Noting that if farm specific CP can be used for some classes or seasons if known and inventory defaults for other unknown classes of stock. |
| Quality assurance / quality control considerations | Compare Method 2 values to NIR default values with entered values to flag potential data entry errors |

| Data / Parameter | LWGjk |
| --- | --- |
| Data unit | kg |
| Description | Liveweight gain per time period and sheep class |
| Method 1 data source | National Inventory Report Volume 2, Table A5.5.4.5 [4] |
| Method 1 value | See Table 12.1.4.5 in Chapter 12 Appendix Select the appropriate default value for location of sheep, sheep class and season |
| Method 2 data source | Farm records and see herd flow model see Chapter 1 Section 1.9 |
| Quality assurance / quality control considerations | Purchase and sale weight records (invoices) can be used for data assurance and control of entered values. Dated print out, screen shot, or photo of scale unit records can also be used for quality assurance of entered values. It is recommended Method 1 default values are sense checked against production data and system to ensure they are appropriate e.g. that default liveweight gain reflects when stock are expected to be growing more due to feed quality and availability. |

| Data / Parameter | Wjk |
| --- | --- |
| Data unit | kg |
| Description | Liveweight of livestock per time period and sheep class |
| Method 1 data source | National Inventory Report Volume 2, Table A5.5.4.1 [4] |
| Method 1 value | See Table 12.1.4.1 in Chapter 12 Appendix Select the appropriate default value for location of sheep, sheep class and season |
| Method 2 data source | Farm records and see herd flow model see Chapter 1 Section 1.9 |
| Quality assurance / quality control considerations | Purchase and sale weight records (invoices) can be used for data assurance and control of entered values. Dated print out, screen shot, or photo of scale unit records can also be used for quality assurance of entered values. It is recommended Method 1 default values are sense checked against production data and system to ensure they are appropriate e.g. if lambing in spring ensure default weights reflect this with lowest weigh value applied in spring for stock <1 year and/or that default weights reflect when stock are expected to be at the lightest and heaviest due to feed availability and quality. |

#### Constants

| Data / Parameter | B0 |
| --- | --- |
| Data unit | m3 CH4/kg VS |
| Description | Emissions potential |
| Data source | IPCC (2019), Chapter 10 [5] |
| Value | 0.24 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian NIR. |

| Data / Parameter | MMSm |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of waste in each MMS |
| Data source | National Inventory Report, Volume 1 [1] |
| Value | If the farm has unfenced water sources inventory defaults of MMSm=14 = 0.95 and MMSm=1 = 0.05 If animals are on bore or reticulated water systems where water is provided in troughs (i.e. no unfenced natural water sources) then all waste should be allocated to pasture range and paddock and MMSm=14 = 1 and MMSm=1 = 0. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian NIR. High resolution aerial imagery can be used to cross check that water sources are stock excluded. |

| Data / Parameter | MCFim |
| --- | --- |
| Data unit | fraction |
| Description | Methane conversion factor |
| Data source | M14 = National Inventory Report, Volume 1 [1] M1 = IPCC 2019, Volume 4, Chapter 10, Table 10.17 [12] |
| Value | Methane conversion factor is dependent on climate zone. See Table 12.1.4.8 in Chapter 12 Appendix for these climate zone dependent values. Refer to Chapter 1 Section 1.8 on guidance for determining which climate zone the farm is located in. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian NIR and IPCC refinements. |

Question Reference 4.7.

The current Guidance allows users to select the most appropriate emission factor for atmospheric deposition based on the grazing system (e.g irrigated or non-irrigated pasture).

For sheep, should the default assumption be that they graze on non-irrigated pasture? Or does this vary between locations or animal classes? For example, are lambs commonly finished on irrigated pastures or crops, while breeding ewes are typically maintained on non-irrigated pasture?

| Data / Parameter | EFad |
| --- | --- |
| Data unit | kg N2O-N/kg N |
| Description | Emission factor for atmospheric deposition. |
| Data source | National Inventory Report Volume 1, Table 5.21 [1] |
| Value | See Table 12.2.2.3 in Chapter 12 Appendix This is the same emissions factor as used for direct nitrous oxide emissions from inorganic fertilisers. Apply the EF that best represent the production system In some farm systems animals may graze on crops. Apply the inorganic fertiliser EF that best representing how the sheep are grazed for the majority of the year or calculate a weighted average if the number of days on each grazing system is known. For non-irrigated cropping systems, the emission factor varies depending on whether the agricultural system is in a high rainfall or low rainfall zone. Refer to Chapter 1 Section 1.8 for guidance on determining the relevant rainfall zone. |
| Quality assurance / quality control considerations | Ensure alignment with the latest version of the Australian National Inventory Report |

| Data / Parameter | FracWET |
| --- | --- |
| Data unit | Fraction |
| Description | Fraction of nitrogen that is available for leaching and runoff |
| Data source | National Inventory Report Volume 1 [1] |
| Value | To determine whether the entity is located in an area where leaching occurs refer to Chapter 1 Section 1.8 on how to determine whether the enterprise is located in a leaching zone. Where it is determined that leaching does occur, FracWET = 1 Where it is determined that leaching does not occur, FracWET = 0 |
| Quality assurance / quality control considerations | Farm title data can be checked to ensure spatial farm boundaries used to define climate zone are correct. Other inputs such as electricity or fuel of irrigation purposes can be used to highlight if pastures are irrigated and therefore leaching will be occurring. |

| Data / Parameter | p |
| --- | --- |
| Data unit | kg/m3 |
| Description | Density of methane |
| Data source | National Greenhouse and Energy Reporting (Measurement) Determination 2008 [6] |
| Value | 0.6784 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian NIR. |

Question Reference 4.8.

Ash content of feed intake for sheep is currently set at 0.08.  Stakeholders have previously suggested this value should be higher. Is there data available to support a revision to this parameter?

| Data / Parameter | A |
| --- | --- |
| Data unit | fraction |
| Description | Fractional ash content of feed intake. |
| Data source | National Inventory Report, Volume 1 [1] |
| Value | 0.08 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian NIR. |

| Data / Parameter | EFPRP |
| --- | --- |
| Data unit | kg N2O-N/kg N |
| Description | Emission factor urine and dung deposited on pasture, range and paddock |
| Data source | IPCC 2019, Volume 4, Chapter 11, Table 11.1 [11] |
| Value | 0.004 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian NIR. |

| Data / Parameter | SRWk |
| --- | --- |
| Data unit | kg |
| Description | Standard reference weight |
| Data source | National Inventory Report Volume 2, Table A5.5.4.7 [4] |
| Value | See Table 12.1.4.6 in Chapter 12 Appendix Select appropriate default value for location of sheep and sheep class |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian NIR. |

| Data / Parameter | FracGASMsoil |
| --- | --- |
| Data unit | (kg NH3–N + NOx–N)/kg N |
| Description | Fraction of nitrogen volatilised from urine and faeces deposited on pasture |
| Data source | IPCC 2019, Volume 4, Chapter 11, Table 11.3 [11] |
| Value | 0.21 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian NIR. |

| Data / Parameter | FracLEACH |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of N that is lost through leaching and runoff |
| Data source | National Greenhouse Gas Inventory Volume 1 [13, p. 1] |
| Value | 0.24 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Greenhouse Gas Inventory. |

| Data / Parameter | EFleach |
| --- | --- |
| Data unit | kg N2O-N/kg N |
| Description | Emission factor for leaching and runoff |
| Data source | National Greenhouse Gas Inventory Volume 1 [13, p. 1] |
| Value | 0.011 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian NIR. |

## References

[1]	Department of Climate Change, Energy, the Environment and Water, “National Inventory Report 2023, Volume I.” Australian Government, 2023. [Online]. Available: https://www.dcceew.gov.au/sites/default/files/documents/national-inventory-report-2023-volume-1.pdf

[2]	E. J. McGahan, K. D. Casey, H. van Sliedregt, E. A. Gardner, P. J. Watts, and R. W. Tucker, “Beefbal – A nutrient balance model for feedlots,” Department of Primary Industries, Toowoomba, version 1.1, 2004.

[3]	S. Wiedemann, T. Sullivan, and E. McGahan, “GHG prediction methods for feedlots, poultry and pigs.,” Federal Department of the Environment, Canberra, Technical report for the Department of Environment Greenhouse Gas Inventory Team.

[4]	Department of Climate Change, Energy, the Environment and Water, “National Inventory Report 2023, Volume II.” Australian Government, 2023. [Online]. Available: https://www.dcceew.gov.au/sites/default/files/documents/national-inventory-report-2023-volume-2.pdf

[5]	IPCC, “Volume 4: Agriculture, Forestry and Other Land Use. Chapter 10: Emissions from Livestock and Manure Management.” IPCC, 2019.

[6]	Office of Parlimentary Counsel, Canberra, “National Greenhouse and Energy Reporting (Measurement) Determination 2008.” Department of Climate Change, Energy, the Environment and Water, 2024. [Online]. Available: https://www.legislation.gov.au/F2008L02309/latest/downloads

[7]	IPCC Expert Groups, “IPCC Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories. Chapter 4: Agriculture.” IPCC. [Online]. Available: https://www.ipcc-nggip.iges.or.jp/public/gp/english/4_Agriculture.pdf

[8]	SCA, “Feeding Standards for Australian Livestock: Ruminants.,” CSIRO, Melbourne, Australia, 1990.

[9]	M. Freer, A. D. Moore, and J. R. Donnelly, “GrazPlan – Decision Support Systems For Australian Grazing Enterprises. II. The Animal Biology Model For Feed Intake, Production and Reproduction and the Grazfeed DSS,” Agricultural Systems, vol. 54, pp. 77–126, 1997.

[10]	D. J. Minson and C. K. McDonald, “Estimating Forage Intake from the Growth of Beef Cattle,” Tropical Grasslands, vol. 21, no. 3, Sept. 1987, [Online]. Available: chrome-extension://dbchgeokljmcmdjbpfiaagmjdkohknec/content-script/index.html?file=https%253A%252F%252Fwww.tropicalgrasslands.info%252Fpublic%252Fjournals%252F4%252FHistoric%252FTropical%252520Grasslands%252520Journal%252520archive%252FPDFs%252FVol_21_1987%252FVol_21_03_87_pp116_122.pdf

[11]	IPCC, “Chapter 11: N2O Emissions from Managed Soils, and CO2 Emissions from Lime and Urea Application,” IPCC Guidelines for National Greenhouse Gas Inventories, Volume 4, 2019. [Online]. Available: chrome-extension://dbchgeokljmcmdjbpfiaagmjdkohknec/content-script/index.html?file=https%253A%252F%252Fwww.ipcc-nggip.iges.or.jp%252Fpublic%252F2019rf%252Fpdf%252F4_Volume4%252F19R_V4_Ch10_Livestock.pdf

[12]	IPCC, “Chapter 10: Emissions from Livestock and Manure Management,” IPCC Guidelines for National Greenhouse Gas Inventories, Volume 4, 2019. [Online]. Available: chrome-extension://dbchgeokljmcmdjbpfiaagmjdkohknec/content-script/index.html?file=https%253A%252F%252Fwww.ipcc-nggip.iges.or.jp%252Fpublic%252F2019rf%252Fpdf%252F4_Volume4%252F19R_V4_Ch10_Livestock.pdf

[13]	Department of Climate Change, Energy, the Environment and Water, “National Inventory Report 2022: Volume 1,” Australian Government, 2024. [Online]. Available: https://www.dcceew.gov.au/sites/default/files/documents/national-inventory-report-2022-volume-1.pdf

[14]	KM Christie-Whitehead and Dairy Australia, “Australian Dairy Carbon Calculator,” Tasmanian Institute of Agriculture; Dairy Australia, Launceston, Tasmania; Melbourne, Victoria, Feb. 2025. [Online]. Available: https://www.dairyaustralia.com.au/en/climate-and-environment/greenhouse-gas-emissions/australian-dairy-carbon-calculator

[15]	[Core Writing Team IPCC R. K. Pachauri and L. A. Meyer (eds. )], “Climate Change 2014: Synthesis Report. Contribution of Working Groups I, II and III to the Fifth Assessment Report of the Intergovernmental Panel on Climate Change.” IPCC, 2014.