# DRAFT Greenhouse Gas Emissions Estimation and Reporting Guidelines for Agriculture, Fisheries and Forestry

Methodological Guidance

Chapters 14 - 15: Scope 2 and Scope 3

© Commonwealth of Australia 2026

Ownership of intellectual property rights

Unless otherwise noted, copyright (and any other intellectual property rights) in this publication is owned by the Commonwealth of Australia (referred to as the Commonwealth).

Creative Commons licence

All material in this publication is licensed under a Creative Commons Attribution 4.0 International Licence except content supplied by third parties, logos and the Commonwealth Coat of Arms.

Inquiries about the licence and any use of this document should be submitted via our online contact form.

Cataloguing data

This publication (and any material sourced from it) should be attributed as: DCCEEW 2026, DRAFT Greenhouse Gas Emissions Estimation and Reporting Guidelines for Agriculture, Fisheries and Forestry - Methodological Guidance, Department of Climate Change, Energy, the Environment and Water, Canberra, March. CC BY 4.0.

This publication is available at dcceew.gov.au.

Department of Climate Change, Energy, the Environment and Water

GPO Box 3090 Canberra ACT 2601

Telephone 1800 920 528

Web dcceew.gov.au

Disclaimer

The Australian Government acting through the Department of Climate Change, Energy, the Environment and Water has exercised due care and skill in preparing and compiling the information and data in this publication. Notwithstanding, the Department of Climate Change, Energy, the Environment and Water, its employees and advisers disclaim all liability, including liability for negligence and for any loss, damage, injury, expense or cost incurred by any person as a result of accessing, using or relying on any of the information or data in this publication to the maximum extent permitted by law.

Acknowledgements

We thank Environmental Accounting Services for drafting this document, and the Voluntary Greenhouse Gas Estimation and Reporting Standards Reference Group for their valued contributions.

Acknowledgement of Country

We acknowledge the Traditional Owners of Country throughout Australia and recognise their continuing connection to land, waters and culture. We pay our respects to their Elders past and present.

Contents

14	Scope 2: Emissions from Purchased Electricity	5

14.1	Estimation methodology	5

14.1.1	Purchased electricity (location-based)	5

14.1.2	Purchased electricity (market-based)	5

14.2	Data/Parameters	6

14.2.1	Input Data (Required)	6

14.2.2	Data (only required for market-based approach)	7

14.2.3	Constant Parameters	7

15	Scope 3 Emissions	10

15.1	Purchased livestock	12

15.1.1	Estimation methodology	14

15.1.2	Data/Parameters	15

15.2	Purchased feed	19

15.2.1	Estimation methodology	19

15.2.2	Data/Parameters	20

15.3	Purchased mineral supplements for livestock	22

15.3.1	Estimation methodology	22

15.3.2	Data/Parameters	22

15.4	Purchased fertiliser	24

15.4.1	Estimation methodology	24

15.4.2	Data/Parameters	25

15.5	Purchased agrichemicals	27

15.5.1	Estimation methodology	27

15.5.2	Data/Parameters	27

15.6	Purchased lime	30

15.6.1	Estimation methodology	30

15.6.2	Data/Parameters	30

15.7	Purchased services and contractors	32

15.7.1	Estimation methodology	32

15.7.2	Data/Parameters	33

15.8	Purchased packaging	34

15.8.1	Estimation methodology	34

15.8.2	Data/Parameters	35

15.9	Purchased grow media	36

15.9.1	Estimation methodology	36

15.9.2	Data/Parameters	37

15.10	Other purchased goods and services	38

15.11	Upstream emissions from fuel	38

15.11.1	Estimation methodology	39

15.11.2	Data/Parameters	40

15.12	Upstream emissions from purchased electricity	42

15.12.1	Estimation methodology	42

15.12.2	Data/Parameters	43

15.13	Management of waste	45

15.13.1	Estimation methodology	45

15.13.2	Data/Parameters	46

15.14	Upstream freight	47

15.14.1	Estimation methodology	47

15.14.2	Data/Parameters	48

References	49

# Scope 2: Emissions from Purchased Electricity

Scope 2 emissions are indirect emissions from the generation of purchased energy, such as electricity, steam, heat, or cooling consumed by the entity (e.g. emissions from the power station producing purchased electricity).

This module covers the estimation of emissions that result from the consumption of electricity that has been purchased or acquired from the grid. Refer to Section 6.5 in the Common Requirements Framework (CRF) for more information.

The Scope 2 emission estimation does not include on-farm electricity generation, such as solar power (refer to Chapter 1 Section 1.10 for more information).

The Scope 2 emission estimation only accounts for the emissions generated from the physical burning of fuels at the power station to create electricity. Upstream and downstream emissions in the electricity supply chain, such as raw material transportation to the power station and transmission and distribution losses, are included as a Scope 3 emission.

## Estimation methodology

Scope 2 emissions shall be estimated using the location-based method. The market-based method may be used in addition to the location-based method, for entities wanting to report under both methods.

### Purchased electricity (location-based)

The location-based method reflects an entity’s electricity emissions in the context of its location and uses an emissions factor that reflects the emissions intensity of electricity generation within the state or territory where it operates. The approach for estimating location-based Scope 2 emissions is based on the approach of the National Greenhouse Accounts Factors (NGAF).

- The location-based Scope 2 emissions from purchased electricity, E2,elec (t CO2e), are estimated by:

E2,elec=Qelec×EF2,elec ×10-3

Where		Qelec = amount of electricity purchased from the grid (kWh)

EF2,elec = location-based Scope 2 emission factor for electricity (kg CO2e /kWh)

### Purchased electricity (market-based)

The market-based method reflects a company’s electricity emissions in the context of its investments in different electricity products and markets. Under this method, an entity that voluntarily purchases renewable energy (e.g., GreenPower electricity) or participates in the Large-scale Renewable Energy Target scheme (LRET) will be recognised for its investments in these renewables.

The approach for estimating market-based Scope 2 emissions is adapted from the National Greenhouse Accounts Factors (NGAF).

- The market-based Scope 2 emissions from purchased electricity, E2,elec (t CO2e), are estimated as:

E2,elec=(Qelec×1-(RPP+JRPP)-(RECsurrendered-REConsite ×10-3)) × EFRMF2,elec ×10-3

Where		Qelec = amount of electricity purchased from the grid (kWh)

EFRMF2,elec = Scope 2 residual mix emission factor for electricity (kg CO2e/kWh)

RPP = Renewable Power Percentage under the Large-scale
Renewable Energy Target (LRET) for the applicable period (per cent)

JRPP = jurisdictional renewable power percentage for the applicable period (only applies to the ACT) (per cent)

RECsurrendered = number of eligible Renewable Energy Certificates voluntarily surrendered in the reporting year equivalent to megawatt hours (MWh)

REConsite = number eligible Renewable Energy Certificates that have been or will be issued for electricity produced on-site during the year and consumed by the entity equivalent to megawatt hours (MWh)

Large-scale generation certificates (LGCs) that have been purchased or issued for electricity generated under the LRET, and are voluntarily surrendering through the REC Registry are reported under RECsurrendered.

Electricity purchased from an accredited GreenPower provider should also be reported as RECsurrendered, based on the amount of electricity purchased (in MWh). Where an entity sources all of its electricity consumption during the reporting period from a GreenPower provider, the surrendered RECs may be calculated as RECsurrendered=Qelec×10-3.

If an entity has a liability under the RET scheme and has been given exemption, they should estimate the market-based emissions using the non-adapted approach in the National Greenhouse Accounts Factors (NGAF).

## Data/Parameters

### Input Data (Required)

| Data / Parameter | Qelec |
| --- | --- |
| Data unit | kWh |
| Description | The amount of electricity purchased from the grid |
| Data source | Invoices |
| Quality assurance / quality control considerations | Data on electricity use should be collected across the entire reporting period. Where invoice periods do not align with the reporting period, electricity consumption may be estimated based on interpolating the available data. |

### Data (only required for market-based approach)

| Data / Parameter | RECsurrendered |
| --- | --- |
| Data unit | MWh |
| Description | Eligible Renewable Energy Certificates voluntarily surrendered in the reporting year |
| Location-based approach data source | N/A – this parameter is not used in the location-based approach |
| Location-based approach value | N/A – this parameter is not used in the location-based approach |
| Market-based approach data source | Retailer invoice or Clean Energy Regulator REC registry |
| Quality assurance / quality control considerations | An eligible Renewable Energy Certificate is: a Large-scale Generation Certificate (LGC) that is voluntarily surrendered and has a generation date of less than 36 months prior to the end of the reporting year; or a purchase of GreenPower electricity from an accredited GreenPower Provider. Note: It is anticipated that upon commencement of the Renewable Electricity Guarantee of Origin (GO) scheme late in 2025, that GO certificates will also be added to the list of eligible certificates. |

| Data / Parameter | REConsite |
| --- | --- |
| Data unit | MWh |
| Description | Eligible Renewable Energy Certificates that have been or will be issued for electricity produced on-site during the reporting period and consumed by the entity |
| Location-based approach data source | N/A – this parameter is not used in the location-based approach |
| Location-based approach value | N/A – this parameter is not used in the location-based approach |
| Market-based approach data source | Clean Energy Regulator REC registry and Meter readings |
| Quality assurance / quality control considerations | An eligible Renewable Energy Certificate in this context is a Large-scale Generation Certificate (LGC). |

### Constant Parameters

| Data / Parameter | EF2,elec |
| --- | --- |
| Data unit | kg CO2e /kWh |
| Description | Location-based emission factor for Scope 2 emissions of purchased electricity. Applicable only to the location-based approach. |
| Data source | Australian National Greenhouse Accounts Factors Table 1 |
| Value | Refer to the most recently published National Greenhouse Accounts Factors |
| Quality assurance / quality control considerations | The NGAF publish EF2,elec values annually, ensure that the emission factor is selected based on the year and State in which the grid is located to which the farming enterprise is purchasing its electricity from. The emission factors are based on financial years (e.g. 2025 NGA Factors report provide EF for 2025-26). If an entity is required to report on a calendar year basis, they should collate electricity use data per 6-month period and multiply each total by the emission factor for that financial year, as published in the NGA Factors reports. If only annual figures are available, entities should apply an emissions factor that is the average for the two financial years covered by the calendar year. |

| Data / Parameter | EFRMF2,elec |
| --- | --- |
| Data unit | kg CO2e /kWh |
| Description | Residual mix emission factor for Scope 2 emissions of purchased electricity Applicable only to the market-based approach. |
| Data source | Australian National Greenhouse Accounts Factors Table 2 |
| Value | Refer to the most recently published National Greenhouse Accounts Factors |
| Quality assurance / quality control considerations | The NGAF publish EFRMF2,elec values annually, ensure that the emission factor is selected based on the year and State in which the grid is located to which the farming enterprise is purchasing its electricity from. The emission factors are based on financial years (e.g. 2025 NGA Factors report provide EF for 2025-26). If an entity is required to report on a calendar year basis, they should collate electricity use data per 6-month period and multiply each total by the emission factor for that financial year, as published in the NGA Factors reports. If only annual figures are available, entities should apply an emissions factor that is the average for the two financial years covered by the calendar year. |

| Data / Parameter | RPP |
| --- | --- |
| Data unit | per cent |
| Description | Large Renewable Power Percentage under the Large-scale Renewable Energy Target Applicable only to the market-based approach. |
| Data source | Clean Energy Regulator (CER) |
| Value | Refer to the most recently published data by the CER |
| Quality assurance / quality control considerations | The CER publishes RPP annually, ensure that the appropriate value is selected for the year of the reporting period. The RPPs are based on the calendar year. If an entity is required to report on a financial year basis, they should, should apply the RPP that is the average for the two calendar years covered by the financial year. |

| Data / Parameter | JRPP |
| --- | --- |
| Data unit | per cent |
| Description | Jurisdictional renewable power percentage Applicable only to the market-based approach. |
| Data source | Australian National Greenhouse Accounts Factors Applicable only to enterprises located in ACT |
| Value | Refer to the most recently published National Greenhouse Accounts Factors |
| Quality assurance / quality control considerations | The NGAF publish JRPP values annually, ensure that the appropriate value is selected for the year of the reporting period. The JRPPs are calculated on a financial-year basis. If an entity is required to report on a calendar year basis, they should apply a JRPP that is the average for the two calendar years covered by the financial year. |

# Scope 3 Emissions

For Scope 3 emissions from purchased goods and services, coverage of methods reflects open-source data available at the time of writing but may be updated prior to finalisation of this Guidance based on updates to the AusLCI Carbon Emissions Factors.

Question Reference 15.1.

Where AusLCI CEF data are not available, the scope 3 EFs used have been aligned with the Greenhouse Gas Accounting Framework- tools. It is anticipated that into the future EFs in the Guidelines will be updated using default EFs sourced from publicly available life-cycle assessment databases.

Are there any additional life-cycle based EFs for purchased goods and services suitable to Australian agricultural enterprises that can be accessed without licencing restrictions?

Question Reference 15.2.

A Method 2 pathway is provided to allow higher-specificity alternatives to default emission factors for purchased goods and services. It is proposed that supplier-provided product emission factors be third-party verified as conforming with:
• ISO 14067:2018, or
• Greenhouse Gas Protocol Product Life Cycle Accounting and Reporting Standard.

Should the Method 2 pathway also allow approaches described in the GHG Protocol Scope 3 Technical Guidance, including:
• emission factors from life-cycle databases other than AusLCI
• supplier-specific product life-cycle emission factors conformant with the GHG Protocol Product Standard
• hybrid methods combining supplier Scope 1, Scope 2, and key Scope 3 activity data?

If Method 2 pathways are adopted, should third-party verification be required, or would self-declared emission factors conformant with ISO 14021:2016 (Environmental labels and declarations — Self-declared environmental claims) be sufficient?

This module provides methods for estimating upstream scope 3 emissions associated with purchased goods and services, fuel- and energy-related activities, transport and distribution, and waste generated in operations, in alignment with the GHG Protocol Scope 3 reporting categories (see table below).

The methods included in this Guidance rely on the use of default emissions factors sourced from published literature and the Australian Life Cycle Inventory Database Initiative Carbon Emissions Factors (AusLCI CEF) [1], which are publicly available and free from licensing restrictions. Consequently, the Guidance does not include methods for all upstream reporting categories—such as capital goods, business travel, employee commuting, and non-land leased assets—nor does it cover all possible purchased goods and services.

Where these excluded categories or sources are relevant and material to an entity’s inventory, emissions may be estimated using emission factors from reputable national and international databases (e.g. AusLCI Datasets, Ecoinvent). Entities which choose to include additional categories are advised to apply geographically and technologically appropriate emission factors, which are transparently documented, including key assumptions and limitations. Estimated emissions should be clearly identified within the inventory, with ongoing efforts to improve data quality where feasible. Any supplier-provided emission factors shall conform to the same guidance as that for any supplier-provided Scope 3 life-cycle emission factor: shall be third-party verified as conforming either to ISO 14067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard.

The emissions factors applied should be consistent with the AusLCI CEF carbon neutrality assumption without infrastructure method. This method includes factors from sources such as fossil fuels, as well as from biogenic methane, but excludes upstream impacts from capital equipment and biogenic carbon that may be sequestered during growth but is subsequently released back into the atmosphere.

Method 2 Emission Factors for Purchased Goods

The Method 2 approach for emission estimation of Scope 3 purchased goods (e.g., purchased livestock, purchased feed, etc.) requires the use of supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 14067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. Product use or end-of-life processes shall be excluded from the cradle-to-gate EFs used in Scope 3 emission estimation of purchased goods. Where possible, it is recommended to exclude capital goods and infrastructure from the LCA boundary, to be consistent with the carbon neutrality assumption without infrastructure of the AusLCI CEF.

| Indirect GHG emissions (Scope 3) | Reporting Category |
| --- | --- |
| Purchased livestock | Category 1 – Purchased goods and services |
| Purchased feed |  |
| Purchased mineral supplements |  |
| Purchased fertiliser |  |
| Purchased agrichemicals |  |
| Purchased lime |  |
| Purchased services/contractors |  |
| Purchased packaging |  |
| Purchased grow media |  |
| Other purchased goods |  |
| Well-to-tank emissions of transport and stationary combustion fuels | Category 3 – Fuel- and energy-related activities not included in Scope 1 or Scope 2 |
| Upstream emissions of purchased electricity |  |
| Upstream freight | Category 4 – Upstream Transportation and Distribution |
| Management of waste | Category 5 – Waste generated in operations |

## Purchased livestock

This module covers the estimation of embedded greenhouse gas emissions associated with purchased livestock. Embedded emissions in purchased livestock include the emissions that have been generated in the raising of the animal on a different farm, before being brought into the farm.

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| c | Livestock type |
| o | Origin of purchased livestock (applicable only to cattle not of dairy origin) |

The following livestock types c are included in this module:

- Dairy cattle

- Beef cattle

- Sheep (merino and cross-bred sheep)

- Poultry (meat and layers)

- Swine and

- Other Livestock - which includes buffalo, goats, deer, camels, alpacas, horse, mules/asses, emus/ostriches.

Question Reference 15.3.

Are ‘purchased livestock’ emissions factors available for aquaculture purchases of broodstock or juvenile fish, mollusk or crustaceans?

Question Reference 15.4.

Should purchased ‘eggs’ be included in ‘purchased livestock’? If yes are emissions factors available?

Question Reference 15.5

AusLCI has a new option for purchased ’chicken breeding and hatching, conventional rearing systems” with units of “per chick”. Would the ’per chick’ unit  be preferred to current guidance of per kg for purchased poultry? Should both quantification approaches be available?

Livestock types shall be selected to reflect the type of livestock purchased. For example, if a mixed sheep and beef farm purchased sheep that stem from a merino enterprise, the merino sheep livestock type should be selected.

Emission factors of purchased livestock, other than for cattle, sheep, poultry and swine, are limited. In the absence of livestock-specific emission factors, it is assumed that the emission factors of either sheep or beef cattle can be scaled to other livestock, based on the comparative size of the animal. This approach is consistent with approach in National Inventory Report[2] and assumptions in other modules. The attribution of emission factors to each livestock types are summarised below:

Question Reference 15.6.

The National Inventory Report uses 1/3 of the EF of beef cattle for deer and mules/asses to calculate methane production of manure (Table A5.5.7.2). It was determined the purchased EF would be too low for these livestock types if 1/3 of the beef EF was applied so average sheep EF was used. Can the industry advise if this assessment needs to be reviewed, especially considering the impact of embedded emissions in ruminants versus non-ruminants?

| Livestock Type c | Methodology Assumption |
| --- | --- |
| Buffalo | Use average of beef cattle EF |
| Goats | Use average of sheep EF |
| Deer | Use average of sheep EF |
| Camels | Use average of beef cattle EF |
| Alpacas | Use average of sheep EF |
| Horse | Use average of beef cattle EF |
| Mules/Asses | Use average of sheep EF |
| Emus/Ostriches | Use average of sheep EF |

### Estimation methodology

#### Method 1 — Purchased Livestock

- Total emissions from purchased livestock, E (t CO2e), are calculated as:

E=co(Nco×Wco × EFco)×10-3

Where		Nco = number of animals purchased in livestock type c from origin o (head)

Wco  = average liveweight of purchased livestock at point of purchase (kg/head)

EFco = emission factor per livestock type and origin (kg CO2e/kg)

#### Method 2 — Purchased Livestock

Method 2 is the same as Method 1, except that entity-specific values for Wco and/or EFco are required.

### Data/Parameters

#### Input Data (Required)

| Data / Parameter | Nco |
| --- | --- |
| Data unit | Head |
| Description | Number of animals purchased in livestock type c from origin o |
| Data source | Farm stock records, invoices |
| Quality assurance / quality control considerations | All animals purchased in the reporting period shall be reported. Nco should not exceed the values used for livestock numbers in Scope 1 calculations for enteric fermentation or manure management emissions. |

#### Data (Method 1 and 2 Options)

Question Reference 15.7.

Under Method 1, should the liveweight of purchased livestock be based on the based on default weights used in provided in Chapter 3 (Enteric Fermentation) and Chapter 4 (Manure Management), or should farmers be required to report actual recorded weights at the time of purchase for Method 1?

| Data / Parameter | Wco |
| --- | --- |
| Data unit | kg/head |
| Description | Liveweight of purchased livestock of type c from origin o at point of purchase |
| Method 1 data source | Dairy cattle – see Appendix Table A.1.3.1 and A.1.3.2 Beef cattle – see Appendix Table A.1.2.1 and A.1.2.2 Merino and cross-bred sheep – see Appendix Table A.1.4.1 Swine – see Appendix Table A.1.6.5 and A.1.6.6 Poultry – see Appendix Table A.1.7.5 Buffalo, goats, deer, camels, alpacas, horse, mules/asses and emus/ostriches – see Appendix Table A.1.5.4 |
| Method 1 value | Dairy cattle – select appropriate value based on animal class. Beef cattle – select appropriate value based on class, state and regions of origin, and season. Select the liveweight based on these parameters at point of purchase (i.e., the season the livestock was purchased). Merino and cross-bred sheep – select appropriate value based on class, state of origin, and season Note that the National Inventory Report does not provide separate liveweight estimates for merino and cross-bred sheep. Select the liveweight based on these parameters at point of purchase (i.e., the season the livestock was purchased). Swine – select appropriate value based on livestock type and/or age range. Select the liveweight based on these parameters at point of purchase (i.e., age range when animal was purchased) Poultry – select appropriate value based on livestock class at point of purchase. Buffalo, goats, deer, camels, alpacas, horse, mules/asses and emus/ostriches – select appropriate value based on livestock type, and average weight option best aligned with weight at point of purchase. A weighted average liveweight of all livestock of type c from origin o should be used, where animals of different classes are purchased (applicable to both Method 1 and Method 2 approaches). For example, for a farm purchasing a total of 30 beef cattle, if 20 bulls of average liveweight 650 kg and 10 heifers of average liveweight 220 kg were purchased, the weighted average Wco would be: Wco=20×650+10×22020+10=507 kg/head |
| Method 2 data source | Purchasing records/invoices |
| Quality assurance / quality control considerations | Liveweight data from purchasing records/invoices could be compared to the National Inventory Report default values to flag potential data entry errors. |

Question Reference 15.8.

The emission factor for purchased meat poultry has been applied to layer systems due to lack of life cycle assessment research in Australia. Are there other industry EF’s which could be employed for layer poultry?

Question Reference 15.9.

Purchased livestock EF for other livestock categories are based on EF’s for either beef or sheep (refer to Reference Question 15.5). Are there alternative industry EF’s which should be employed for any of the other livestock types?

| Data / Parameter | EFco |
| --- | --- |
| Data unit | kg CO2e/kg |
| Description | Emission factor for purchased livestock per type of livestock c and origin o |
| Method 1 data source | Refer to the AusLCI CEF carbon neutrality method with infrastructure excluded [1] |
| Method 1 value | Dairy cattle - Select appropriate value based on animal class, i.e. cull cows may use “Spent dairy cow, market for cull dairy cow” option. Prime dairy heifers may use “prime beef cattle” option based on region. Beef cattle - Select appropriate value based on animal class and region. Sheep - Select appropriate value based on animal class (i.e. breeding stock, mutton, prime lamb, wether weaners) and region. If applicable, may include breed (i.e. merino vs non-merino) in selection criteria. Swine - Select appropriate value based on manure management system (i.e. covered pond, uncovered pond, deep litter, outdoor rearing) which best aligns with production system on farm. Poultry - Select appropriate value based on options available. As there is only one country wide option for chickens available (“conventional rearing system, meat chicken production”). All poultry types may use this value unless a Method 2 data source has been identified. Other Livestock - Select EF based on methodology assumptions: - Buffalo – use average of beef cattle EF - Goats – use average of sheep EF - Deer – use average of sheep EF - Camels – use average of beef cattle EF - Alpacas – use average of sheep EF - Horse – use average of beef cattle EF - Mules/Asses – use average of sheep EF - Emus/Ostriches – use average of sheep EF |
| Method 2 data source | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 14067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. |
| Quality assurance / quality control considerations | Supplier-provided life-cycle emission factors (Method 2) could be compared to the National Inventory Report default values to flag potential data entry errors. |

#### Constant Parameters

N/A

## Purchased feed

This module covers the estimation of embedded greenhouse gas emissions associated with purchased feed for livestock and aquaculture. The embedded emissions from purchased feed are emissions that result from the production of the feed.

The following subscripts are used in this module

| Subscript | Meaning |
| --- | --- |
| j | Type of feed purchased |

Feed types for livestock may include;

- Forage (such as hay, silage, legumes),

- Other concentrates (such as cereals, creep feeds, oil seeds, meat and blood meal, millrun etc).

Feed types for aquaculture may include;

- Complete feeds (such as low animal protein formulated feeds), and

- Other additions (such as squid and fish).

### Estimation methodology

#### Method 1 — Purchased Feed

- Emissions from purchased feed E (t CO2e) are calculated as:

E=j(Qj×EFj)

Where 		Qj = quantity of feed type j purchased (t)

EFj = emission factor of purchased feed (kg CO2e/kg)

#### Method 2 — Purchased Feed

Method 2 is the same as Method 1, except that a verified supplier-provided value for EFj is required.

### Data/Parameters

#### Input Data (Required)

| Data / Parameter | Qj |
| --- | --- |
| Data unit | t |
| Description | Quantity of feed type j purchased |
| Data source | Invoices, receipts or other records of purchased quantities |
| Quality assurance / quality control considerations | Qj does not include any home-grown feed. Qj should be based on the quantity of feed purchased within the reporting period where it can be reasonably assumed that all feed purchased was consumed in that reporting period. Where feed is stored and used in the subsequent reporting period, it may be included in the subsequent reporting period emission estimation instead. Where feed is purchased and then not used and instead becomes a waste, it shall still be included in Qj for the purpose of annual emission estimation. |

#### Data (Method 1 and 2 Options)

Question Reference 15.10.

Purchased feed has been updated to include all options currently available in AusLCI. A gap has been identified for aquaculture, as AusLCI does not yet include all fish feed types. The current guidance references the Blueshift tool feed EF’s  (available within the Appendix). Should aquaculture producers have the option of using the Blueshift values or should they be required to select the next closest feed ingredient from the AusLCI? Are there other feed emission factor options for aquaculture which could be included?

| Data / Parameter | EFj |
| --- | --- |
| Data unit | kg CO2e/kg |
| Description | Emission factor of purchased feed type j within system |
| Method 1 data source | Common cereals, fodder and feed types - refer to the Australian Life Cycle Inventory Database [1] based on the carbon neutrality method with infrastructure excluded Aquaculture feed types; whole sardines, squid, whole fish, low animal protein formulated feed and custom bait – see Appendix Table A.3.1.1 |
| Method 1 value | Select appropriate values based on feed type. Parameter within AusLCI CEF varies based on state/region of origin. Where the feed ingredient used is not listed in the Appendix or AusLCI CEF, select the EFj. for the feed ingredient that most closely resembles that used on-farm. |
| Method 2 data source | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 14067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. |
| Quality assurance / quality control considerations | Ensure the most recently available published data from the Australian Life Cycle Inventory Database is used. If AusLCI does not contain a suitable value and an alternative EF is used, it is advisable that users document the source of the emission factor and the justification for its selection. |

#### Constant Parameters

N/A

## Purchased mineral supplements for livestock

This module covers the estimation of embedded greenhouse gas emissions associated with purchased mineral supplements provided to livestock. The embedded emissions from mineral supplements are emissions that result from the manufacture of the products.

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| m | Type of mineral supplement |

Examples of purchased mineral supplements includes mineral and weaner blocks, and dry and wet season mixes that provide urea and phosphorus as well as pre-mixed trace element supplements.

### Estimation methodology

#### Method 1 — Purchased Mineral Supplements

- Total emissions from purchased mineral supplements E (t CO2e) are calculated as:

E=m(Qm×EFm)

Where 	Qm = total quantity of mineral supplement type m used (t)

EFm = emission factor of purchased mineral supplement (kg CO2e/kg)

#### Method 2 — Purchased Mineral Supplements

Method 2 is the same as Method 1, except that a verified supplier-provided value for EFm is required.

### Data/Parameters

#### Input Data (Required)

| Data / Parameter | Qm |
| --- | --- |
| Data unit | t |
| Description | Quantity of mineral supplement type m purchased |
| Data source | Invoices, receipts or other records of purchased quantities |
| Quality assurance / quality control considerations | Qm shall be based on the quantity of mineral supplements used within the reporting period. Where not all mineral supplement that was purchased within the reporting period was used, additional evidence may be required to justify the portion of supplement used. |

#### Data (Method 1 and 2 Options)

Question Reference 15.11.

Currently there are no scope 3 EFs for purchased minerals for poultry and swine. Are there any publicly available EFs which could be included?

| Data / Parameter | EFm |
| --- | --- |
| Data unit | kg CO2e/kg |
| Description | Emission factor of purchased mineral supplement type m |
| Method 1 data source | Refer to the latest Australian Life Cycle Inventory Database [1] based on the carbon neutrality method with infrastructure excluded |
| Method 1 value | Select appropriate value based on mineral supplement type. Where the composition of the mineral supplement is unknown or different to those provided in the AusLCI CEF, select the EFm. for the mineral supplement that most closely resembles that used on-farm. |
| Method 2 data source | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 14067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. |
| Quality assurance / quality control considerations | Ensure the most recently available published data from the Australian Life Cycle Inventory Database is used. If AusLCI does not contain a suitable value and an alternative EF is used, it is advisable that users document the source of the emission factor and the justification for its selection. |

#### Constant Parameters

N/A

## Purchased fertiliser

This module covers the estimation of embedded greenhouse gas emissions associated with purchased organic and inorganic fertilisers. The embedded emissions from purchased inorganic fertilisers are emissions that result from the manufacture, transportation and distribution of the inorganic fertiliser product. Embedded emissions from purchased organic fertiliser are emissions generated on the farm from which the organic fertiliser was purchased.

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| f | Inorganic fertiliser type f |
| c | Component (N, P, K or S) contained in the fertiliser |

### Estimation methodology

#### Method 1 — Purchased Fertiliser

- Emissions from purchased fertiliser E (t CO2e) are estimated as the sum of embedded emissions of all purchased inorganic fertilisers as:

E=fj(TMjf×EFf)×10-3

Where 	TMjf  = quantity of inorganic fertiliser type f purchased and applied to production system j (kg) (calculated in Section 5.1.1)

EFf = emission factor of purchased inorganic fertiliser (kg CO2e/kg)

Under the Method 1 approach, default values of EFf are provided for common inorganic fertilisers based on typical values of fertiliser composition found in literature [3].

Where emission factors for the purchased fertiliser are not available, the emissions may be estimated using the approach from Horticulture Australia Ltd [4], which estimates the embedded emissions in the fertiliser based on the embedded emissions of each of the individual components contained in the fertiliser.

- Emissions from purchased fertiliser E (t CO2e) are estimated as the sum of the emissions from all of its components as:

E=fcjFcf×TMjf100×EFc

Where 		Fcf = fraction of component c contained within the inorganic fertiliser f (wt%)

EFc = emission factor for the production of component (kg CO2e/kg component)

#### Method 2 — Purchased Fertiliser

Method 2 is the same as Method 1, except that a verified supplier-provided value for EFf is required.

### Data/Parameters

#### Input Data (Required)

Refer to Section 5.1.2.1 for the data and parameter table for TMjf.

| Data / Parameter | Fcf |
| --- | --- |
| Data unit | wt% |
| Description | Fraction of component c contained within the inorganic fertiliser f, by weight |
| Data source | Product information label. This data is only required where inorganic fertilisers are used for which default values of EFf are not provided in the Constant Parameters section below. Fcf for the following components should be included, where relevant: - Nitrogen (N) - Phosphoros (P) - Potassium (K) - Sulfur (S) |
| Quality assurance / quality control considerations | Fcf shall be composition fractions by weight. |

#### Data (Method 1 and 2 Options)

| Data / Parameter | EFf |
| --- | --- |
| Data unit | kg CO2e/kg |
| Description | Emission factor of purchased inorganic fertiliser type f |
| Method 1 data source | See Appendix Table A.3.1.2 |
| Method 1 value | Select appropriate value based on fertiliser type. Where a different type of inorganic fertiliser is used that those listed above, EFf shall be calculated for the fertiliser type specifically based on its composition (refer the data and parameter tables for Fcf and EFc). |
| Method 2 data source | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 14067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. |
| Quality assurance / quality control considerations | For the Method 2 approach, the emission intensity values provided by the supplier should include all emissions associated with the production, transportation and distribution of the fertiliser. It is recommended that the emission intensity of the fertiliser product has been quantified using a life-cycle emissions estimation, for example by following the GHG Protocol Product Standard. |

#### Constant Parameters

| Data / Parameter | EFc |
| --- | --- |
| Data unit | kg CO2e/kg component |
| Description | Emission factor for the production of component c contained in the fertiliser |
| Data source | See Appendix Table A.3.1.3 |
| Value | Select the appropriate value based on fertiliser component. |
| Quality assurance / quality control considerations | N/A |

## Purchased agrichemicals

This module covers the estimation of embedded greenhouse gas emissions associated with purchased herbicides, pesticides, and other agrochemicals (other than fertiliser). The embedded emissions from purchased herbicide and pesticides are emissions that result from the manufacture and distribution of the product up to the point of purchase.

The following subscripts are used in this module

| Subscript | Meaning |
| --- | --- |
| h | Agrichemical type |

### Estimation methodology

#### Method 1 — Purchased Agrichemicals

- Total emissions from purchased agrichemicals E (t CO2e) are calculated as:

E=h(Qh×EFh)×10-3

Where 	Qh = quantity of active ingredients (ai) in agrichemical type h purchased and applied to soils (kg ai)

EFh = emission factor for purchased agrichemical active ingredient (kg CO2e/kg ai)

#### Method 2 — Purchased Agrichemicals

Method 2 is the same as Method 1, except that a verified supplier-provided value for EFh is required.

### Data/Parameters

#### Input Data (Required)

| Data / Parameter | Qh |
| --- | --- |
| Data unit | kg of active ingredient |
| Description | Total quantity of active ingredients in agrichemical type h purchased and applied in the reporting period |
| Data source | Receipts, invoices, or records of quantities purchased. Farm records for quantities used or application rates specific to the reporting period. Product specification label. |
| Quality assurance / quality control considerations | Qh should be based on the quantity of agrichemicals applied during the reporting period. A combination of records of quantity purchased as well as application rates may be required to attain sufficient evidence to justify Qh. Active ingredient information should be sourced from the product specification label and used to determine Qh: Qh=ai concentration kg/L×quantity of agrichemical (e.g., L) |

#### Data (Method 1 and 2 Options)

| Data / Parameter | EFh |
| --- | --- |
| Data unit | kg CO2e/kg |
| Description | Emission factor of purchased agrichemical type |
| Method 1 data source | See Appendix Table A.3.1.4 |
| Method 1 value | Select appropriate value based on agrichemical type. |
| Method 2 data source | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 14067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. |
| Quality assurance / quality control considerations | N/A |

#### Constant Parameters

N/A

## Purchased lime

This module covers the estimation of embedded greenhouse gas emissions associated with purchased lime. The embedded emissions from lime are emissions that result from the production of lime.

### Estimation methodology

#### Method 1 — Purchased Lime

The embedded emissions for purchased lime E (t CO2e) are calculated as:

E= Mlime×EFl

Where 		Mlime = total quantity of lime applied to soils (t)

EFl = emission factor for purchased lime (t CO2e/t)

#### Method 2 — Purchased Lime

Method 2 is the same as Method 1, except that a verified supplier-provided value for EFl is required.

### Data/Parameters

#### Input Data (Required)

Refer to Section 5.3.2.1 for the data and parameter table for Mlime.

#### Data (Method 1 and 2 Options)

Question Reference 15.12.

The scope 3 EF for purchased lime is based on data from NZ and the UK, as published by Horticulture Australia Ltd in 2008. Are more recent data or Aus-specific data now available for agricultural lime production that could be used instead?

| Data / Parameter | EFl |
| --- | --- |
| Data unit | t CO2e/t |
| Description | Emission factor for purchased lime |
| Method 1 data source | Based on Horticulture Australia Ltd [4] This EF is based on studies conducted in New Zealand and the United Kingdom, as data on energy use or EFs specific to agricultural lime production in Australia are limited. |
| Method 1 value | 0.432 |
| Method 2 data source | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 14067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. |
| Quality assurance / quality control considerations | N/A |

#### Constant Parameters

N/A

## Purchased services and contractors

This module covers the estimation of greenhouse gas emissions that are generated by third-party services carrying out work on-farm or within the farm enterprise emissions boundary, such as contractors hired for spreading, spraying, or baling tasks.

The emissions associated with such third-party services are quantified based on life cycle emission factors from the Australian Life Cycle Inventory Database [1], using the carbon neutral approach without infrastructure.

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| p | Type of purchased service |

### Estimation methodology

#### Method 1 — Purchased Services and Contractors

- Emissions of purchased services and contractor operations is calculated E (t CO2e) as:

E=p(Ap× EFp)×10-3

Where	Ap= activity data of the service that was purchased (either in area of land serviced (ha) or time of operation (hours), depending on the service purchased)

EFp = life cycle emission factor for the purchased service (kgCO2e/ha or kg CO2e/hr)

Question Reference 15.13.

This emission estimation method is based on services included in the Australian Life Cycle Inventory Database Initiative Carbon Emission Factors. Are there any services that are likely to be material that are missing?

Question Reference 15.14.

Is it feasible to collect primary fuel consumption data from third-party contractors operating on-farm to improve the accuracy of Scope 3 purchased services emissions estimates?

Note, emissions from purchased goods and purchased services are treated differently in GHG estimation. Purchased goods emission factors typically reflect all upstream life-cycle emissions. Purchased services estimates generally only cover emissions from service delivery (e.g. fuel, energy, operational inputs), rather than the full lifecycle of the service provider (such as emissions associated with their capital goods and corporate overheads).

However, should the Guidelines consider additional upstream emissions associated with service provision (e.g., electricity use, embodied emissions in equipment, supply-chain inputs), beyond direct fuel use?

#### Method 2 — Purchased Services and Contractors

There is no Method 2 option for this emission source.

### Data/Parameters

#### Input Data (Required)

Question 15.15.

There are currently no unique scope 3 EFs for purchased services and contractors for aquaculture and fisheries. Are there any publicly available EF which could be included?

| Data / Parameter | Ap |
| --- | --- |
| Data unit | Depends on the purchased service: ha of land serviced or hours of operation |
| Description | The activity data of the service that was purchased, either the area of land serviced by the purchased service p or hours of operation of the purchased service p |
| Data source | Invoices, receipts, farm records |
| Quality assurance / quality control considerations | Where contractors are using fuel from an on-farm fuel tank, the emissions associated with their operation should already be captured in the Scope 1 emission quantification and does not need to be quantified separately as Ep. |

#### Data (Method 1 and 2 Options)

N/A

#### Constant Parameters

| Data / Parameter | EFp |
| --- | --- |
| Data unit | kg CO2e per activity data unit (ha or hours) |
| Description | Life cycle emission factor for the purchased service p |
| Data source | Refer to the latest Australian Life Cycle Inventory Database [1] based on the carbon neutrality method with infrastructure excluded |
| Value | Select appropriate value based on purchased service, Where the purchased service is not included in, select EF for a category that most closely resembles the purchased service in terms of typical fuel consumption during operation. |
| Quality assurance / quality control considerations | Ensure the most up to date Australian Lifecycle Inventory Database is used. |

## Purchased packaging

This module covers the estimation of embedded greenhouse gas emissions associated with packaging materials used within the defined emissions boundary, including primary, secondary, and tertiary packaging purchased for use in the production, containment, and storage of products.

The methodology includes upstream emissions associated with the manufacture and supply of packaging materials up to the point they are used by the reporting entity, including emissions from raw material extraction, processing, and conversion to packaging products.

Emissions associated with the end-of-life management of packaging following product use are excluded from this emissions estimation, as these emissions fall under Scope 3, Category 12 (End-of-life treatment of sold products) and are therefore outside the scope of this methodology.

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| p | Packaging type |

### Estimation methodology

#### Method 1 — Purchased Packaging

- The embedded emissions from use of packaging materials E (t CO2e), are calculated as:

E=p(Qp×EFp)×10-3

Where 	Qp = units of packaging type p (units)

EFp = emission factor for packaging type p (kg CO2e/unit)

#### Method 2 — Purchased Packaging

Method 2 is the same as Method 1, except that a verified supplier-provided value for EFp is required.

### Data/Parameters

#### Input Data (Required)

| Data / Parameter | Qp |
| --- | --- |
| Data unit | units |
| Description | Units of purchased packaging of type p |
| Data source | Invoices for packaging purchased, or other records |
| Quality assurance / quality control considerations | Qp should be based on the number of packaging units used during the reporting period. |

#### Data (Method 1 and 2 Options)

Question Reference 15.16.

The are currently limited scope 3 EFs for purchased packaging. Are there any other publicly available EF available for other packaging types which could be included?

| Data / Parameter | EFp |
| --- | --- |
| Data unit | kg CO2e/unit |
| Description | Emission factor of purchased packaging type |
| Method 1 data source | See Appendix Table A.3.1.6. |
| Method 1 value | Select appropriate value based on unit of packaging. |
| Method 2 data source | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 14067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. |
| Quality assurance / quality control considerations | N/A |

#### Constant Parameters

N/A

## Purchased grow media

This module covers the estimation of embedded greenhouse gas emissions associated with grow media used within the emissions boundary, including all grow media and substrates used in the production, containment, and storage of the agricultural products.

The methodology includes upstream emissions associated with the manufacture and supply of grow media materials up to the point they are used by the reporting entity.

Emissions associated with the end-of-life management of grow media following its use by the reporting entity are excluded from this emissions estimation, as these emissions fall under Scope 3, Category 12 (end-of-life treatment of sold products) and are therefore outside the scope of this methodology.

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| g | Grow media type |

### Estimation methodology

#### Method 1 – Purchased Grow Media

- The embedded emissions from use of grow media products E (t CO2e), are calculated as:

E=g(Qg×EFg)×10-3

Where 	Qg = total quantity of grow media type g (kg or m3)

EFg = emission factor for grow media type g (kg CO2e/kg or kg CO2e/m3)

#### Method 2 – Purchased Grow Media

Method 2 is the same as Method 1, except that a verified supplier-provided value for EFg is required.

### Data/Parameters

#### Input Data (Required)

| Data / Parameter | Qg |
| --- | --- |
| Data unit | kg or m3 |
| Description | Quantity of purchased grow media of type g |
| Data source | Invoices for purchased grow media, substrates, or other records. |
| Quality assurance / quality control considerations | Qg should be based on the quantity of grow media product(s) used during the reporting period. |

#### Data (Method 1 and 2 Options)

| Data / Parameter | EFg |
| --- | --- |
| Data unit | kg CO2e/kg or kg CO2e/m3 |
| Description | Emission factor of purchased grow media type |
| Method 1 data source | Australian Life Cycle Inventory Database [1] based on the carbon neutrality method with infrastructure excluded. |
| Method 1 value | Refer to the latest AusLCI CEF. Where the grow media is unknown or different to those included in the Australian Life Cycle Inventory Database, use the value for EFg. that most closely resembles the grow media product(s) used within production. |
| Method 2 data source | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 14067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. |
| Quality assurance / quality control considerations | Ensure the most recently available published data from the Australian Life Cycle Inventory Database is used. |

#### Constant Parameters

N/A

## Other purchased goods and services

This module covers the estimation of embedded greenhouse gas emissions associated with any other purchased goods that are used in the emission boundary that have not already been discussed.

The estimation methodology is based on the same approach as that used for the specified purchased goods discussed in Sections 15.1 – 15.9 as:

emissions=activity data ×emission factor

The coverage of “other purchased goods and services” should be interpreted as inclusive of all other purchased goods and services not already outlined in Chapter 15. However, as it is neither practical nor proportionate to develop item-specific estimates for all minor inputs, it is recommended that inclusion of emissions estimations of other purchased goods/services is based on materiality. Refer to Chapter 1, Section 1.3 for more information on coverage and limitations of the Guidance, and refer to the Common Requirements Framework, Chapter 5 for further details on boundary setting and materiality.

Geographically and technologically appropriate emission factors should be applied and transparently documented, including key assumptions and limitations. If using supplier-provided life-cycle emission factors, these should conform with the same guidance as for previously discussed Method 2 approaches: requiring third-party verification as conforming either to ISO 14067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard.

If a purchased good has been identified as material, but it is not covered within the Guidance, and supplier-provided life-cycle emission factors are unavailable, the following options could be applied:

- hybrid method, using supplier-provided Scope 1 and Scope 2 emissions data and using secondary data to estimate supplier’s Scope 3 emissions data;

- industry-average life-cycle emissions (e.g., from regional, national, or global LCA databases) based on per unit of mass or unit of product purchased; or

- industry-average life-cycle emission factors (e.g., from regional, national, or global LCA databases) based on per unit of economic value (also known as environmentally extended input–output (EEIO)

In all cases, assumptions, data sources, representativeness and uncertainty implications should be transparently disclosed, and exclusions or data gaps should be justified on the basis of immateriality or lack of data availability, with a commitment to refine estimates in future reporting cycles as improved data become accessible.

## Upstream emissions from fuel

This module covers the estimation of embedded greenhouse gas emissions associated with scope 1 fuel consumption. The upstream emissions from fuel consumption quantify the emissions generated in the extraction, refining, and transportation of the fuel to the point of purchase. The emission quantification approach is based on that of the National Greenhouse Accounts Factors (NGAF) [5].

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| q | Fuel type |

### Estimation methodology

#### Method 1 — Upstream Fuel

- Total upstream emissions from scope 1 fuel consumption E (t CO2e) are calculated as the sum of upstream emissions from scope 1 fuel used for transport purposes (EWTT,trans,q) and upstream emissions from scope 1 fuel used for stationary combustion purposes (EWTT,SC,q) as:

E=q(EWTT,trans,q)+q(EWTT,SC,q)

- In equation (1) upstream emissions from Scope 1 fuel use for transport purposes EWTT,trans,q for fuel q is calculated as:

EWTT,trans,q=QTrans,q× ECTrans,q× EFTrans,3,q × 10-3

Where 	QTrans,q = amount of fuel q combusted for transport energy purposes (kL) (same as that used in Section 8.1)

ECTrans,q = energy content factor for transport energy purposes (joules/litre or m3) (same as that used in Section 8.1)

EFTrans,3,q = Scope 3 emission factor for transport energy purposes (kg CO2e /GJ)

- In equation (1) upstream emissions from Scope 1 stationary combustion fuel use EWTT,SC,q for fuel q is calculated as:

EWTT,SC,q=QSC,q× ECSC,q× EFSC,3,q× 10-3

Where 	QSC,q = amount of fuel used for stationary combustion operations (kL) (same as that used in Section 8.2)

ECSC,q = energy content factor for stationary combustion operations (GJ/kL) (same as that used in Section 8.2)

EFSC,3,q = Scope 3 emission factor for stationary combustion operations (kg CO2e /GJ)

#### Method 2 — Upstream Fuel

There is no Method 2 option for this emission source.

### Data/Parameters

#### Input Data (Required)

Refer to the data and parameter tables in Sections 8.1.2.1 and 8.2.2.1 respectively for QTrans,q and QSC,q.

#### Data (Method 1 and 2 Options)

Not applicable

#### Constant Parameters

Refer to the data and parameter tables in Sections 8.1.2.3 and 8.2.2.3 respectively for ECTrans,q and ECSC,q.

| Data / Parameter | EFSC,3,q |
| --- | --- |
| Data unit | kg CO2e/GJ |
| Description | Emission factor for Scope 3 emissions of fuel q for stationary combustion purposes |
| Data source | Australian National Greenhouse Accounts Factors 2024, Table 8 [5] |
| Value | Refer to the most recently published Australian National Greenhouse Account Factors. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used. |

| Data / Parameter | EFTrans,3,q |
| --- | --- |
| Data unit | kg CO2e/GJ |
| Description | Emission factor for Scope 3 emissions of fuel q for transport energy purposes |
| Data source | Australian National Greenhouse Accounts Factors Table 9 [5] |
| Value | Refer to the most recently published Australian National Greenhouse Account Factors. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used. |

## Upstream emissions from purchased electricity

This module covers the estimation of embedded greenhouse gas emissions associated with purchased electricity. Upstream emissions in the electricity supply chain include emissions associated with the generation of electricity, such as raw material transportation to the power station, as well as transmission and distribution losses through the electricity network.

Scope 3 emissions from purchased electricity shall be estimated using the location-based method. Where the market-based method is also used to calculate scope 2 the scope 3 emissions shall also be estimated using the market-based method.

### Estimation methodology

#### Purchased electricity upstream emissions (location-based)

- Scope 3 emissions from purchased electricity, E3,elec (t CO2e), under the location-based approach are estimated as:

E3,elec=Qelec×EF3,elec × 10-3

Where	Qelec = amount of electricity purchased from the grid (kWh) (same as that used in Section 14.1)

EF3,elec = Scope 3 emission factor for purchased electricity (kg CO2e/kWh)

#### Purchased electricity upstream emissions (market-based)

- Scope 3 emissions from purchased electricity, E3,elec (t CO2e), under the market-based approach are estimated as:

E3,elec=Qelec×1-(RPP+JRPP)-(RECsurrendered-REConsite ×10-3) × EFRMF3,elec ×10-3

Where	Qelec = amount of electricity purchased from the grid (kWh) (same as that used in Section 14.1)

EFRMF3,elec = Scope 3 residual mix emission factor for electricity (kg CO2e/kWh)

RPP = Renewable Power Percentage under the Large-scale
Renewable Energy Target (LRET) for the applicable period (per cent)

JRPP = jurisdictional renewable power percentage for the applicable period (only applies to the ACT) (per cent)

RECsurrendered = number of eligible Renewable Energy Certificates voluntarily surrendered in the reporting year equivalent to megawatt hours (MWh)

REConsite = number eligible Renewable Energy Certificates that have been or will be issued for electricity produced on-site during the year and consumed by the entity equivalent to megawatt hours (MWh)

If an entity has a liability under the RET scheme and has been given exemption, they should estimate the market-based emissions using the non-adapted approach in the National Greenhouse Accounts Factors (NGAF).

### Data/Parameters

#### Input Data (Required)

Refer to Section 14.2 for the data and parameter table for Qelec.

#### Data (Method 1 and 2 Options)

Refer to Section 14.2 for the data and parameter table for: Qelec, RECsurrendered, and REConsite.

#### Constant Parameters

Refer to Section 14.2 for the data and parameter table for RPP and JRPP (applicable to the market-based approach).

| Data / Parameter | EF3,elec |
| --- | --- |
| Data unit | kg CO2e /kWh |
| Description | Emission factor for Scope 3 emissions of purchased electricity Applicable only to the location-based approach. |
| Data source | Australian National Greenhouse Accounts Factors Table 1 |
| Value | Select appropriate value based on State, Territory or grid description. |
| Quality assurance / quality control considerations | The NGAF publish EF3,elec values annually, ensure that the emission factor is selected based on the year and State in which the grid is located to which the farming enterprise is purchasing its electricity from. The emission factors are based on financial years (e.g. 2025 NGA Factors report provide EF for 2025-26). If an entity is required to report on a calendar year basis, they should collate electricity use data per 6-month period and multiply each total by the emission factor for that financial year, as published in the NGA Factors reports. If only annual figures are available, entities should apply an emissions factor that is the average for the two financial years covered by the calendar year. |

| Data / Parameter | EFRMF3,elec |
| --- | --- |
| Data unit | kg CO2e /kWh |
| Description | Residual mix emission factor for Scope 3 emissions of purchased electricity Applicable only to the market-based approach. |
| Data source | Australian National Greenhouse Accounts Factors Table 2 |
| Value | Select appropriate value based on residual mix factor. |
| Quality assurance / quality control considerations | The NGAF publish EFRMF3,elec values annually, ensure that the emission factor is selected based on the year and State in which the grid is located to which the farming enterprise is purchasing its electricity from. The emission factors are based on financial years (e.g. 2025 NGA Factors report provide EF for 2025-26). If an entity is required to report on a calendar year basis, they should collate electricity use data per 6-month period and multiply each total by the emission factor for that financial year, as published in the NGA Factors reports. If only annual figures are available, entities should apply an emissions factor that is the average for the two financial years covered by the calendar year. |

## Management of waste

This module covers the estimation of indirect greenhouse gas emissions that result from the disposal of waste generated in operations outside the entity boundary. This includes:

- livestock manure that is sent off-site and applied to soils that are located outside of the reporting entity’s organisational boundary (i.e., on lands owned by a different entity); and/or

- waste generated within the operations of the reporting entity that is sent to waste treatment facilities outside of the reporting entity’s organisational boundary (i.e., owned by a different entity).

### Estimation methodology

#### Method 1 – Emissions from Manure Sent Off-Site

Refer to Chapter 10.1 for estimation methodology from waste management

- Emissions from manure from manure management systems that is sent off-site to be applied to soils in another farming system or enterprise, E (t N2O) is calculated as the sum of the direct nitrous oxide emissions from application of the manure to soil and indirect emissions from atmospheric deposition and leaching and runoff as:

E=EN2O+Ead+Eleach

Where	EN2O = direct nitrous oxide emissions from application of manure to soils outside of the Scope 1 boundary (t N2O)

Ead = nitrous oxide emissions from atmospheric deposition of manure to soils (t N2O)

Eleach = nitrous oxide emissions from nitrogen leaching and runoff of manure to soils (t N2O)

Refer to Chapter 5 equation 5.2.1.1 (1) for the calculation of EN2O and replace MNSoiljf for the value of MNSoilscope3, the mass of nitrogen applied to soils outside of the boundary, as calculated in Chapter 4.

Refer to Chapter 5 equations 5.4.1.3 (1) and (2) for the calculation of Ead and replace MNSoiljf in equation 5.4.1.3 (2) for the value of MNSoilscope3.

Refer to Chapter 5 equation 5.5.1.1 (1) and (3) for the calculation of Eleach, set Mleach,inorganic,j=0, and replace MNSoiljf in equation 5.5.1.1 (3) for the value of MNSoilscope3.

#### Method 2 – Emissions from Manure Sent Off-Site

There is no Method 2 option for this emission source.

#### Method 1 – Emissions from Waste Treatment Facility

Refer to Chapter 10.1 for the estimation methodology from waste management. The same emission estimation approach applies, except the emissions are categorised as Scope 3 rather than Scope 1, since the disposal of waste is occurring outside of the organisational boundary.

#### Method 2 – Emissions from Waste Treatment Facility

There is no Method 2 option for this emission source.

### Data/Parameters

#### Input Data (Required)

Refer to the data and parameter tables in Chapter 5, Section 5.5.1.1 for input data required to calculate MNSoilscope3, for the estimation of emissions from manure sent off-site.

Refer to the data and parameter tables in Chapter 10, Section 10.2.1 for parameters used in the estimation of Qwt and Vwt respectively, for the estimation of emissions from waste treatment.

#### Data (Method 1 and 2 Options)

N/A

#### Constant Parameters

Refer to the data and parameter tables in Chapters 5.2.2.3, 5.4.2.3, and 5.5.2.3 for parameters used in the estimation of EN2O, Ead, and Eleach respectively, for the estimation of emissions from manure sent off-site.

Refer to the data and parameter tables in Chapter 10, Section 10.2.1 for parameters used in the estimation of EFwt and CFw respectively, for the estimation of emissions from waste treatment.

## Upstream freight

This module covers the estimation of embedded greenhouse gas emissions associated with third-party freight services for the inbound transportation of purchased goods and the transportation of products to the entity’s processing or storage facilities. Transportation activities occurring beyond this point – such as distribution to customers or final markets – are considered downstream under the cradle-to-gate boundary and are not included within the scope of this module.

Where an entity is applying a cradle-to-first-point-of-sale boundary, this method may be used to estimate the upstream third-party outbound transportation of products to the customer where ownership or control of products remains with the entity.

Refer to Chapter 1, Section 1.3 for additional information on the coverage and limitations of the guidance.

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| f | Freight type |

### Estimation methodology

#### Method 1 — Upstream Freight

- The embedded emissions from freight E (t CO2e), are calculated as:

E=f((Wf×Df)×EFf)

Where 		Wf = total mass of goods transported per freight type f (t)

Df = total distance transported per freight type f (km)

EFf = emission factor for freight type f (t CO2e/t km)

#### Method 2 — Upstream Freight

If fuel use is known, use Chapter 8.1.1.1 and 15.8.1.1 to estimate embedded greenhouse gas emissions associated with fuel consumption.

### Data/Parameters

#### Input Data (Required)

| Data / Parameter | Wf |
| --- | --- |
| Data unit | tonnes |
| Description | Mass of goods transported per freight type f |
| Data source | Invoices for freight, weighbridge receipts or other farm records. |
| Quality assurance / quality control considerations | Wf should be based on the mass of products transported using external freight during the reporting period. Any product transportation captured under Scope 1 fuel use should not be included in this quantity. |

| Data / Parameter | Df |
| --- | --- |
| Data unit | km |
| Description | Total distance transported per freight type f |
| Data source | Data from supplier, or other farm records. |
| Quality assurance / quality control considerations | Df should be based on the distance travelled by product using external freight during the reporting period. Any product transportation captured under Scope 1 fuel use should not be included in this quantity. |

#### Data (Method 1 and 2 Options)

Refer to the data and parameter tables in Chapter 8.1.1.1 and 15.8.1.1 respectively for QTrans,q, ECTrans,q and EFTrans,3,q.

#### Constant Parameters

| Data / Parameter | EFf |
| --- | --- |
| Data unit | t CO2e/tkm |
| Description | Emission factor for freight type f |
| Data source | Refer to the latest Australian Life Cycle Inventory Database [1] based on the carbon neutrality method with infrastructure excluded. |
| Value | Select appropriate value based on freight type. |
| Quality assurance / quality control considerations | Freight type chosen should as closely as possible reflect freight type used. Invoices for external freight may be used to verify freight type. |

# References

[1]	ALCAS, “AusLCI Emissions Factors,” Australian Life Cycle Assessment Society. Accessed: Aug. 18, 2025. [Online]. Available: https://www.auslci.com.au/index.php/EmissionFactors

[2]	Department of Climate Change, Energy, the Environment and Water, “National Inventory Report 2023, Volume I.” Australian Government, 2023. [Online]. Available: https://www.dcceew.gov.au/sites/default/files/documents/national-inventory-report-2023-volume-1.pdf

[3]	S. Wood and A. Cowie, “A Review of Greenhouse Gas Emission Factors for Fertiliser Production,” Cooperative Research Centre for Greenhouse Accounting, Research and Development Division, State Forests of New South Wales., For IEA Bioenergy Task 38, 2004.

[4]	N. J. O’Halloran, P. D. Fisher, and M. A. Rab, “Vegetable Industry Carbon Footprint Scoping Study Preliminary Estimation of the Carbon Footprint of the Australian  Vegetable  Industry,” Horticulture Australia Ltd, Sydney, Discussion Paper 4, 2008.

[5]	Department of Climate Change, Energy, the Environment and Water, “Australian National Greenhouse Accounts Factors,” 2024.

The GHG Protocol Technical Guidance for Calculating Scope 3 Emissions Chapter 1 should be referred to for more guidance on methods for calculating emissions from purchased goods and services.