## TERMS OF REFERENCE

## Selection of Management Service Provider for Maintenance and Operation of “Thimaaveshi 2” Speedboat -

## Project Code:  P160739

## BACKGROUND

Ministry of Environment (ME) under Maldives Clean Environment Project (MCEP) is seeking to hire a qualified and competent Management Service Provider for Operation & Maintenance of “Thimaaveshi 2” Speedboat. The Ministry of Environment now invites eligible companies to submit their “Expression of Interest” (EOI) for providing the services. The bidder should provide information demonstrating that they fulfil the required qualification criteria and relevant experience to perform the services.

## OBJECTIVES

The main objective is to hire a qualified and competent Management Service Provider for Operation & Maintenance of “Thimaaveshi 2” Speedboat. The Service Provider shall provide all the required service for the utilisation of the speedboat as requested by the PMU, Maldives Clean Environment Project (MCEP). The operations, repair and maintenance of the vessel will be contracted for a duration of 12 calendar months, commencing November 2020.

## SCOPE OF WORKS & GENERAL GUIDLINES

Based on the objectives above, the detailed scope of services and requirements for the operations and maintenance of the speedboat are as follows:

- Vessel will be mainly docked in Kudahuvadhoo, Dhaalu Atoll or in a main island in one of the following atolls; Meemu, Faafu, Dhaalu, Thaa and Laamu. This will be decided by the Client.

- As per the general guideline, speed boat operations will be restricted from 18:30 – 05:00 hrs every day. However, if Project Management Unit (PMU) of Maldives Clean Environment Project (MCEP) requests to travel during the mentioned time, travel should be arranged after confirmation safety aspects from the captain.

- Vessel should only be used for the purpose of the PMU of MCEP or any other party that is permitted by PMU.

- Fuel for the operations will be arranged by PMU. The Service Provider is supposed to keep records of fuel usage (fuel logs).

- Operations, repair and maintenance of the vessel should be in compliance with National Maritime Legislations in the Maldives and should comply with other related laws and regulations.

- Travels requested by PMU should be arranged accordingly (time of the travel and other requirements) without any hindrance to the work schedule of the PMU unless there is a valid reason (which has to be communicated to PMU in advance).

- If any damage is caused to the boat or any belongings (which is included in the inventory of the vessel) due to irresponsibility of the captain and crew, the cost of recovery/maintenance should be borne by the Service Provider.

- Boat should not be moved/transferred without prior permission of the PMU. Similarly, the boat should not be used for travel that is not approved by PMU. For approval of trips, PMU will devise a system and appoint a focal point for all such communications.

- Operations of the vessel should be regularly logged using the format provided by PMU.

- No single use plastics should be allowed onboard the vessel (bottles, bags, straws etc…).

- Vessel crew should make sure, there is no ocean dumping/ littering of waste at all times.

- All crew and passengers should wear life jackets during the entire voyage.

- Fire extinguishers at the speedboat should be regularly serviced and maintained.

- When the boat is docked in a harbor (during trips) the Service Provider should make sure there is at least 1 crew member onboard looking after the boat and equipment, at all times.

- Captain hired for operation should be a licensed boat captain category A and B in par with the Maldives Transport Authority (MTA) regulations (2015/R-229). And he/she should be experienced and familiar with the travel routes within the Maldives, specially the 5 atolls of Zone IV and V (Meemu, Faafu, Dhaalu, Thaa and Laamu Atolls). In addition to this, license and relevant documents of the captain (selected) should be submitted for approval of the PMU. If the hired captain has to take a leave or resigns, approval of the PMU should be acquired (with the relevant documents) prior to recruiting a new captain. In such cases, the Service Provider is required to provide a substitute with the same qualification and experience proposed by the Service Provider.

- Two crew members (excluding the Captain) should be present during all the trips. Therefore, details of captain and crew members should be submitted during bid submission (with police records and any other relevant documents showing their qualification and experience).

- Hull, engine and equipment of the boat should be regularly serviced (routine maintenance) as per the requirement/table provided by PMU. Regular records should be maintained for all maintenance works. The Service Provider should cover the cost of routine maintenance for all the mentioned works mentioned under 3.1 of Periodic Maintenance schedule.

### 3.1 Periodic Maintenance Schedule:

The "” symbol indicates the time of action to be carry out.

| Item | Actions | Every _ hrs |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50hrs | 100hrs | 300hrs | 600hrs |
| Anode(s) External | Inspect, replace as necessary Replace every six months otherwise |  |  |  |  |
| Anodes(s) Internal (accessible with minor disassembly) | Inspect, replace as necessary Replace every six months otherwise |  |  |  |  |
| Anodes (s) Internal (accessible with major disassembly) | Replace | Replace during major service procedures |  |  |  |
| Battery | Inspect, fill & charge or replace as necessary |  |  |  |  |
| Fuel, oil & cooling water leakage | Inspect, repair if necessary |  |  |  |  |
| Cowling clamp | Inspect, adjust as necessary |  |  |  |  |
| Engine starting condition/abnormal noise | Inspect |  |  |  |  |
| Engine idling speed/abnormal noise | Inspect |  |  |  |  |
| Engine oil ** | Replace |  |  |  |  |
| Engine oil filter/cartridge ** | Replace |  |  |  |  |
| Primary fuel filter/water separator | Inspect or replace as necessary |  |  |  |  |
| Intermediate fuel filters *** | Inspect or replace as necessary |  |  |  |  |
| Boat-mounted 10-Micron fuel filter | Replace |  Every 100 hrs maximum |  |  |  |
| Fuel filter (vapor separator tank) *** | Replace |  |  |  |  |
| Fuel pumps (diaphragm style) | Inspect or replace as necessary |  |  |  |  |
| Fuel lines, fuel tank to engine | Inspect or replace as necessary |  |  |  |  |
| Fuel lines, engine | Inspect or replace as necessary |  |  |  |  |
| Gear oil | Replacement |  |  |  |  |
| Grease Points | Grease |  |  |  |  |
| Impeller & water pump housing | Inspect or replace as necessary |  |  |  |  |
| Impeller & water pump housing | Replace |  |  |  |  |
| Oil control valves (OCV), filter | Inspect & clean |  |  |  |  |
| Power trim/tilt unit | Inspection |  |  |  |  |
| Propeller, propeller nut & cotter pin | Inspect or replace as necessary |  |  |  |  |
| Pressure control valve (PCV), cooling system | Inspect or replace as necessary |  |  |  |  |
| Spark plugs | Inspect, adjust or replace as necessary Replace every six months otherwise |  |  |  |  |
| Spark plug caps & leads | Inspect or replace as necessary |  |  |  |  |
| Pilot Water Indicator | Inspect |  |  |  |  |
| Thermostat(s) | Inspect or replace as necessary |  |  |  |  |
| Timing Belt | Inspect or replace as necessary |  |  |  |  |
| Valve clearance | Inspect & adjust |  |  |  |  |
| Water inlet | Inspect |  |  |  |  |
| Main ignition switch & stop switch | Inspect or replace as necessary |  |  |  |  |
| Wire harness connections | Inspect or replace as necessary |  |  |  |  |
| (Yamaha) gauges | Inspection |  |  |  |  |
| Throttle linkage & throttle cable | Inspect, adjust as necessary |  |  |  |  |
| Shift linkage & shift cable | Inspect, adjust as necessary |  |  |  |  |

PERIODIC MAINTENANCE SCHEDULE OF ENGINES

| Item | Actions | Every 1000 hours |
| --- | --- | --- |
| Exhaust guide & manifold | Inspect or replace as necessary |  |
| Timing belt | Replace |  |
| Timing belt tensioner | Inspect or replace as necessary |  |

Pre- departure Daily Checks

| Pre-start |  |  | Safety equipment |  |  | Anchoring equipment |  |  | Safety briefing |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Propulsion |  |  | Steering |  |  | Trawl equipment |  |  | Navigation equipment |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| GPS equipment |  |  | Passenger no |  |  | Navigation/trawl lights |  |  | Comm/alarm systems |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Running Log

| Time/watch-keeper | Port or location, type of activity, vessel use recreation, commercial | Details (activities of the vessel/position of the vessel/navigation track/weather experienced/ illness or injury of persons on board/safety training undertaken/recording of marine incidents/ assistance to other vessels/unusual occurrences/records of communication in relation to an emergency/arrival — departure/refuelling — bunkering/bilge levels/radio communications/ description of activities and incidents/other comments/passenger numbers |
| --- | --- | --- |

Engine Running Log

| Time | RPM |  | Water temp | Engine oil level & temp | Engine oil press | Gearbox oil level & temp | Gearbox oil press | Pyrometers | Battery volts | Aux battery volts | Radio battery volts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P |  |  |  |  |  |  |  |  |  |  |
|  | S |  |  |  |  |  |  |  |  |  |  |
|  | C |  |  |  |  |  |  |  |  |  |  |

| Fuel | Initial load | Start level | Engine hours |  |
| --- | --- | --- | --- | --- |
|  | Bunkered | End level | Port start | Generator 1 start |
|  | Total load | Liters used | Port finish | Generator 1 finish |
| Nautical miles travelled / Hours travelled |  |  | Starboard start | Generator 2 start |
| Maintenance/defect list |  |  | Starboard finish | Generator 2 finish |
| Defect Rectified (time/date) |  |  | Center start | Auxiliary 1 start |
| Parts serviced/replaced |  |  | Center finish | Auxiliary 2 finish |

|  |  | Incident Report/s |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Marine | Medical |  |  | Pollution | Other event |

* NOTE: The service intervals were developed based upon “typical” use that includes operating at varied speeds, sufficient time for the engine warm up and cool down, medium to light loads and average cruising speed near 3000 ~ 4000 rpm range. As with any engine, if the user's normal operating conditions are different, more frequent service should be considered, especially engine oil and gear oil. Examples might include: extended Wide Open Throttle use or long periods of idling or trolling, carrying heavy loads and frequent starting and stopping or shifting. Changing fuel formulations could require more frequent fuel filter changes and fuel system maintenance.

** NOTE: Recommended service provider only oil and oil filter changes.

*** NOTE: Installations without a boat-mounted 10-micron filter may require more frequent changing.

*** NOTE: For new and overhauled engines.

Vessel should be regularly cleaned including inside and outside of the hull, both engines, all the parts/equipment of the vessel, at the expense of the service provider, as follows:

| # | Activity | Frequency |
| --- | --- | --- |
| 1 | Outside of the hull and engines should be cleaned to get rid of biological fouling | After each trip and in every two weeks. |
| 2 | Cleaning of seats and interior of the boat | Once a week and also before and after each trip |
| 3 | Cleaning of internal and front, topside (using water) and wiping (interior) of all the fiberglass surfaces. | Daily |
| 5 | Deep cleaning of the vessel including the hull and all parts with polishing and compounding. | Once every month |
| 6 | Both engines cover washed with clean water | Once a week and after every trip |

*The condition of the vessel, machinery and its parts/equipment should be well maintained by the Service Provider (should be maintained in the same condition when the vessel was handed over by the PMU)

## ELIGIBILITY CRITERIA

Bidder’s proposed Captain (Number of people required; 1):

- Must have sea vessel operating license (Captain license) of category A and B in par with the Maldives Transport Authority (MTA) regulations (2015/R-229) with minimum of 2 years’ experience as a boat captain.

- Must be experienced and familiar with the travel routes within the Maldives.

- Must have a clear police record. (The Police report/certificate must be issued not prior than 20 days from the date of bid submission)

Bidder’s proposed Crew (Number of persons required; 2):

- Minimum one-year work experience as a crew in a vessel/speedboat or in the relevant field.

- Must have a clear police record. (The Police report/certificate must be issued not prior than 20 days from the date of bid submission)

Note: Bidder shall furnish documentary evidence to support the above-mentioned qualifications/criteria(s) under section 4 such as copy of license, CVs, reference/experience letters etc

## EVALUATION CRITERIA

The EOI’s submitted will be evaluated based on the criteria below.

| Specific Experience of the Bidder (company, firm, sole proprietorship etc.) | Pass/Fail |
| --- | --- |
| Minimum Requirement: Bidder shall furnish documentary evidence for specific experience of minimum one year on vessel/speedboat operations and maintenance during the past Five (5) years. (Bidder Shall meet the minimum requirement in order to be eligible for this criteria) For required documentary evidence, refer to section 8 For required documentary evidence, refer to section 9. |  |
| Financial Capability Bidder shall furnish documentary evidence that the bidder has performed 5 projects with total combined value of such works/services not less than MVR 300,000 during the past three (3) years. (Bidder Shall meet the minimum requirement in order to be eligible for this criteria) For required documentary evidence, refer to section 8 |  |

Award of the contract;

Project will be awarded to the substantially responsive lowest bidder.

## REPORTING

The successful Service Provider will report to Project Manager or an alternate nominated by the Project Manager.

## CONTRACT DURATION

The duration of the contract is 12 calendar months.

## PROPOSAL

## Proposal submission shall consist the following documents:

Documents of the proposed captain and Crews

- Copy of National Identity Card of Captain and Crew

- Copy of captain’s license

- Cvs of Captain and Crews (separate CV for each proposed candidate)

- Police reports of Captain and crew (The Police report/certificate must be issued not prior than 20 days from the date of bid submission)

- Work experience reference letters of the Captain

Documents of the bidder

- Copy of the business registration and business profile

- Work/project completion certificates or letters. (The value of the contract/project and the duration of the work shall be specified in the letter)

- Tax clearance report of the business by Maldives Inland Revenue Authority. (Tax Clearance report/certificate must be issued not prior than 20 days from the date of bid submission)

### Following cost breakdown should be provided by the Bidder:

| Details | Monthly rate | Total for 1 year |
| --- | --- | --- |
| Staff |  |  |
| Salary of licensed Captain (1) |  |  |
| Salary 2 Crew Members |  |  |
| Service allowance for crew and captain (to cover food and incidental expenses during travel) |  |  |
| Services |  |  |
| Cleaning (detail of the total cost should be given) |  |  |
| Security |  |  |
| Routine maintenance (details of the cost should be given) |  |  |
| Administrative costs (details of the cost should be given) |  |  |
| Water for trips (plastic bottles not allowed onboard) |  |  |
| Any other costs (details of the cost should be given) |  |  |
| Total price |  |  |