Low temperature UHV

Equipment: Low temperature UHV AFM/STM  (Createc)

No. of Equipment: UPOL8

Responsible coordinator: Prof. RNDr. Radek Zbořil, Ph.D.

Name of Institution: Palacký University Olomouc, Regional Centre of Advanced Technologies and Materials

Address of Institution: Šlechtitelů 27, 78371 Olomouc, Czech Republic

E-mail: rcptm.services@upol.cz

Telephone: (+420) 58 563 4973

Homepage: www.rcptm.com

Contact person: Mgr.Klára Čépe, Ph.D.

E-mail: klara.cepe@upol.cz

Telephone: +420 58 563 1429

Equipment Description

Description of equipment:

Set of Ultra High Vacuum chambers with Cryostat and Scanning Tunneling Microscope/Atomic Force Microscope for surface analysis.

Specifications and technical features:

Sample/ substrate heating control

Ultra-high vacuum environment

Sample deposition

Simultaneous measurement of force and tunneling current at 5K with a Q+ sensor

Preparation chamber.

cleaning of crystal substrates in situ by standard sputter (by an ion gun) and/or annealing (e-beam heater up to 1500oC)

controlled evaporation of molecules from a molecular beam epitaxy cell

precision dosing system of gases

residual gas analysis (by a mass spectrometer)

a hybrid UHV system pumped by turbomolecular, ion sorption, titanium sublimation pumps

SPM chamber.

submolecular and atomic resolution on atomically clean substrates and submonolayer films prepared in-situ

advanced characterization of molecules and single atoms by STM, AFM spectroscopies, e.g. Kelvin probe spectroscopy, scanning tunneling spectroscopy or atomic force spectroscopy

functionalization of the probes by probe particles for improved resolution on the investigated molecular species

possibility to incorporate an optical system for sample excitation by UV light

Cryostat.

Liquid Nitrogen/Liquid Helium cryostat for measurements below 5K

nitrogen and helium hold time 60h

Specification of expertise relevant to NanoEnviCz workpackages:

WP3a,c,e,f,h, WP4a, WP6d,f, WP7a,b,h,

Detailed description of expertise

Please, specify the main research topics connected with equipment:

On-surface chemistry

two-dimensional heat-induced on-surface chemical reactions, assisted by the substrate catalytic properties

Reactivity of the molecules and atoms

evaluation of the electrostatic potential from the distortions in images taken by functionalized tips

probing the energetic potential landscape in three dimensions above the adsorbed species

Keywords describing research area:

molecules, on-surface chemistry, electrostatic potential, two-dimensional systems, self-assembly, scanning probe microscopy, low-temperature

Competence

Relevance for applied and industrial research:

Analysis of surfaces and thin films at atomic scale.

Relevance for fundamental studies:

Synthesis of of new self-assembled molecular layers with custom structure and functionality.

Analysis of new on-surface catalysed and uncatalysed reactions of molecules and their products.

Deep analysis of the molecule-substrate and molecule-molecule interactions, characterization of the molecule electronic, structural and chemical properties, assesment of the functional group action, investigation of quantum-mechanical nature of the single atom/molecular adsorbates.