Note: Content of this document may not reflect the most current information.  Periodic reviews for potential revisions of the document may be done at the request of the NPCC Board of Directors.  Please send corrections or revision requests to npccstandard@npcc.org .

Contributors:

| Guy V. Zito | RSC Chair, NPCC Staff |
| --- | --- |
| Gerard Dunbar | NPCC Staff |
| Ruida Shu | NPCC Staff |
| Michael Jones | RCC and RSC Co-Vice Chair, National Grid |
| Herb Schrayshuen | Power Advisors, LLC |
| NPCC Task Forces | N/A |

NPCC also thanks the Regional Standards Committee (RSC) and the State and Provincial regulatory authorities who provided guidance and information during the development of this whitepaper.

Revision History

| Version | Date | Action | Change Tracking (New, Errata or Revisions) |
| --- | --- | --- | --- |
| 1 | XX/XX/21 | Effective Date | New |

Contents

Objective	5

NPCC Reliability Directories	6

Directory # 1 - Design and Operation of the Bulk Power System	7

Directory # 2 - Emergency Operations	8

Directory # 4 - System Protection Criteria	9

Directory # 5 - Reserve	10

Directory # 6 - Reserve Sharing Groups	11

Directory # 7 - Remedial Action Schemes	11

Directory # 8 - System Restoration	12

Directory # 11 - Disturbance Monitoring Equipment Criteria	12

Directory # 12 - Automatic UFLS Program Requirements	12

Directory Prioritization Recommendation	13

Appendix A- Future Directory Review Considerations	14

Appendix B- Decarbonization Goals within NPCC	15

Executive Summary

The development of this document was initiated by the NPCC Board of Directors to provide support, guidance, and information for use by NPCC Members and stakeholders. This whitepaper identifies recommended considerations and prioritization to manage the revision of NPCC Criteria located within each of the NPCC Reliability Directories . These revisions may be necessary because of increasing amounts of decarbonized Distributed Energy Resources (“DER”), and Variable Energy Resources (“VER”) being deployed or planned to be deployed due to de-carbonization goals within the NPCC Region.

Currently, within the NPCC region there are numerous state and provincial regulatory targets to achieve decarbonization goals. (See Appendix B). Additionally, several national laboratories and research organizations including EPRI, NYSERDA, and the National Renewable Energy Laboratory are actively considering issues surrounding decarbonization.

As DER and VER continue to replace traditional industry generation resources the resource fuel mix and operational characteristics of the system will fundamentally change and necessitate changes to how the system is planned and operated.

Simulating the usual cases and contingencies in transmission planning and operations will no longer be adequate to capture the impact created by DER and these functions (planning and operations ) will need to reinvent themselves to address new issues such as ramping and inertia. Other concerns such as data collection by Distribution Providers, transmission protection philosophy and study tools employed for analysis will also need to be addressed.

Additionally, the North American Electric Reliability Corporation (NERC) Reliability Standards are not applicable to equipment on the distribution systems unless such equipment has a direct impact on the “reliable operation” of the BPS, such as Automatic Underfrequency Load Shedding (UFLS). However, as penetration of DER and VER increases, planning and operating assessments used to assure the reliable operation of the BPS will need to accurately represent how DER and VER, interact with the BPS and NPCC’s Criteria contained the NPCC Reliability Directories may need to be revised or augmented.

NPCC recognizes that continent-wide efforts in North America are underway at the NERC level to define DER and VER and to address some aspects related to planning and modeling.  The NERC LTRA (Long Term Reliability Assessment) which is developed annually by NERC, recognizes the unprecedented transformational changes underway in the electricity sector and provides a summary of current efforts underway to address these emerging risks. There are specific reliability activities related to DER being conducted by the NERC System Planning Impacts from Distributed Energy Resources Working Group (“SPIDERWG”), and Inverter-Based Resource and Performance Working Group (“IRPWG”).

NPCC Staff and its members have been actively participating in these continent-wide efforts and are contributing to the work underway to address outstanding issues within the scope of those groups.  With the understanding of current ongoing efforts,  NPCC can fill an important role by identifying additional areas where the Region may provide information and services and revise the existing NPCC Criteria to promote the reliable deployment of DER and VER. Such opportunities exist in the areas of data acquisition, modeling, testing and verification, observability, protection systems and other operational characteristics of DER and VER.

Although the purpose of this NPCC whitepaper is not to create new Criteria or Standards, it is recognized that the review of the Directories may result in a need to add Criteria such as for energy sufficiency. The intent of this document is to utilize and analyze information to make recommendations on prioritizing reviews of NPCC criteria for potential DER impact.  Concurrent with NPCC’s revisions of the Reliability Directories, NERC’s SPIDERWG and other groups are developing guidance documents and NPCC will continue to consider NERC guidance documents during the Directory review process to achieve continued alignment and avoid duplication.

This whitepaper contains a prioritization of Directory reviews and categorizes them (low medium or high) using the input on the potential impact of the DER and VER, provided by NPCC’s Members, NERC, the industry, and Staff observations during the NPCC DER VER Forums.  Accordingly, there are two NPCC Regional Reliability Directories that this whitepaper recommends as ‘high priority’ or ‘high to moderate ‘ priority for review: Directory # 1 “Design and Operation of the Bulk Power System” and Directory # 4 “Bulk Power System Protection Criteria”.  Directory # 1 has entered into the revision process and the project Scope document contains review of DER and VER potential impacts on NPCC Criteria.

# Objective

The objective of this effort is to conduct a review of NPCC’s Criteria in the Directories for potential impacts of DER and VER deployment and not to conduct an exhaustive review of each Directory.  DER and VER deployment and technologies continue to be developed and implemented and this whitepaper is meant only as guidance based on what NPCC and its members anticipate at the time of its writing.

Unlike large transmission-connected generation facilities, DER facilities often do not provide essential grid reliability services (although some are technically capable of doing so). Some DER, classified as VER, are also inherently intermittent in nature and the introduction of these intermittent resources has resulted in increasing amounts of energy storage. Although, pumped storage technology has been available to power systems for many decades, there are challenges to the development of new pumped storage facilities and new battery-based storage technology is under active development and significant deployment. The framework for operating storage is currently undergoing significant debate and discussion. Modes of operation from peak shaving, essential reliability ancillary services, and time-of-day energy arbitrage are also under development. Any NPCC Directory changes which can be made to accommodate the evolution of storage technology or remove any perceived impediments to deployment of battery energy storage systems should be considered during Directory reviews.

For the purpose of this NPCC guidance whitepaper, DER refers to:

Any non-BPS or non -BES connected real or reactive power resources (generating units, multiple generating units at a single location, distributed generation installations, battery storage systems, etc.) located within the boundary of any distribution utility’s service territory, regardless of capacity.,. Some DER technologies are more intermittent in their production characteristics than resources that operate based on a controllable fuel input.

For the purpose of this NPCC guidance whitepaper, VER refers to:

An energy resource that:  has primary energy source variability that is beyond the full control of the facility owner or operator.

In this whitepaper, NPCC will also leverage work in progress by the SPIDERWG.  NPCC Staff has been participating in a sub-group of the SPIDERWG to analyze all NERC Reliability Standards, including the Operation and Planning Standards, for potential DER Impact.  However, as part of the SPIDERWG analysis, Cyber Security Standards were not evaluated.  The relationship of each NPCC Directory to specific recommendations provided for the associated NERC Standards was considered in the recommendations along with additional issues that may impact the NPCC Region. Also considered was the recent FERC order (Order 2222) on allowing individual DER aggregations as low as 100 kW to participate in wholesale markets in the US.  Although such aggregation improves visibility and control, the effect of aggregation  can have a significant effect on the power system and if not properly understood can impact the reliable operation of the BPS, as seen in California, where resource planning targets have not kept pace with the evolving power mix, and demand during peak hours outpaces the supply of solar-produced power. NPCC is now beginning to observe aggregations of DER entering into the capacity wholesale markets and has considered those potential impacts in this summary.  The suggested priorities in this whitepaper for the review of Directory criteria may need to be modified as emerging issues related to DER and VER deployment, interconnection, planning, and operations are identified, and their technology continues to improve.

Upon determination of the potential impact of DER and VER, with the considerations given to past work done, NPCC is providing a recommendation on prioritizing the review of each Directory. Each Directory is reviewed for revision on a three-year cycle. However, there may be a need to reprioritize the review and potential revision of a Directory based on need.  For the purposes of this analysis, it is recommended that consideration be given to immediately begin a review on Directory #1 Design and Operation of the Bulk Power System (currently initiated with Scope developed). Additionally, a focused review of  Directory # 4 System Protection Criteria to consider emerging protection issues is also recommended. These two Directories will have the most immediate impact because of DER and VER in the NPCC Region.

# NPCC Reliability Directories

The requirements of an NPCC Directory apply only to those facilities defined as NPCC bulk power system elements as identified through the performance-based methodology of NPCC Document A-10 Classification of Bulk Power System Elements the current list of which is maintained by the NPCC Task Force on System Studies and approved by the NPCC Reliability Coordinating Committee.

The potential reliability impact of increased penetration of Distributed Energy Resources, and  Variable Energy Resources (VER) provide a unique opportunity for NPCC Members to utilize the NPCC Task Forces and Working Groups to identify opportunities to enhance the criteria and address emerging risks which may be outside the jurisdiction of FERC and the NERC Reliability Standards.

As the penetration of DER and VER increases, a recognition that facilities on the distribution system and not on the BPS are impacting the planning and operating assessments contained in the criteria will be critical.

Resource variability, limited large-scale on-demand response, decreased short-circuit strength, load offset, flow direction changes, frequency stability, and UFLS effectiveness present an opportunity to strategically enhance the NPCC criteria to address these DER related issues.

NPCC currently maintains nine Regional Reliability Directory documents, each containing NPCC’s reliability Criteria.  Directory # 12 is scheduled to be retired in 2021 due to the adoption of NPCC’s Regional Standard on Automatic UFLS, PRC-006-NPCC-2, by all the applicable governmental authorities across the NPCC Region.  Therefore Directory # 12 was not considered in this analysis.

Details of the review and findings of each Directory for potential DER and VER impact appear below:

## Directory # 1 - Design and Operation of the Bulk Power System

Applicable NERC Standards-

- EOP-011- Emergency Operations

• FAC-011- System Operating Limits Methodology for the Operating Horizon

• IRO-002- Reliability Coordination Monitoring and Analysis

• IRO-014- Coordination Among Reliability Coordinators

• MOD-031- Demand and Energy Data

• MOD-032- Data for Power System Modelling and Analysis

• TOP-001- Transmission Operations

• TOP-002- Operations Planning

• TOP-003- Operational Reliability Data

• TPL-001- Transmission System Planning Performance Requirements

• VAR-001- Voltage and Reactive Control

Determination of Impact from DER and VER:

Responsible entities may need to consider DER data in their long-term operational planning considerations.  As large amounts of DER and VER are integrated as part of the power system, certain additional conditions may need to be addressed in the planning and operational reviews specified in Directory#1. For example, the addition of large amounts of DER solar resources may lead to mid-day minimum load conditions, which can be significantly lower than previously observed nighttime minimum load conditions, and which can impact other modeling parameters such as the bus-by-bus load distribution .

Existing requirements are applicable to "Small non-dispatchable DERs" connected to the bulk electric system; however, these requirements may be insufficient in the future for planning and operation with large amounts of DER. 	Likewise, although some voltage and frequency withstand/ride-through capabilities specified in IEEE Std 1547-2018 currently apply, they may not be sufficiently specific, and the criteria could address such a gap.

There  may also be a need for additional focus on how well Remedial Action Schemes (RAS) are coordinated with DER, particularly as more DER is deployed on the system.  The SPIDERWG has issued guidelines  to specify DER data requirements more granularly to capture the operational characteristics of DER and VER and require Distribution Providers to obtain and make available DER data, specifically, pertaining to aggregations, and reactive capabilities at a minimum. However,  NPCC should review the Directories in the context of the absence of this requirement in NERC Standards and consider the difficulty  for Planning Coordinators and Transmission Planners to obtain the necessary data from Distribution Providers.

Additionally, the loss of short circuit strength associated with the increased penetration of DER and VER along with the retirement of base generation and severe weather assumptions because of new technology (VER) could be considered in a review of Directory#1 .

Due to the uncertain nature of DER and VER, the methodologies used to determine operating limits may  not be accurate. The uncertainty of DER has a larger impact with increasing penetration of DER. Therefore, a consideration for new or clarified Criteria and methodologies may be needed to account for this uncertainty. The increasing penetration of DERs will likely require additional focus from Reliability Coordinators to when determining operating limits.  This may be done at the NERC level but NPCC should be cognizant of this potential effect on operations.

Finally, any review of Directory#1 to review the impact of DER and VER should consider both current and proposed methods of determining resource adequacy.

## Directory # 2 - Emergency Operations

Applicable NERC Standards-

- BAL-002- Disturbance Control Standard – Contingency Reserve for Recovery from a Balancing Contingency Event

• BAL-003- Frequency Response and Frequency Bias

• BAL-005- Automatic Generation Control

• EOP-011- Emergency Operations

• IRO-009- Reliability Coordinator Actions to Operate Within IROLs

• TOP-001- Transmission Operations

• TOP-002- Operations Planning

Determination of Impact from DER and VER:

It is important to understand that Balancing Authorities must understand all impacts, not just DER and VER, on balancing and regulating needs to meet the performance requirements of BAL-002 “Disturbance Control Standard – Contingency Reserve for Recovery from a Balancing Contingency Event.”  Year-over-year reviews of Balancing Authorities have shown consistent performance to the Standards and Criteria.  DER and VER need to be considered in the emergency operating plans insofar as how these resources are  used to avoid emergency conditions (DER shouldn't be curtailed during an energy emergency), and that under-frequency load shedding is not made ineffective by  lack of DER visibility. As DER penetration  continues to increase it may become necessary  to have adaptive UFLS systems in place in the future.  There is a benefit to consider providing the industry with guidance on considering DERs in UFLS studies and providing recommended practices for studies of gross load versus net load. Further consideration of how UVLS programs should be coordinated with DER's ride-through capability and there is also importance to modeling gross load and aggregate DER in under frequency load shedding (UFLS) simulations to ensure representation of the actual behavior of the BPS.

The increased amounts of DER  and associated data will need to be considered in the Real-time analysis process. DER data will have no precedence over any other data gathered in the evaluation process conducted by the Reliability Coordinator. However, if the DER  is identified as impactful  to  a system operating limit , the Reliability Coordinator should make every effort to coordinate with the appropriate distribution entities per the IEEE 1547 Standard (which may require the RC to change their methodology slightly to include the IEEE 1547 Standard language). Likewise, there may be an opportunity to achieve regional agreement within the criteria on protection issues to prevent issues such as observed in California.  Transmission and Distribution entities must follow the Operating Instructions received from their Reliability Coordinator, regardless of whether DER or VER operation was a factor in the limit, unless the entity provides a reliability reason for non-compliance. Furthermore, existing Standard requirements already address the next day Real-time evaluation for the System Operating Limit (SOL) as well as the Interconnection Reliability Operating Limits (IROLs).

## Directory # 4 - System Protection Criteria

Applicable NERC Standards-

- PRC-001- System Protection Coordination

- PRC-002- Disturbance Monitoring and Reporting Requirements

- PRC-004- Protection System Misoperation Identification and Correction

- PRC-005- Protection System, Automatic Reclosing, and Sudden Pressure Relaying Maintenance

- PRC-006- Automatic Underfrequency Load Shedding

- PRC-010- Undervoltage Load Shedding

- PRC-012- Remedial Action Schemes

- PRC-018- Disturbance Monitoring Equipment Installation and Data Reporting

- PRC-019- Coordination of Generating Unit or Plant Capabilities, Voltage Regulating Controls, and Protection

- PRC-023- Transmission Relay Loadability

- PRC-024- Generator Frequency and Voltage Protective Relay Settings

- PRC-025- Generator Relay Loadability

- PRC-026- Relay Performance During Stable Power Swings

- PRC-027- Coordination of Protection Systems for Performance During Faults (Future enforcement)

- FAC-008- Facility Ratings

Determination of Impact from DER and VER:

Although the criteria in Directory#4 presently focuses more on protection system design and independence, rather than specific criteria for relay settings, DER and transmission connected VER issues will likely impact considerations surrounding relay protection settings in the future.

As DER continues to be deployed in the Region there are instances where distribution feeders are back-feeding  into the Transmission System  creating design issues for protective relays and other control devices which may be designed for directional flows.

Additionally, as more and more transmission level generators come offline, deceasing levels of fault current will need to be considered to ensure that relay protection can operate as designed.

Finally,  the deployment of advanced Inverter functions and “grid forming” inverters may require a greater level of system protection coordination.

## Directory # 5 - Reserve

Applicable NERC Standards

• BAL-002- Disturbance Control Standard: Contingency Reserve for

Recovery from a Balancing Contingency Event

• BAL-005- Automatic Generation Control

• EOP-011- Emergency Operations

Determination of Impact from DER and VER:

The Criteria itself may not need revision for DERs and VERs, but a consistent and uniform approach regarding DERs (e.g., counting DERs either explicitly as resources or implicitly as load modifiers) may be needed.  A review of  the Criteria in the context of further deployment of these resources would be helpful, and any associated potential Glossary changes/additions may also be an effective way to implement consistent treatment from a Regional perspective. Reporting of net generation and net load will be affected by how DER is counted and the resultant apportionment of Interconnection Frequency Response Obligation (“IFRO”) to each BA.  Consideration should be given to address counting and capacity of DERs consistently.  NERC Reliability Subcommittee is currently reviewing this.

## Directory # 6 - Reserve Sharing Groups

Applicable NERC Standards

- BAL-002- Disturbance Control Standard: Contingency Reserve for Recovery from a Balancing Contingency Event

Determination of Impact from DER and VER:

NPCC’s Directory #6 includes specific requirements for types of spinning reserve.  These reserves were specified to support the NPCC Region meeting the BAL-002 standard as it relates to returning Area Control Error to pre-contingency levels in specific recovery periods.  As DER increases there will be less inertia on the system which may necessitate a review of the required spinning reserve assignments. Although,  DER and VER may eventually play a role in system reserve, at this time an immediate review of this issue is not required.

## Directory # 7 - Remedial Action Schemes

Applicable NERC Standards

- PRC-012 - Remedial Action Schemes

- PRC-017- Remedial Action Scheme Maintenance and Testing

Determination of Impact from DER and VER:

In general, this Directory specifies a process of implementation and periodic assessment of RAS. The process does not need to change with increasing levels of DERs or VER. The RAS may include tripping/shedding of a large amount of load, which is typically implemented with the tripping of feeder breakers in a distribution substation. If the presence of DERs is negligible, there may be little risk to BES reliability if DERs are also tripped with the load. However, in geographical areas with high penetration of DERs, loss of DERs due to  automatic underfrequency load shedding or RAS activated load shedding may have an impact on grid reliability. Even if the RAS is not configured to shed load, it is necessary that the design and performance of RAS considers interactions with DERs, especially when penetration of DERs is high. As such, the RAS design and analysis should consider the impact of DERs. Consideration of the potential future impact of DERs on RAS design and performance may or may not need to be specifically addressed in a  future revision of this Directory.

## Directory # 8 - System Restoration

Applicable NERC Standards

• EOP-005- System Restoration from Blackstart Resources

• EOP-006- System Restoration Coordination

• PRC-005- Protection System, Automatic Reclosing, and Sudden

Pressure Relaying Maintenance”

Determination of Impact from DER and VER:

As the deployment of DER continues, an item for consideration is if a determination should be made on which DER would be eligible as a Blackstart Resource to benefit system restoration to improve resilience. Although the general process for determining eligibility as a Blackstart Resource may address the need, the issue of limiting the ability of intermittent resources, such as solar resources, to provide restoration services needs to be considered. The standards under which certain DER were interconnected to the distribution system and the choice of inverter hardware may limit the usability of these facilities as black start resources. A revision to the Criteria to address if and how the use of intermittent resources in system restoration could contribute to reliability should be considered in a future revision.

## Directory # 11 - Disturbance Monitoring Equipment Criteria

Applicable NERC Standards

- PRC-002- Disturbance Monitoring and Reporting Requirements

Determination of Impact from DER and VER:

The Directory focuses on event data for analysis that may be useful for model validation, in addition to   disturbance monitoring equipment,  sequence of events recorders, and fault recorders.  For a system disturbance, it will be beneficial to record the response of larger DER installations or aggregations and assess whether DER will impact existing disturbance monitoring equipment . From a model validation perspective, the response of aggregate DERs may be adequate and further discussion when the Directory is revised should be considered.

Directory # 12 - Automatic UFLS Program Requirements (Scheduled to be retired in 2021)

NPCC’s Regional Standard, PRC-006-NPCC-2 has received full approval from all applicable governmental authorities within the NPCC Region.

# Directory Prioritization Recommendation

DER and VER impact on NPCC’s Reliability Directories should be considered during the next scheduled review with due consideration given to the suggested prioritization schedule below.

| Directory # | Title | Last Task Force Review | Current Status | Next Scheduled Review | Task Force | Priority |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Design and Operation of the Bulk Power System | September 2015 | TFCP/TFCO Drafting Scope | 2021 | TFCP/TFCO | High |
| 2 | Emergency Operations | June 2018 |  | 2021 | TFCO | Low |
| 4 | System Protection Criteria | January 2020 |  | 2023 | TFSP | High to Moderate |
| 5 | Reserve | September 2019 |  | 2022 | TFCO | Moderate |
| 6 | Reserve Sharing Groups | September 2019 |  | 2022 | TFCO | Low |
| 7 | Remedial Action Schemes | December 2020 |  | 2023 | TFCP/TFSP | Low |
| 8 | System Restoration | October 2017 | TFCO CO-11 Begin Review 1st Qtr. 2021 | 2021 | TFCO | Moderate |
| 11 | Disturbance Monitoring Equipment Criteria | October 2016 | Currently Under Review TFSP Posting 1st Qtr. 2021 | 2021 | TFSP | Low |
| 12 | UFLS Program Requirements | July 2013 | TFSS to Retire in 2021 | N/A | TFSS | N/A |

# Appendix A- Future Directory Review Considerations

As Directories are reviewed there are some questions the Task Forces could use as they examine the Criteria for revision.  The following questions were developed using some information from the NERC Energy Reliability Assessment Task Force (ERATF). They are only suggestions and not meant to be an all-encompassing set of issues to raise during the Directory reviews. The suggestions below may bridge operations and planning related Directory Criteria.

Directory Review Considerations:

- What flexibility is required to balance volatility in resource and load uncertainty through multiple operating horizons and seasons of the year to achieve NPCC’s reliability and energy sufficiency?

- Should emergency procedures and reserve criteria be revised to reflect current and new fleet structure and operating needs?

- When and how should demand response be considered when assessing fuel availability and energy adequacy?

- Should the fuel availability / energy adequacy of battery or long-duration storage be evaluated and considered in NPCC’s Directories?

- Should NPCC’s more specific Criteria consider including the reliability benefit of strategically overbuilding a similar technology (i.e., solar) augmented by either storage or some portion of the firm capacity fleet (albeit operating at low-capacity factors only when needed) could provide for a resilient and reliable transition?

- Should longer time horizons be considered for long term planning studies with local de-carbonization regulatory goals considered?  (for example, Electric Vehicle charging on the distribution system and build out of the transmission system needed to accommodate and integrate offshore wind.)

- How could large amounts of DER affect UFLS performance and System Protection functions.  Do the Directories need to be revised to include additional more stringent or specific requirements for effective grounding, harmonics, short circuit etc., beyond what may be published in standards such as IEEE-1547-2018 and IEEE-519?

- Are schedules for NPCC reviews unnecessarily time consuming and posing obstacles to the deployment of DER or VER?  Examples are the time to examine and approve a Remedial Action Scheme or perform an A-10 analysis to determine if system elements are part of or newly recognized as part of the NPCC Bulk Power System.

- Should NPCC have disturbance monitoring and performance measuring equipment and communication requirements for the Distribution System to capture data for system events?

# Appendix B- Decarbonization Goals within NPCC

• CT‐ 80% below 2001 levels by 2050

• MA‐ Net zero by 2050 (ET) and 80% below 1990 by 2030

• ME‐ Net zero by 2050 (ET) and 80% below 1990 by 2030

• NH‐ 80% below 1990 levels by 2050

• NY‐ Net zero by 2040, 9,000 Offshore Wind, 6000 Solar, 3000 Storage

• RI‐ 80% below 1990 levels by 2050

• VT‐ 80% below 1990 levels by 2050

• NS‐ Net Zero by 2050

• ON‐ 30% below 2005 levels by 2030 (Paris Climate)

• QC‐37.5 percent of 1990 levels by 2030, Net Zero by 2050

• US Federal Proposed goal‐net‐zero emissions no later than 2050

NPCC Directories contain requirements which are expected to be met by NPCC Full Members as is applicable to their functions in maintaining the reliability of the NPCC Bulk Power System.

“reliable operation” is defined in 16 U.S. Code § 824o and means “operating the elements of the bulk-power system within equipment and electric system thermal, voltage, and stability limits so that instability, uncontrolled separation, or cascading failures of such system will not occur as a result of a sudden disturbance, including a cybersecurity incident, or unanticipated failure of system elements.”