7-9. Solar Facilities.

7.9.1 Purpose. The purpose of this section is to establish general guidelines for the siting of solar photovoltaic facilities, hereinafter referred to as solar facilities. The goals of this section are to:

1.	Promote the health, safety and general welfare of the community by allowing the generation of sustainable energy by solar facilities in order to reduce air pollution and greenhouse gases, protect environmental resources, and foster sustainable economic development.

2.	Provide standards for the placement, design, construction, operation, monitoring, modification and removal of solar facilities. Such siting standards shall address public safety, minimize impacts on natural resources, and provide adequate financial assurance for the eventual decommissioning of such facilities.

3.	Not prohibit or unreasonably regulate the installation of solar energy systems or the building of structures that facilitate the collection of solar energy per M.G.L. c. 40A, § 3, except where necessary to protect the public health, safety or welfare.

7.9.2 Applicability. This section applies to all solar facilities proposed to be constructed after the effective date of this section. This section also pertains to physical modifications that materially alter the type, configuration, or size of these installations or related equipment. This section does not apply to minor modifications or maintenance of a solar facility.

Solar facilities shall be allowed with a Special Permit of the City Council on parcels of land within the RA, R1, R1A, R2, RM-LD, RM-20, RM-40, RM-60, RO and DR zones. Solar facilities shall be allowed as-of-right in all other zones in accordance with the provisions of this Section.

7.9.3 Definitions.

As-of-right: As-of-right shall mean that development may proceed without the need for a special permit, variance, amendment, waiver, or other discretionary approval. In permitted zones, as-of-right development will be subject to site plan review for solar photovoltaic arrays having a kilowatt direct current (kW-DC) rated nameplate capacity of greater than 250 or more to determine conformance with Holyoke's Zoning Ordinance. Projects cannot be prohibited, but can be reasonably regulated.

Ground mounted solar facility: A solar facility that is structurally mounted on the ground.

Large scale ground mounted solar facility: A solar facility that is structurally mounted on the ground and has a minimum nameplate capacity of 10 kW-DC or greater.

Project site: A parcel or combination of parcels, which the solar facility operator has control of, on which the solar facility is or will be located.

Rated nameplate capacity: The maximum rated output of electric power production equipment in direct current (DC). This output is typically specified by the manufacturer with a "nameplate" on the equipment.

Roof mounted solar facility: A solar facility that is structurally mounted on the roof of a building, residence, parking garage, or any other structure.

Site plan review: review by the planning board to determine conformance with subsection 7.9.6 of this section of Holyoke's Zoning Ordinances.

Small scale ground mounted solar facility: A solar facility that is structurally mounted on the ground and has a maximum nameplate capacity of less than 10 kW-DC.

Special permit: the permit issued by the City Council to allow the use of land for a solar facility in accordance with the provisions of M.G.L. ch. 40A.

Solar facility: A facility comprised of one or more solar panels, as well as all access roads and appurtenant structures.

7.9.4 General requirements.

1.	Large scale ground mounted solar facilities: Large scale ground mounted solar facilities shall be allowed with a special permit of the City Council and site plan review approval by the planning board on parcels of land within the RA, R1, R1A, R2, RM-LD, RM-20, RM-40, RM-60, RO and DR zones, and allowed as-of-right in all other zones with site plan review from the planning board pursuant to this section, subject to the following conditions:

a.	Site plan review. No large scale ground mounted solar facility shall be constructed, installed or modified as provided in this section without first obtaining a special permit of the City Council and/or site plan review approval by the Holyoke Planning Board in compliance with subsections 7.9.5 and 7.9.6 of this section.

b.	Monitoring and maintenance. The solar facility shall comply with subsection 7.9.8 of this section.

c.	Site control. The applicant shall submit with its application for a special permit and/or site plan review, documentation of actual or prospective control of the project site sufficient to allow for installation and use of the proposed facility. Notice of change of ownership shall be given to the city council and/or planning board in compliance with subsection 7.9.9 of this section.

d.	Financial surety. The applicant shall provide a financial surety if so required by the city council and/or planning board as determined in compliance with subsection 7.9.10 of this section.

i. 	The financial surety provided by the applicant shall include a surety in the amount of the estimated cost of the landscaping improvements, which surety may be used by the City in the event the landscaping is not completed as approved.

e.	Compliance with laws, ordinances and regulations. The construction and operation of all large scale ground mounted solar facilities shall be consistent with all applicable local, state and federal requirements, including but not limited to all applicable safety, construction, electrical and communications requirements.

f.	Proof of liability insurance. The applicant shall be required to provide evidence of liability insurance in an amount sufficient to cover loss or damage to persons and structures occasioned by the failure of the facility.

2.	Small scale ground mounted solar facilities. Small scale ground mounted solar facilities shall be allowed with a special permit of the city Council on parcels of land within the RA, R1, R1A, R2, RM-LD, RM-20, RM-40, RM-60, RO and DR zones , and as-of-right with a building permit in all zones provided that they meet the following conditions:

a.	Compliance with laws, ordinances and regulations. The construction and operation of all small scale ground mounted solar facilities shall be consistent with all applicable local, state and federal requirements, including but not limited to all applicable safety, construction, electrical and communications requirements.

b.	Proof of liability insurance. The applicant shall be required to provide evidence of liability insurance to the building commissioner in an amount sufficient to cover loss or damage to persons and structures occasioned by the failure of the facility.

c.	Design standards. The solar facility shall comply with subsection 7.9.6.2 design standards and subsection 7.9.6.3 environmental standards where applicable.

d.	Monitoring and maintenance. The solar facility shall comply with subsection 7.9.8 of this section.

e.	Landscaping surety. The applicant shall provide a financial surety in the amount of the estimated cost of the landscaping improvements, which surety may be used by the City in the event the landscaping is not completed as approved.

3.	Roof mounted solar facilities. Roof mounted solar facilities shall be allowed as-of-right with a building permit in all zones provided that they meet the following conditions:

a.	Compliance with laws, ordinances and regulations. The construction and operation of all roof mounted solar facilities shall be consistent with all applicable local, state and federal requirements, including but not limited to all applicable safety, construction, electrical and communications requirements.

b.	Proof of liability insurance. The applicant shall be required to provide evidence of liability insurance to the building commissioner in an amount sufficient to cover loss or damage to persons and structures occasioned by the failure of the facility.

c.	Structural engineering report. A structural engineering report may be required by the building commissioner illustrating the structural integrity of the structure and its ability to support the proposed roof mounted solar facility.

d.	Monitoring and maintenance. The solar facility shall comply with subsection 7.9.8 of this section.

7.9.5	Special Permit.

1.	Submittal requirements. The project proponent is required to provide to the Holyoke City Council with the following;

a. 	Two (2) original applications;

b. 	Required Fee and publication costs of the public hearing notice; and

c. 	A plot plan acceptable to the City Council showing the location of the lot,

the lot dimensions, vegetative buffering, and location and size of the proposed ground mounted solar facility

2.	Special Permit Criteria:

A special permit may be granted where the City Council finds that the benefit to the city and the neighborhood outweigh the adverse effects of the proposed use, taking into account the characteristics of the site and of the proposal in relation to that site. In addition to any specific factors that may be set forth in this ordinance, the determination shall include consideration of each of the following:

a.	Social, economic, or community needs which are served by the proposal;

b.	Traffic flow and safety, including parking and loading;

c.	Adequacy of utilities and other public services;

d.	Neighborhood character and social structures;

e.	Impacts on the natural environment;

f.	Potential fiscal impact, including impact on city services, tax base, and employment; and

g.	To provide and maintain adequate screening via improved landscaping.

7.9.6 Site plan review.

1.	Submittal requirements. The project proponent is required to provide the Holyoke Planning Board with the following:

a.	Application. Two original application forms and a designer's certificate.

b.	Fee. Required fee.

c.	Siting and design. Eight full copies of a site plan. The plan shall be on 24" × 36" sheets at a scale of 1"=40' or 1"=200', as appropriate, on as many sheets as necessary. Site plans shall be prepared by a Massachusetts licensed professional engineer and/or a registered land surveyor, as applicable. The site plan shall include the following:

1)	Location map. Copy of the most recent USGS quadrangle map, at a scale of 1:25,000, showing the proposed facility site and the area within at least two miles from the facility.

2)	Site plan. A one inch equals 200 feet plan of the proposed solar facility site, with contour intervals of no more than ten feet, showing the following:

a.	Property lines and physical dimensions of the project site and adjacent parcels within 100 feet of the project site;

b.	Location of permanent structures or buildings on the project site and on adjacent parcels of the project site;

c.	Location and details of all security measures for the site; and

d.	Location of all existing and proposed roads, both public and private, on the project site.

3)	Project plan. A plan indicating all proposed changes to the landscape of the site, including temporary or permanent roads or driveways, grading, vegetation clearing and planting, exterior lighting, screening vegetation or structures shall include the following:

a.	Proposed changes to the landscape of the site, grading, vegetation to be removed or altered, amenities such as lighting or fencing, screening vegetation or structures, and wetlands delineation. Lighting shall be designed to minimize glare on abutting properties and be directed downward with full cutoff fixtures to reduce light pollution;

b.	Location of the ground mounted solar facility, type of mounting devices, access roads, lighting, ground equipment, fencing, electrical infrastructure, and associated equipment;

c.	Plans for accessory buildings or other structures, and location and details of all planned security measures;

d.	Layout and details of surfacing for access roads and parking including temporary roads and staging areas; and

e.	Any existing overhead utility lines.

d.	Operation and maintenance plan. The applicant shall submit a plan for the general maintenance of access roads and stormwater controls, as well as general procedures for operational maintenance of the ground mounted solar facility.

e.	Schematics.

a.	Schematic or blueprints of the ground mounted solar facility signed by a professional engineer licensed to practice in the Commonwealth of Massachusetts showing the proposed structures and any shading from nearby structures;

b.	Schematic or outline electrical diagram showing proposed solar panels, associated components and electrical interconnection methods, all with National Electrical Code compliant disconnects and overcurrent devices;

c.	Description of the major system components to be used including the photovoltaic panels, mounting system and inverter.

f.	Compliance documents. The applicant will provide the following with the application:

a.	A description of financial surety that satisfies subsection 7.9.10.3 of this section;

b.	Proof of liability insurance that satisfies subsection 7.9.4 of this section;

c.	Name, address, and contact information for:

- Proposed system installer,

ii.	The landowner,

- The project proponent, as well as all co-proponents; and

iv.	Any agents representing the applicant.

d.	Evidence of utility notification that satisfies subsection 7.9.8.2 of this section.

g.	Notification.

a.	List of property owners and their addresses for all parcels of land within 300 feet of the project site, to be obtained from the most recent property list from the Holyoke Assessor's Office;

b.	Provide stamped A10 sized envelopes representing twice the number of abutters listed above to be used by the planning board to mail notice of the site plan review hearing and notice of decision.

c.	The applicant shall be responsible for the cost of publication of the public hearing notice.

h.	Waiver of documents. The planning board reserves the right to waive documentary requirements as it deems appropriate.

2.	Design standards.

a.	Screening. A ground mounted solar facility shall be screened, to the extent possible, from abutting properties.

(a)	General. Screening shall consist of landscaping, fence, grassed earthen berm, or some combination of these screening devices. If utilizing a natural buffer, it shall be maintained above the highest level of the solar panels. When a screen consists of plant materials, said materials shall provide screening at the time of planting and be of a type that will be expected to form a year-round, dense screen.

(b)	Abutting residential uses. When such facility is directly abutting existing residential uses, such screening shall consist of:

(1)	Project site of less than two acres: Screening as determined to be adequate in the form of either vegetation or fencing.

(2)	Project site of between two and five acres: A minimum of 50 feet of vegetation buffer with 25 feet being undisturbed closest to the residential property, and the other 25 feet being allowed to be selectively cleared.

(3)	Project site of greater than five acres: A minimum of 100 feet of vegetation buffer with 50 feet being undisturbed closest to the residential property, and the other 50 feet being allowed to be selectively cleared.

(4)	Permit for screening reduction: An applicant may request permission to reduce such buffer requirements in such instances it is determined to not have a detrimental effect to the abutters and in such instances where the buffer will have a detrimental effect on the ability to generate power.

(c)	Abutting nonresidential uses including public accepted streets. Screening as determined to be adequate in the form of either vegetation or fencing.

b.	Lighting. Lighting of solar facilities shall be consistent with local, state and federal law and shall further be directed downward and shall incorporate full cut-off fixtures to reduce light pollution. Lighting of other parts of the installation, such as appurtenant structures shall be limited to that required for safety and operational purposes and shall be shielded from abutting properties.

c.	Signage. Signs on the solar facility shall comply with the requirements of the city's sign regulations, and shall be limited to:

(1)	Those necessary to identify the owner, provide a 24-hour emergency contact phone number, and warn of any danger.

(2)	Educational signs providing information about the facility and the benefits of renewable energy.

(3)	Solar panels shall not be used for displaying any advertising signage, except for any labeling or identification by the manufacturer and/or operator of the solar facility.

d.	Utility connections. Reasonable efforts should be made to locate utility connections for ground mounted solar facilities underground, depending on appropriate soil conditions, shape and topography of the site and any requirements of the utility provider. Electrical transformers for utility interconnections may be above ground, if required by the utility provider. All solar facility installations shall conform to the requirements of the interconnection agreement and/or such further requirements as may be promulgated from time to time, as appropriate and as approved by the connecting utility.

e.	Appurtenant structures. All such appurtenant structures, including but not limited to, equipment shelters, storage facilities, transformers and substations, shall be architecturally compatible with each other. Structures should be shaded from view by vegetation and/or joined or clustered to avoid adverse visual impacts. Appurtenance structures shall meet the regulations for accessory structures as found in section 4.4 of the zoning ordinance.

f.	Emergency services. The applicant shall provide a copy of the project summary and site plan to the local emergency planning committee (LEPC). The applicant shall cooperate with the LEPC in developing an emergency response plan. All means of disconnecting the solar facility shall be clearly marked. The applicant or facility owner shall identify a responsible person for public inquiries or complaints throughout the life of the project.

g.	Unauthorized access. All solar facilities shall be designed to prevent unauthorized access in compliance with any and all federal, state and local regulations. Electrical equipment shall be locked where possible. Video surveillance cameras shall be oriented in such a fashion so as to minimize capturing activity outside the solar facility.

3.	Environmental standards.

a.	Land clearing. Clearing of natural vegetation shall be limited to what is necessary for the construction, access to, operation and maintenance of the ground mounted solar facility or otherwise prescribed by applicable laws, regulations and ordinances.

b.	Rare and endangered species. The applicant shall provide evidence of compliance with the Massachusetts Endangered Species Act and requirements of the Commonwealth of [Massachusetts] Natural Heritage and Endangered Species Program.

c.	Wetlands. The applicant shall provide evidence of compliance with the Massachusetts Wetlands Protection Act, the Massachusetts Rivers Protection Act and the City of Holyoke Wetlands Protection Ordinance.

d.	Stormwater. The applicant shall demonstrate compliance with all local, state and federal stormwater management laws and regulations.

e.	Water resource protection overlay district (WRPOD). If a large scale ground mounted solar facility is located within the WRPOD I or II as illustrated on the Holyoke Zoning Map, the applicant must present the project to the Barnes Aquifer Protection Advisory Committee.

7.9.7 Action by the planning board. Site plan review shall be conducted in accordance with the notice, hearing and filing procedures set forth in M.G.L. c. 40A for special permits, except as otherwise set forth in this section 7-9. After determining if the site plan is in conformance with the requirements of this ordinance, and after considering the criteria set forth in this section 7-9, the planning board may approve, approve with modifications, or disapprove the site plan application or grant leave to withdraw. Approval may be subject to any conditions, modifications and/or restrictions as the planning board may deem necessary. Leave to withdraw or disapproval by the planning board must be supported by written findings.

7.9.8 Monitoring and maintenance.

1.	Facility conditions. The applicant shall maintain the solar facility in good condition. Maintenance shall include but not be limited to vegetation upkeep, structural repairs and the integrity of security measures. Site access shall be maintained to a level acceptable to the local fire chief, local emergency planning committee and emergency medical services. The project owner shall be responsible for the cost of maintaining the solar facility and any access road, unless accepted as a public way, and the cost of repairing any damage as a result of operation and construction. The project owner shall also be responsible for ensuring that the solar facility does not at anytime lower the quality of service supplied to nearby customers or cause safety problems to the interconnected electrical grid.

2.	Utility notification. Before installation of the solar facility, the applicant shall inform the utility company that operates the electrical grid where the facility is to be located of its intent to install an interconnected generator and shall satisfy all interconnection agreements.

3.	Modifications. All material modifications to a large scale ground mounted solar facility, other than regular maintenance, made after issuance of the site plan review approval shall require approval by the planning board as provided in this section.

7.9.9 Change in ownership. If the owner and/or operator of a large scale ground mounted solar facility changes, notice shall be given to the planning board with the contact information of the new owner/operator within one month of the change in ownership and/or operations.

7.9.10 Abandonment or decommissioning.

1.	Removal requirements. Any large scale ground mounted solar facility which has reached the end of its useful life or has been abandoned consistent with subsection 7.9.10.2 of this section shall be removed. The owner or operator shall physically remove the installation no more than 150 days after the date of discontinued operations. The owner or operator shall notify the planning board by certified mail of the proposed date of discontinued operations and plans for removal. Decommissioning shall consist of:

a.	Physical removal of all solar panels, structures, equipment, security barriers and electrical lines from the site.

b.	Disposal of all solid and hazardous waste in accordance with local, state, and federal waste disposal regulations.

c.	Stabilization or re-vegetation of the site as necessary to minimize erosion. The planning board may allow the owner or operator to leave landscaping, designated below-grade foundations or service roads in order to minimize erosion and disruption to vegetation.

2.	Abandonment. Absent notice of a proposed date of decommissioning, the large scale ground mounted solar facility shall be considered abandoned when the facility fails to operate for more than one year without written consent of the planning board. The planning board shall determine in its decision what proportion of the facility is inoperable for the facility to be considered abandoned. If the applicant fails to remove the large scale ground mounted solar facility in accordance with the requirements of this section within 150 days of abandonment or the proposed date of decommissioning, the city shall have the authority to enter the property and physically remove the facility.

- Financial surety. The planning board shall require the applicant for a large scale ground mounted solar facility to provide a form of surety, either through escrow account, bond or otherwise, to cover the estimated cost of removal in the event that the city must remove the facility, of an amount and form determined to be reasonable by the planning board, but in no event to exceed more than 125 percent of the estimated cost of removal and compliance with the additional requirements set fourth herein, as reasonably determined by the applicant. Such surety will not be required for municipally or state owned facilities. The applicant shall submit a fully inclusive estimate of the costs associated with removal, prepared by a qualified engineer. The amount shall include a mechanism for a cost of living adjustment.

  - The financial surety provided by the applicant shall include a surety in the amount of the estimated cost of the landscaping improvements, which surety may be used by the City in the event the landscaping is not completed as approved.