| Form 4.2-4 HCOC Assessment for Time of Concentration (DA ) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Compute time of concentration for pre and post developed conditions for each DA (For projects using the Hydrology Manual complete the form below) |  |  |  |  |  |  |  |  |
| Variables | Pre-developed DA1 |  |  |  | Post-developed DA1 |  |  |  |
|  | DMA E | DMA F | DMA G | DMA H | DMA E | DMA F | DMA G | DMA H |
| 1 Length of flowpath (ft) Use Form 3-2 Item 5 for pre-developed condition |  |  |  |  |  |  |  |  |
| 2 Change in elevation (ft) |  |  |  |  |  |  |  |  |
| 3 Slope (ft/ft), So = Item 2 / Item 1 |  |  |  |  |  |  |  |  |
| 4 Land cover |  |  |  |  |  |  |  |  |
| 5 Initial DMA Time of Concentration (min) Appendix C-1 of the TGD for WQMP |  |  |  |  |  |  |  |  |
| 6 Length of conveyance from DMA outlet to project site outlet (ft) May be zero if DMA outlet is at project site outlet |  |  |  |  |  |  |  |  |
| 7 Cross-sectional area of channel (ft2) |  |  |  |  |  |  |  |  |
| 8 Wetted perimeter of channel (ft) |  |  |  |  |  |  |  |  |
| 9 Manning’s roughness of channel (n) |  |  |  |  |  |  |  |  |
| 10 Channel flow velocity (ft/sec) Vfps = (1.49 / Item 9) * (Item 7/Item 8)^0.67 * (Item 3)^0.5 |  |  |  |  |  |  |  |  |
| 11 Travel time to outlet (min) Tt = Item 6 / (Item 10 * 60) |  |  |  |  |  |  |  |  |
| 12 Total time of concentration (min) Tc = Item 5 + Item 11 |  |  |  |  |  |  |  |  |
| 13 Pre-developed time of concentration (min): Minimum of Item 12 pre-developed DMA |  |  |  |  |  |  |  |  |
| 14 Post-developed time of concentration (min): Minimum of Item 12 post-developed DMA |  |  |  |  |  |  |  |  |
| 15 Additional time of concentration needed to meet HCOC requirement (min): TC-HCOC = (Item 14 * 0.95) – Item 13 |  |  |  |  |  |  |  |  |