| Form 4.3-6 Volume Based Biotreatment (DA ) – Bioretention and Planter Boxes with Underdrains |  |  |  |
| --- | --- | --- | --- |
| Biotreatment BMP Type (Bioretention w/underdrain, planter box w/underdrain, other comparable BMP) | DA DMA BMP Type | DA DMA BMP Type | DA DMA BMP Type (Use additional forms for more BMPs) |
| 1 Pollutants addressed with BMP List all pollutant of concern that will be effectively reduced through specific Unit Operations and Processes described in Table 5-5 of the TGD for WQMP |  |  |  |
| 2 Amended soil infiltration rate Typical ~ 5.0 |  |  |  |
| 3 Amended soil infiltration safety factor Typical ~ 2.0 |  |  |  |
| 4 Amended soil design percolation rate (in/hr) Pdesign = Item 2 / Item 3 |  |  |  |
| 5 Ponded water drawdown time (hr) Copy Item 6 from Form 4.2-1 |  |  |  |
| 6 Maximum ponding depth (ft) see Table 5-6 of the TGD for WQMP for reference to BMP design details |  |  |  |
| 7 Ponding Depth (ft) dBMP = Minimum of (1/12 * Item 4 * Item 5) or Item 6 |  |  |  |
| 8 Amended soil surface area (ft2) |  |  |  |
| 9 Amended soil depth (ft) see Table 5-6 of the TGD for WQMP for reference to BMP design details |  |  |  |
| 10 Amended soil porosity, n |  |  |  |
| 11 Gravel depth (ft) see Table 5-6 of the TGD for WQMP for reference to BMP design details |  |  |  |
| 12 Gravel porosity, n |  |  |  |
| 13 Duration of storm as basin is filling (hrs) Typical ~ 3hrs |  |  |  |
| 14 Biotreated Volume (ft3) Vbiotreated = Item 8 * [(Item 7/2) + (Item 9 * Item 10) +(Item 11 * Item 12) + (Item 13 * (Item 4 / 12))] |  |  |  |
| 15 Total biotreated volume from bioretention and/or planter box with underdrains BMP: Sum of Item 14 for all volume-based BMPs included in this form |  |  |  |