Salt Lake City Department of Public Utilities

2009 Water Conservation Master Plan

stephanie.duer@slcgov.com  

Goals/Actions Template

Water Conservation Master Plan
	2009 SLC WATER CONSERVATION MASTER PLAN
Insert Department/Division/Organization/Company Name



	GOAL
	ACTION
	TIMELINE FOR IMPLEMENTATION
	MEASURE

	Reduce load impact on City stormwater systems
	Divert stormwater to cistern for use in landscape irrigation
	Diversion complete, awaiting permission from city to use water for landscape irrigation
	City approval needed.

	Enhance awareness among employees, students and visitors of importance of water conservation
	Building science on display highlights water conservation in building entry. 
In addition, the students are creating a building dashboard that will track building water use and provide enhanced information on the water re-use system and water conservation strategies.
	Signage being produced and software currently being developed by students. All should be completed by September for building grand opening. 
	Signage and software completion.

	Promote native plants in that use less water
	Westminster College is planting all new planted areas with native and adapted vegetation. All Science Center planted areas are water wise and adapted plant varieties. 
	Information on plant types and varieties may be included in the project website. All plants have been planted.
	Planting complete.
Website information forthcoming. 

	Supplementing rainwater catchment with evaporative cooling flush water.
	A regular flush of the evaporative cooling system results in hundreds of gallons of water entering the City wastewater system each week. The Science Center has captured this untreated water and is using it for landscape irrigation. 
	Diversion and cistern installation complete.
	Complete.

	Overall reduction in water use in the Science Center
	Building signage also highlights water saving strategies in the building, such as how to use the dual flush toilets.
Building water use was reduced by more than 40% through installation of dual flush toilets, 1 pint urinals and sensors at faucets. The installation of these items is complete.
	Complete.
	Complete. 


Page 1 of 2
2009_WCMP_actions_template.doc
7/22/2010

