6. GREEN BUILDINGS AND SERVICE:

6.1. GREEN CLEANING POLICY:

6.1.1. The organizations shall have in place a green cleaning policy that is distinct and separate from the Environmental Policy.

See Green Cleaning Policy

6.1.2. The organizations shall indicate the specific buildings, group of buildings to which the green cleaning policy applies.

The green cleaning policy applies to all 68 Colorado College buildings, houses, and offices serviced by Sodexo Environmental Services.

6.1.3. The organization shall incorporate the measurement tools, feedback cycle, and continuous improvement plan requirements to the effectiveness of green cleaning technologies, procedures, and processes.

6.2. Green/High Performance Cleaning Program

6.2.1. Staffing: See Section 2.1a and 2.1d.

Work-loading is compliant with Section 2.1.1., and the Quality Management Plan.

6.2.2. Training

6.2.2.1 Supervisory Training- See Section 3.5.

6.2.2.2. Worker Training- See Section 3.6.

6.2.2.3. Safety Training- See Section 4.1.

6.2.2.4. Green Cleaning Training.

At the core of our green cleaning program is training.  All custodians are fully trained on green cleaning techniques using hands-on training in a small group setting.  Vendors are often used as they provide subject matter expertise and third party experiences.   Green cleaning training is conducted annually.

6.2.3. Chemical Handling, Storage and Disposal

See Section 4.1.a. and 4.1.b.

6.2.4. Floor Care Log: The organizations shall have a written maintenance plan for the buildings listen in Section 6.1.2. which tracks the:

6.2.4.1. Number of coats of floor finish applied as base and top coats

6.2.4.2. Scheduled maintenance and restoration procedures and dates

Sodexo ES created a maintenance calendar for all floor services using cleaning

management software including the flooring type, product use and frequency. Base

coats are determined by flooring type and amount of use.  The software allows us to

ensure that floors are refinished according to a specific schedule.

6.2.5. Carpet Care Log: The organizations shall have a written carpet care plan for the buildings listed in 6.1.2 that includes scheduled maintenance and restoration procedures and dates.

Environmental Services created a maintenance calendar for all floor services including

carpets using cleaning management software including the flooring type, product use and

frequency. Base coats are determined by flooring type and amount of use.  The software

allows us to ensure that floors are refinished according to a specific schedule.

6.2.6. Hand Hygiene: The organizations shall develop strategies for promoting and improving hand hygiene.

The College and the department promote the frequent hand washing and the use of PPE

when staff work in an area that would expose them to infections disease.

6.3. Custodial Effectiveness Assessment (Quality System)

6.3.1. The organization shall evaluate the effectiveness of its green/high performance cleaning program by meeting the requirements set forth in Section 1 (Quality System).

The effectiveness of the green cleaning process is an on-going process and has been in place since 2007.  After benchmarking our program we have gone through a few minor changes but the main principles, supplies and chemicals have remained the same, meeting our green cleaning policy and the college sustainability requirements.

6.4. Purchase of Cleaning Products and Materials (the requirements of this section apply only to products that are purchased by the organization for use in buildings listed in Section 6.1.2.)

6.4.1. Dilution Control: The organization shall use chemical concentrates with appropriate dilution systems to minimize chemical use to the maximum extent possible.

Sodexo ES uses a controlled chemical dispensing unit, the Ecolab QuikClik in nearly all buildings to ensure that chemicals are dispensed at the proper rate, minimize waste, and avoid chemical contact or spillage issues.  Each unit is calibrated annually by the Ecolab rep. or ES supervisors.

6.4.2. Cleaning Tools: The organization shall use the environmentally preferable cleaning materials, such as microfiber tools wipes, whenever practical.

Sodexo ES uses microfiber cloths and mops for all cleaning.  Cotton or rayon mops are used only for applying floor finish or cleaning in areas not conducive to microfiber, such as the ice rink.

6.4.3. Cleaning Products: Products in this group include:

- General-purpose, bathroom, glass and carpet cleaners used for industrial and institutional purposes

- Cleaning and degreasing compounds

- Hard surface cleaners

- Carpet and upholstery care products

See table below.

6.4.3.1. Criteria: A specified percentage of products in this group shall:

- Meet applicable Green Seal or Environmental Choice certification criteria or,

- Meet the requirements for recognition under the U.S. Environmental Protection Agency’s (EPA’s) Design for the Environmental Program (DfE).

CLEANING PRODUCTS

Manufacturer		                  Product		    	  Green Seal Ranking

| Ecolab | Peroxide Glass & Surface Cleaner | Certified |
| --- | --- | --- |
|  | Neutral Bathroom Cleaner | Certified |
|  | Neutral Disinfectant Cleaner | Certified |
|  | Revitalize Carpet & Upholstery | Certified |

Degreaser is used as needed but is not part of the routine cleaning procedure.

6.4.3.2. Requirement: A minimum of sixty (60) percent, by purchase cost of the cleaning products provided by the organization and used in the buildings to which this section applies (listed in Section 6.1.2) shall meet the above criteria (6.4.3.1.).

Sodexo ES purchasing meets these criteria.

6.4.4. Disinfectants, metal polish, floor finishes, strippers or other products not included in Section 6.4.1. Products in this group include:

- Disinfectants

- Digestion additives for cleaning and odor control

- Drain or grease traps additives

- Hard floor care

6.4.4.1. Criteria: A specified percentage of products in this group shall satisfy one of the following:

- Meet applicable Green Seal or Environmental Choice certification criteria or,

- Meet the requirements for recognition under the U.S. Environmental Protection Agency’s (EPA’s) Design for the Environmental Program (DfE) or,

- Comply with the California Code of Regulations maximum allowable VOC levels for the specific product category.

Disinfectants are EPA’s Design for the Environmental Program (DfE) recommended.

Digestion additives; drain and grease trap additives are not used.  Stripper is not used.

6.4.4.2. Requirement: A minimum of sixty (60) percent, by purchase cost, of the products specified in Section 6.4.4. that are provided by the organization and used in buildings to which this section applies (listed in Section 6.1.2.) shall meet the above criteria (6.4.4.1.).

Sodexo ES meets this criteria.  See purchase orders for the last six months.

6.4.5. Disposable janitorial paper products and trash bags: This provision only applies to those purchases that are in the control of the organization. Products in this group include:

- Paper towels and napkins

- Facial tissue

- Toilet tissue

- Hand towels

- Kitchen towels

- Plastic trash bags

6.4.5.1. Criteria: A specified percentage of products in this group shall satisfy one of the following:

- Meet applicable Green Seal or Environmental Choice certification criteria or,

- Meet the requirements of the U.S. Environmental Protections Agency’s (EPA’s) Comprehensive Procurement Guidelines for Janitorial Paper and Plastic Trash Can Liners or,

- Be derived from rapidly renewable resources or made from tree-free fibers

PAPER PRODUCTS

Manufacturer	                       Product	                        Recycled Content            Green Seal

| Georgia Pacific | EnMotion Paper Towel | 100% recovered, 50% post-consumer | Recommended |
| --- | --- | --- | --- |
|  | Multi-Fold Towel | 100% recovered, 50% post-consumer | Recommended |
|  | Centerpull Towel | 40% | Recommended |
|  | Envision 2-ply Toilet Paper | 95%, 40% post-consumer | Recommended |
|  | Compact Coreless TP | 100% recovered, 25% post consumer | Recommended |

CAN LINERS

Manufacturer	                        Product                           Recycled Content            Green Seal

| Republic | HDPE liner 22 mic. | 10% | Recommended |
| --- | --- | --- | --- |

Response: Facial tissues are not used.

6.4.5.2. Requirement: A minimum of fifty (50) percent, by purchase cost, of the products provided by the organization and used in buildings to which this section applies (listed in Section 6.1.2.) shall meet the above criteria (6.4.5.1.)

100% of all paper products and liners meet Green Seal or EPA’s Design for the Environmental Program (DfE) qualifications.

6.4.6. Hand soaps: This provision only applies to those purchases that are in the control of the organization. Products in this group include:

- Industrial and institutional hand cleaners

- Hand cleaners and hand soaps

6.4.6.1. Criteria: A specified percentage of products in this group shall satisfy one of the following:

- Be free of antimicrobial agents (except as a preservative and unless required by health codes and other regulations) or,

- Meet the applicable Green Seal or Environmental Choice certification criteria or,

- Meet the requirements for recognition under the U.S. Environmental Protection Agency’s (EPA’s) Design for the Environmental Program (DfE).

Environmental Services uses antibacterial soap per client request but is piloting a Green

Seal certified soap in two of the large dorms on campus

6.4.6.2. Requirement: A minimum of sixty (60) percent, by purchase cost, of the products in this group provided by the organization and used in buildings to which this section applies (listed in Section 6.1.2.) shall meet the above criteria (6.4.5.1.).

Sodexo ES uses antibacterial soap per client request.

6.4.6. Hand soaps: This provision only applies to those purchases that are in the control of the organization. Products in this group include:

- Industrial and institutional hand cleaners

- Hand cleaners and hand soaps

6.5. Cleaning Equipment

6.5.1. Requirement: At least sixty (60) percent by either purchase cost or number of units of cleaning equipment provided by the organization in buildings to which this section applies (listed in Section 6.1.2) shall meet the following criteria:

6.5.1.1. Vacuum cleaners shall be:

6.5.1.1.1. Certified by the Carpet and Rug Institute’s (CRI’s) Seal of Approval/Green Label Vacuum Cleaner program.

6.5.1.1.2. Operate with a sound level of 70dBA or less.

Response: See table below.

VACUUM CLEANERS

Manufacturer			   Product			    CRI or Green Seal Ranking

| Windsor | Sensor XP12 | CRI Certified – Bronze |
| --- | --- | --- |
|  | Versamatic VS18 | CRI Certified – Silver |
| ProTeam | Super Coach Pro 10 | CRI Certified – Gold |
|  | Super Coach | CRI Certified – Gold |
|  | Super QuarterVac | CRI Certified – Gold |
|  | TailVac | CRI Certified – Bronze |

6.5.1.2. Carpet extraction equipment shall be certified by the Carpet Rug Institute’s (CRI’s) Seal of Approval/Green Label Deep-cleaning Extractor program.

Response: See table below.

INTERIM MAINTENANCE SYSTEMS

Manufacturer			            Product			    CRI or Green Seal Ranking

| Windsor | Taz Deluxe | CRI Certified |
| --- | --- | --- |
|  | iCapsol Mini | CRI Certified |

EXTRACTORS

Manufacturer			            Product			    CRI or Green Seal Ranking

| Windsor | Commodore 20 Extractor | CRI Certified – Platinum |
| --- | --- | --- |
|  | Cadet Extractor | CRI Certified – Platinum |
|  | Mini Pro Extractor | CRI Certified – Silver |

6.5.1.3. Powered floor maintenance equipment (including electric and battery-powered floor buffers and burnishers) shall be:

6.5.1.3.1. Equipped with vacuums, guard and/or other devices for capturing fine particles and,

6.5.1.3.2. Operate with a sound level of 70BA or less.

ES does not use any electric or battery-powered buffers or burnishers.

6.5.1.4. Propane-powered floor equipment shall:

6.5.1.4.1. Operate with a sound level of 90dBA of less, and

6.5.1.4.2. Have high-efficiency, low-emissions engines with catalytic converters and mufflers that meet the standards for the specific engine size as set forth by the U.S. Environmental Protection Agency (EPA) of the California Air Resources Board (CARB).

PROPANE-POWERED FLOOR EQUIPMENT

Manufacturer			            Product			            Decibel Rating

| Nilfisk Advance | Whirlamatic Pro | Less than 90 decibels |
| --- | --- | --- |

Response: Propane-powered floor equipment is only operated when buildings are unoccupied.

6.5.1.5. Automated scrubbing machines shall be:

6.5.1.5.1. Equipped with variable-speed feed pumps and,

6.5.1.5.2. On-board chemical metering, or

6.5.1.5.3. Use dilution control systems for refilling, or

6.5.1.5.4. Use only water with no added cleaning products

See table below.

FLOOR CARE MACHINES

Manufacturer			  Product			              Features

| Advance | Advenger Riding Autoscrubber | Dilution control refilling & metering |
| --- | --- | --- |
|  | Micromatic M17B Autoscrubber | Dilution control refilling & metering |
|  | Adfinity 17ST | Dilution control refilling |
| Clarke | Focus II Boost Autoscrubber | Dilution control refilling & metering |
|  | Focus II Boost 28 Autoscrubber | Dilution control refilling & metering |
|  | BOS-18 | Dilution control for refilling |

6.5.1.6. Battery-powered equipment shall be equipped with environmentally preferable batteries such as absorbed glass mat and gel cell batteries.

Most battery-power scrubbers use gel cell batteries, though some older do not have them.

6.5.1.7. Equipment shall be:

6.5.1.7.1. Ergonomically designed to minimize vibration, noise, and operation fatigue.

6.5.1.7.2. Designed with safeguards, such as rollers or rubber bumpers, to reduce potential damage to building surfaces.

All equipment is designed with safeguards, rubber bumpers, noise and vibration controls.

6.5.2. Equipment Inventory List: The organization shall maintain a list of all powered equipment that, at a minimum, includes:

6.5.2.1. Identification of each type of equipment by make and model

6.5.2.2. Number of pieces in use

6.5.2.3. Date purchase

6.5.2.4. Purchase cost

6.5.2.5. Applicability of relevant sustainability criteria (Sections 6.5.1. through 6.5.1.6.)

A complete inventory list is kept with all above items.

6.5.3. Equipment Maintenance Log: The organization shall maintain equipment as set forth in Section 2.2.d (Equipment Maintenance and Repair).

Equipment maintenance log is kept on all equipment listed above

6.6. Indoor Chemical and Pollutant Source Control

6.6.1. To the extent the organization provides entryway matting systems at the buildings listed in Section 6.1.2., the entryway matting systems (except of emergency exits) shall be:

6.6.1.1. Ten (10) feet long in the primary direction of travel, and

6.6.1.2. The organization shall have an appropriate cleaning plan for the matting systems in place.

The Facilities Dept. of the college provides matting for entrances, and most are of appropriate size.  Sodexo ES has a policy of cleaning matting as needed with deep cleaning once per week.

6.7. Integrated Pest Management (IPM) (*This element only applies to the extent that the organization delivers integrated pest management services and only with respect to those buildings listed in Section 6.1.1. at which those services are provided.  If the organization does not provide integrated pest management services, this element is not applicable).

6.7.1. The organization shall have an integrated pest management plan that addresses the management of pest populations inside the building.  Such plan shall include:

6.7.1.1. Integrated methods, inspections, pest monitoring, and evaluation of the need for pest control.

Sodexo ES is not responsible for pest management.

6.8. Building Exterior and Hardscape Management Plan (This element only applies to the extent that the organization is the primary provider of building exterior services and only with respect to those buildings listen in Section 6.1.2. at which building exterior services are provided.  If the organization does not provide building exterior services, this element is not applicable).

Sodexo ES is not responsible for building exterior management; this element is handled by the Colorado College facilities department.

6.9. Solid Waste Management (Recycling)

The organization shall have a waste reduction and waste management plan consistent with section 4.1.b. (Waste Disposal and Recycling Plan).  This element applies to the extent that the organization manages the waste stream.

See section 4.1.b.

Such plan may include waste stream audit procedures and a commitment to the diversion of waste through an active waste reduction and recycling program.  This element applies to the extent that the organization manages the waste stream.

Waste stream audit completed annually as part of the greenhouse gas inventory and AASHE STARS.  The audit is completed during a 13-week quarter from January-April each year.  For the past two years, Sodexo ES has removed bathroom paper for compost, and Colorado College won the annual Recyclemania tournament in 2015 in the Compost category.

Spot audits are occasionally done to ensure that the responsibilities of Sodexo ES are done per policy.

Recycling and compost is removed daily from all campus buildings on a routine scheduled basis.

6.10. Resource Conservation

The organization may have a written plan that addresses its efforts to conserve resources including cleaning materials, water, energy and transportation used during the cleaning process.

Sodexo has a program named SMART (Sustainability Management And Reporting Tool), which helps unit build a comprehensive sustainability management plan.  Data is entered into the tool every quarter.  The plan addresses sustainability efforts for all areas of the facility, though Sodexo ES is only responsible for the area of Environmental Services.

Sodexo ES at Colorado College also participates in WasteLess Week, a program created by Sodexo to educate students and staff about sustainability issues and food waste.