# Mapping Lehigh University’s Campus Metabolism

Maria S. Cohen

ESE 460 – Energy Systems Engineering Project

# Table of Contents

Table of Contents	2

List of Figures	4

List of Tables	4

Executive Summary	5

Introduction	5

Methodology	6

Discussion	7

Inputs	7

Outputs	8

Results	8

Energy	9

Description:	9

Energy Flow Diagram:	11

Electricity and Natural Gas Usage	13

Food	17

Description	17

Food Flow Diagram	20

Food Usage	21

Materials	21

Description	21

Materials Flow Diagram:	23

Materials Usage	23

Water	24

Description	24

Water Flow Diagram:	26

Water Usage	26

Recommendations and Future Work	31

References	33

Acknowledgements	34

Appendix A	35

2017 Electricity and Gas Data	35

Appendix B	36

Dining Options at Lehigh University	36

Resident Dining :	36

Lehigh Concessions:	36

Lehigh Retail Dining:	37

Asa Packer Dining Room:	38

Catering:	38

Appendix C	39

Present Recycling and Disposal of Materials	39

Appendix D	42

2017 Water Data	42

Appendix E	43

Food Ordering Example	43

# List of Figures

Figure 1: Flow Overview	6

Figure 2: Energy Flow	12

Figure 3: Sankey Diagram for Total Energy Consumption at Lehigh University (GWh)	14

Figure 4: Sankey Diagram for Academic Buildings Electricity Consumption (MWh)	15

Figure 5: Top 13 Academic Buildings by kWh per sqft	16

Figure 6: Food Flow	20

Figure 7: Materials Flow	23

Figure 8: Water Flow	26

Figure 9: Sankey Diagram for Total Water Consumption	27

Figure 10: Sankey Diagram for Housing Water Consumption	29

Figure 11: Sankey Diagram for Academic Water Consumption	30

# List of Tables

Table 1: Transportation Vehicle Inventory	11

Table 2: Personnel Responsible for Energy at Lehigh University	13

Table 3: Top 20 Buildings by Electricity Consumption by kWh/sqft	16

Table 4: 2017 Electricity Consumption Top Consumers Summary Table	17

Table 5: Personnel Responsible for Food at Lehigh University	21

Table 6: 2017 Water Consumption Top Consumers Summary Table	30

# Executive Summary

Lehigh University’s Campus Sustainability Plan 2020 (Lehigh University, 2017) was adopted in May 2016. This plan outlines the sustainability goals the university has set to reach by 2020 for the following areas: Academics & Educational Experience, Campus & Community Engagement, Operations and Planning & Administration. This project would be applicable to the Operations section of the Plan.

To evaluate how the campus presently operates, we use the method of Campus Metabolism and Material Flow Analysis (MFA). This method allows us to identify the different inputs, outputs and the stakeholders at Lehigh. Campus Metabolism takes a systems approach to understanding the inflows, users, transformations, and outflows for an institution. This project analyzes the water, energy, food and materials subsystems at Lehigh University three campuses. We find that all four flows are very linear. Some way loops can be closed are: using an automatic composting system (connect food and materials flows), use waste oil for combined heat and power for smaller buildings, and use rain barrels and more rain gardens to close the water loop.

We also analyze the building consumption of energy and water.  This will help target efforts in efficiency on campus.  All buildings are first labelled using the Interactive Campus Map’s categories.  We then determine the total energy consumption by adding electricity and gas consumption per building. We find the top consumers by function are Academic buildings with 51.5% of total energy use on campus (49.9% of total electricity consumed on campus) and Residential Housing with 41% of total potable water consumption on campus.  In the analysis of building electricity consumption we categorize buildings by electricity per gross square foot but a better indicator of the efficiency of a building would be using electricity per gross square foot per person. For this analysis classroom size, frequency of classes and number and occupancy of offices for staff, faculty and graduate students would be needed. Additional recommendations are given at the end of the report.

# Introduction

Lehigh University’s Campus Sustainability Plan 2020 (Lehigh University, 2017) was adopted in May 2016. This plan outlines the sustainability goals the university has set itself in four main areas:

- Academics & Educational Experience

- Campus & Community Engagement

- Operations

- Planning & Administration

For each of these areas there are specific goals that Lehigh University expects to meet by 2020. To be able to reach these goals and improve the present performance, Lehigh should evaluate how is it operating now and then identify where the campus can be more efficient.

In this project, we will use the method of Campus Metabolism and Material Flow Analysis (MFA) to identify the different inputs, outputs and the stakeholders at Lehigh and then present results as graphic flows.

The concept of Campus Metabolism is derived from the urban metabolism method (Gay, 2017), which takes a systems approach to identify and evaluate the inflows, transformations, and outflows for an urban setting. This project will analyze the water, energy, food and materials subsystems and identify in what ways flows can be closed to increase efficiency and sustainability on each campus.

The identification of flows in and out of the system boundary is also known as the Material Flow Analysis. (Lopes Silva, et al., 2014). For our case, the system boundary is the entire Lehigh University campus. A schematic identifying the different components of this analysis is show in Figure 1

Energy

Food

Water

Materials

Lehigh Campus

INPUTS/INFLOWSO

OUTPUTS/OUTFLOWS

Solid Waste

Waste Water

Recycling

Emissions

System Boundary

Figure 1: Flow Overview

A quantitative analysis is also performed using data provided from Facilities Services. With this data we can evaluate the consumption of gas, electricity and water on campus.

# Methodology

We consider four systems: water, energy, food and materials. We identify for each system what are the inputs, outputs, in addition to who are the stakeholders within each system boundary. The inputs or inflows are the different resources consumed on campus for each system. The outputs are all outflows which can be materials such as solid waste or wastewater or chemical emissions such as carbon dioxide. Stakeholders are the responsible personnel for each flow.

To inventory the different inputs and outputs for each system, we interviewed personnel in charge of different portions of each flow for each subsystem. From these conversations, a list of stakeholders with contact information was collected and a diagram illustrating the different flows was drawn. Next, we quantified the gas, electricity and water consumption on campus from data provided by Facilities. Finally, we looked into and suggested where new loops can be closed with either a new technology or a modification of the present flow.

# Discussion

A strong sense of sustainability already exists in every department I have interviewed. Using low flow fixtures to save water, using sensors for efficient irrigation, using ground well water for irrigation of fields, adding an electric bus to the transportation fleet, using motion sensors for lighting in different locations, portioning food preparation, having a wide variety of recycling programs for many materials, etc. All these practices contribute to the sustainability on campus but there is always room for improvement.  The best way to identify where to direct efforts towards increasing efficiencies is by identifying the flows for each subsystem and quantifying the total inputs and outputs.

For each subsystem we have developed a schematic indicating flows between the inputs, system boundaries and outputs. It is evident in all four diagrams that the flows are very linear. A more detailed discussion for each subsystem is given in the Results section.

## Inputs

The inputs or inflows are the different resources consumed on campus for each system. Each subsystem has different inputs from the other.

Input data that is measured campuswide by Facilities is electricity, natural gas and potable water.  For the comparison of food input to output measurements in weight would be the most convenient metric. Unfortunately, even though there are records of food orders, there is no standard for the units for the different purchases.  For example, celery can be ordered by the half case or the pack, oranges by the count and grape tomatoes by the pint. Even the unit of a pack is not standard since a pack of celery would not be equivalent in weight to a pack of parsley. This makes it difficult to quantify the input weight for food. An example of a food order can be found in Appendix E.

Campuswide materials inputs is very difficult to quantify since each department orders its own supplies. There is no central information.

## Outputs

All the electricity that is delivered to Lehigh University is used 100% on campus. Even though there are emissions involved in the generation of the electricity by power plants these are not considered as an output on campus.

The outputs due to all other fuels used on campus (natural gas, propane, gasoline and diesel) are also emissions. These emissions are produced on campus and released to the environment.

As discussed in the Results section, the only present measurement of food output is the amount of recovered food that is donated and the two-day Zero Waste Challenge food waste measurements. This flow could be measured more accurately but it would involve more man power.

Materials outputs can be quantified buy consolidating information from all the different recycling streams, surplus items and waste.  With the new centralized Surplus program established spring 2018 the measurement of the first two streams should be more feasible.

One challenge in measuring materials waste is that it is not possible to separate all food waste from materials waste. Even if all dining locations at Lehigh U. were to measure their food waste, there are many food sources from off campus and the waste becomes part of Lehigh’s waste stream. This additional food waste would not be able to be quantified and therefore the percentage of materials from the inputs that becomes part of the waste stream cannot be correctly quantified.

# Results

The initial goals for this project were to identify the flows for the energy, water, food and materials subsystems at Lehigh campuses and make a list of stakeholders. These flows are shown in the next sections.  The purpose of identifying these flows was to help make Lehigh University more efficient and sustainable.

As we prepared these flows, it became apparent that quantifying as many flows as possible would give another perspective into where to direct efficiency efforts. Because of this, we have analyzed the usage of electricity, gas and water by function of the building and by individual building. The data for this analysis was provided by Mr. Doug Spengel, Associate Director, Utilities and Energy Management.

The data provided for the Calendar year 2017.  Having a full year of data gives a better picture of consumption on campus. The data was listed by campus (Asa Packer, Goodman or Mountaintop), building name, monthly and total consumption for the year.  All buildings were classified by the primary function each building using the classification given in the official interactive campus map on Lehigh University’s website (Lehigh University, 2018). These categories are: Academics, Administration, Athletics, Dining, Housing, Library, Parking and Student Life.

Sankey diagrams were created from this data using the free online Sankeymatic.com website.  A Sankey diagram gives a quick visualization of where and how much of a particular quantity is used in each branch. The thickness of each flow is proportional to the amount used.

In the next sections we will discuss each subsystem.

## Energy

### Description:

At Lehigh University, energy is used mainly in the form of electricity and natural gas.  There are other sources such as gasoline and diesel fuel for transportation, propane for the Fud Truck and two fireplaces and wood for the Pizza Cart.

Most of the heating on campus is provided by steam produced from the burning of natural gas delivered to campus by UGI Utilities. Some older buildings that are part now of Lehigh use electric baseboard heating or electric heat pumps.

Dining Services uses mostly electricity and natural gas used for cooking. The Fud Truck is the only eatery that uses propane and the 1865 House of Pi Pizza Cart uses wood. Both are part of Dining Services Retail Dining.

Most electricity on all campuses is provided from PPL Electric Utilities. Lehigh purchases power from a third party, and PPL delivers and bills for it.  Lehigh is connected to the grid via a 69KV line that is fed to a substation owned and maintained by the university. The voltage is then stepped down for distribution on campus.

There is one single meter for PPL but on campus buildings have individual “Shark® meters” submeters that were installed and are maintained by Lehigh University.

There is some photovoltaic generation on campus but it is not significant comparing to the overall consumption. The only building that has some photovoltaic generation is the Transportation building from any excess generated at the 4.08 kW Community Garden. It is made of 16 panels with each panel rated at 255W. This system was installed as part of a student project. Another smaller 510 W system is used for a Lehigh University sign lighting at the corner of College Drive & William Street.

The latest addition of solar energy are BigBelly solar trash and recycling compactors that are located at Farrington Square. These compactors are leased as a trial for a year. Since they are compactors, these bins can hold more trash so custodial services can cut down on times they need to be emptied. Another advantage of these bins is that there is an app that tells when it needs to be emptied making custodial services more efficient.

Three smaller systems exist on campus: one at a weather station at the Mountaintop campus, one that powers a seismometer in the woods area at Mountaintop campus and the third is a cell phone charging station at STEPS green area.

All the photovoltaic generation on campus is approximately 5kW. If we assume generation for 12 hours for each day year-round, the annual solar generation would be 43.2MWh. Comparing this generation to the overall campus annual electricity consumption of 63.5 GWh, solar generation is 0.03% of Lehigh’s consumption.

Energy is also used for transportation on campus.  Most of the fleet at Lehigh University uses gasoline and diesel. In 2016 the Transportation Services used 77.07kgals of unleaded gas and 39.70 kgals of diesel. According to the Alternative Fuels Data Center (US Department of Energy, 2014), gasoline with 10% ethanol has an energy content of 112,114-116,090 Btu/gal (average 114,102 Btu/gal) and low sulfur diesel has 119,550 Btu/gal. From these values we can find the total energy from fuels used in transportation was 14,250 MMBtu for the 2017 calendar year.

In addition to gas and diesel vehicles, there are a variety of electric vehicles on campus. There is a forty-passenger electric bus and an electric parking services car. There are two charging stations for these vehicles to charge.  The charging station for the bus is at Mountaintop and parking services uses a charging station at the Farrington Square parking lot. A third charging station has been added in 2018. This charging station is public and is located at the Alumni Building Garage. There are three other electric vehicles on campus, athletics has two flatbed carts and Concessions has one electric cart.

Below is a list of the Transportation Department inventory, indicating the type of fuel and number of passengers per vehicle.

| Type of Vehicle | Quantity | Fuel Type | Passengers |
| --- | --- | --- | --- |
| bus | 1 | gas | 21 |
| bus | 2 | diesel | 29 |
| bus | 4 | diesel | 48 |
| bus | 4 | diesel | 95 |
| bus | 1 | electric | 40 |
| car | 1 | electric | 5 |
| van | 4 | gas | 12 |
| van | 2 | diesel | 15 |
| Chevy Malibu | 16 | gas | 5 |
| mini van | 18 | gas | 7 |
| Chevy Traverse | 2 | gas | 8 |
| various work vans, trucks and service vehicles | 60 | gas |  |
| police vehicles | 11 | gas |  |
| ATV | 1 | gas |  |
| motorcycle | 1 | gas |  |
| trailers | 20 |  |  |

Table 1: Transportation Vehicle Inventory

### Energy Flow Diagram:

From communications with personnel in charge of different aspects of energy on campus Figure 2 and Table 2 were constructed.

Figure 2: Energy Flow

| Name | Phone | Email | Title | Office Address |
| --- | --- | --- | --- | --- |
| Doug Spengel | 610-758-3974 | dbs288@lehigh.edu | Associate Director, Utilities and Energy Management |  |
| Robert Bruneio | 610-758-4827 | rsb5@lehigh.edu | Manager, Transportation and Transit Services | 126 Goodman Drive |
| Don Pasda | 610-758-5198 | djp207@lehigh.edu | Director Building and Utility Systems | 461 Webster Street, Building 3 |
| Gary Falasca | 610-758-3972 | gf02@lehigh.edu | Director Custodial, Grounds Maintenance and Athletic Buildings | 3-461 Webster St. |
| Steve Schiegel | 610-758-5196 | ss0n@lehigh.edu | Energy Conservation Engineer | 3-461 Webster St. |

Table 2: Personnel Responsible for Energy at Lehigh University

### Electricity and Natural Gas Usage

The data Facilities has, specifies the electricity consumption by building using Lehigh’s submeters or PPl meters. We also have the campuswide natural gas consumption by building. For natural gas, there are a series of gas and steam meters. Some of the consumption by building of natural gas was inferred from the steam data. These data can be found in Appendix A.

As mentioned before, we consider there are no output flow from electricity. On the other hand, there are outflows due to the burning of natural gas.  Lehigh consumed 213,378 Mcf (1 Mcf=1000 cubic feet) of natural gas in 2017. This means 25.9 million pounds of carbon dioxide were emitted on campus. To find this total we used the following equivalences (Energy Information Administration, 2018):

- 117.0 lb of carbon dioxide is emitted per million British thermal units (MMBtu) of natural gas.

- 1,000 cubic feet (Mcf) = 1.037 MMBtu.

All emissions generated on campus are not captured, they are released directly to the environment.

#### Energy Sankey Diagrams

To give a view of how the energy is distributed throughout the campus we use Sankey Chart diagrams.  The first Sankey diagram shows the total energy flow on campus.  This includes data of electricity and natural gas consumption on campus.  Given the very small magnitude of energy consumption on campus due to solar photovoltaics, we did not consider it as part of this diagram.

To calculate the total energy, we had to find the total amount of energy in the 213,378 Mcf of natural gas that Lehigh consumed in 2017. To do this we used how much energy is stored per Mcf of natural gas using the following conversions:

1Mcf of natural gas = 1.037 million Btu (MMBtu)

1MMBtu=293.07 kWh.

From these calculations we obtained Figure 4 which shows that Academic buildings on campus are the greatest consumers of energy and Housing is the second. The difference between these two is significant. Academics uses 51.3% of the total energy consumed on campus meanwhile Housing uses 17.7%.

Figure 3: Sankey Diagram for Total Energy Consumption at Lehigh University (GWh)

When comparing campuses, we find that Asa Packer uses 79.8%, Mountaintop 16.2% and Goodman 4.1% of the total energy consumed at Lehigh University.

As seen from Figure 3, the Academic buildings used the most energy. These totals include gas and electricity consumption but they do mirror the electricity consumption.

Taking a closer look at the consumption of electricity by building in Academics (Figure 4), we notice that in general the large lab buildings are the greater consumers of electricity. In particular, STEPS is the largest consumer with 5,581MWh followed by Iacocca Hall with 4,593 MWh.

Figure 4: Sankey Diagram for Academic Buildings Electricity Consumption (MWh)

It is not surprising that labs would consume more electricity but given that STEPS is a Certified LEED Gold building, we decided a different approach and calculated the ratio of electricity consumption to gross square feet of each building.  From this distribution we tabulate the top 20 buildings on campus as shown in Table 3.

The top two consumers under this metric are Chillers. It is not surprising that these buildings have the highest rating since they provide the heating and cooling for many buildings on Asa Campus. The third building is the Ulrich Fields and Scoreboards (63.08 kWh/sqft). This result is also understandable given that the lights for these fields are connected to this meter and the Ulrich building is not large.

What is surprising is that Rathbone is ranked as the fourth top building with 54.64 kWh/sqft. As a comparison, Lamberton Hall consumes 18.55 kWh/sqft.

From this calculation we find that the STEPS building is still ranked as the top Academic building with 40.64 kWh/sqft and 6th overall.

Table 3: Top 20 Buildings by Electricity Consumption by kWh/sqft

If we look at the distribution of Academic buildings  by kWh/sqft we find the following distribution:

Figure 5: Top 13 Academic Buildings by kWh per sqft

We summarize some of the results in this section in the following table:

Table 4: 2017 Electricity Consumption Top Consumers Summary Table

2017 Electricity Consumption Top Consumers Summary Table

## Food

### Description

There are many dining options at Lehigh University:

- Resident Dining

- Concessions

- Retail Dining

- Catering

These are all under a service contract with Sodexo.  All the directors for each location is a Sodexo employee. Lehigh’s contact is:

| David Joseph | 610-758-3500 | dmj0@lehigh.edu | Executive Director Student Auxiliary Services |
| --- | --- | --- | --- |

Each dining hall orders their own food and the delivery frequency is going to depend on the needs of each location.  Any ordering is first from Sodexo’s approved list of vendors.  The main vendors are:

- Amoroso

- Balford Farms

- BK Enterprises

- Common Market

- J. Ambrogi Foods

- Lancaster Farm Fresh Co-Op

- Pepsi Cola

- Rockland Bakery

- Sysco

In any dining service there is going to be some food waste.  This will happen in the kitchen when preparing foods and in the dining hall after diners have eaten. Dining Services tries to minimize this waste in different ways. First, in the kitchen it prepares food in batches for the all you can eat dining halls. In addition, the dining halls do not use trays. This is to reduce students overloading with food when they are serving themselves. They still have the option to get more if they would like to.

The dining halls will also allow diners, by request, to taste a new food before putting a whole serving on their dish. In this way, if it is a new food and they prefer not to have more, they do not have a whole serving on their dish that would go to waste.

The food waste that does occur in the different dining options at Lehigh University will undergo a different process depending on location.

#### Enviropure® System:

Rathbone dining hall uses the Enviropure® System for treating food waste. This system has been in use for over a year.  Potable water continuously flows through a channel by the area where used dishes by customers are dropped off. Most organic or compostable remnants are dumped into the flow. Corn husks and beef fat are not placed in the Enviropure ® System. 
If there is a large supply of organic material from prepping or non-recoverable food, it is placed in a separate input for the same system. Food from dishes is scraped into a continuous water flow. This flows via pipes from the kitchen level to the lower level where the “digester” is located.  A biomix that will breakdown the organic material over a 24-hour period is also added periodically to this digester. This biomix contains minerals (copper, magnesium, potassium and zinc), nutrients (sulphate, TKN), aminoacids (ascorbic, benzoic and lipoic) and vitamins (B6/B12, C, E and K) (Enviropure Systems, 2013).

The output of the Enviropure ® System is a constant flow of grey water that is deposited directly into the sanitary sewer system. After checking with the manufacturer of the Enviropure ® System and also with Facilities, it is concluded that this grey water cannot be used in any other way and must be disposed of.

#### SOMAT system:

The SOMAT system is located below Lower Court at the University Center.  Anything that is prepared in the University Center will use this system. That is, if catering has an event or if a retail outlet is based out of the University Center, its organic waste will go through the SOMAT system.

Any compostable item can go through this system. For example, compostable napkins can be part of the organic waste treated by this system.  First, any organic material is grinded. It then flows to the SOMAT system where liquid from the food waste is extracted. This process reduces the volume and weight of waste being disposed. The extracted liquid to into the sanitary sewer system.  At the moment, the remnant solid organic waste is hauled off campus with any inorganic waste.

#### Food Recovery Network (FRN)

Lauren Sleeger, Director of Rathbone has implemented the Food Recovery Network (FRN) program for Rathbone and Lower Court. The chef at each location will determine after each serving cycle, what food can repurposed (for another meal), donated, or tossed. What can be donated will be collected and frozen at the end of the day. Once a week this food is donated to a local charity such as the New Bethany Ministry, Victory House, Second Harvest of Lehigh Valley or the Hispanic Center of Lehigh Valley.  From January to September 2017, 376.55 pounds of food has been donated.

The distribution and collection for recoverable food is done with the support of Lehigh student volunteers.  Lauren Sleeger would like to expand the program to include all dining outlets but more student volunteers would be needed.  It is a new program and there seems to be interest from students.

#### Vegetable Waste Oil

Another food output is vegetable waste oil. Most dining services at Lehigh University has some type of deep frying. All vegetable waste oil from all fryers is recycled.  This is a Sodexo wide mandate which is in line with Lehigh’s sustainability goals. The oil is picked up twice a week by the FiltaFry company.

All vegetable waste oil is filtered straight into the trucks that haul it away to be mostly recycled into biodiesel. In 2017, Lehigh reported that 15.99 tons of fryer grease were recycled. (Lehigh University, 2018)

### Food Flow Diagram

From the information gathered we have designed the food flow on campus. As shown in the diagram of Food Flow below, (Figure 6), there are a variety of dining options on campus. A complete list of the locations and person in charge and their contact information can be found in Appendix B.

In this section, we include a list of the different directors for the different dining areas on Lehigh’s campus and their contact information. This list can be found in Table 3.

Figure 6: Food Flow

| Name | Phone | Email | Title |  |
| --- | --- | --- | --- | --- |
| Bruce Christine | 610-758-4512 | brc305@lehigh.edu | General Manager-Dining Services (Student Aux. Services) |
| Claudine Sidney | 610-758-6225 | cms408@lehigh.edu | Director of Catering |
| Daniel Burke | 610-758-4991 | dab206@lehigh.edu | Concessions Director |
| David Joseph | 610-758-3500 | dmj0@lehigh.edu | Executive Director Student Auxiliary Services |
| Jason Lenig | (610) 758-4170 | jala@lehigh.edu | Director of Retail Dining |
| Lauren Sleeger | (610) 758-4169 | e00515@lehigh.edu | Director of Rathbone |
| Oumou Fall | (610) 758-6415 | e00216@lehigh.edu | Director of Hawk's Nest |
| Paul Rinaldi | (610) 758-5169 | par308@lehigh.edu | Director of Cort Lower UC |
| Treni Pion | (610)758-4168 | Trsa@lehigh.edu | Assistant Director of Catering |

Table 5: Personnel Responsible for Food at Lehigh University

### Food Usage

How much food is being consumed at each Lehigh Dining option is not an easy quantity to determine. Specific data would need to be measured.  At the moment, each facility does its own ordering and quantities vary in size and metric (as discussed in Inputs). There is no overall tracking and no standardized system or measurement.

How much food waste goes into the solid waste stream is also difficult to quantify. Each dining outlet tries to be sustainable but there is no metric. There is no monthly data to quantify progress or any issues.

This does not mean that Dining Services does not try to reduce waste.  As mention under Description, there are different methods by which Dining Services does try to reduce waste on its side. The one variable they have no control over is the customer.  To increase the awareness of how much food is wasted by the diner, Dining Services has zero waste challenges which are done twice a year at Rathbone and Lower Cort. During this time, all waste from all plates of diners is collected and weighed for a full week. During this time it was found that 13,949 pounds of food waste was generated at these two dining locations over a week.

## Materials

### Description

Materials consists of all physical objects that come on campus that are used on campus. An example of these types of objects would be paper, equipment, motor oil for vehicles, etc. There are also materials that are generated on campus like wood chips and leaves.

For materials that come on campus from a purchase there are two processes for them. Purchases that are over $10,000 need to go through the purchasing office. Other purchases are completed directly by each department. Any purchase on campus, big or small, should follow the Sustainable Purchasing Policy. There is a checklist that can be used to check if the items being purchased fulfill the social, ecologic, etc. sustainable parameters set forth by the policy. The following of this policy is by an Honors System. Purchasing personnel do not check if orders comply with the policy.

Lehigh University is a member of the Lehigh Valley Association of Independent Colleges (LVAIC).  This association is a consortium of 6 colleges and universities from the Lehigh Valley. (LVAIC, 2017). By pooling together, the consortium can negotiate more favorable contracts with different vendors. Each member decides which contracts they would like to participate in. As can be seen in the list below, some contracts are for materials inputs (e.g., paper or lab gasses) and others are for materials outputs (e.g., e-waste and hazardous waste disposal).

Some of the contracts through the LVAIC consortium that Lehigh participates with are:

- A1limo

- AERC: e-waste

- AIR Gas: lab gas

- Brightview Industries: landscaping

- CDW-G: IT

- Enterprise – car rental

- Hazardous waste disposal

- Thermo Fisher – Lab supplies

- WB Mason: copier/computer printer paper

- Canteen: vending machines

- MD Earl: janitorial

- Prestige: Janitorial

In addition to the recycling contracts through the LVAIC consortium, there are many other recycling streams for materials at Lehigh. A detailed list of the existing recycled items and which office is responsible for that stream is provided in Appendix C. There will likely be some changes in this list with the establishment of the new Surplus Program on campus. The goal of this new program is to repurpose, sell, donate or if needed appropriately destroy any surplus material on campus. This new system will also include surplus lab equipment, furniture, fleet vehicles, IT equipment, etc.  The plan is for there to be a retail location on campus where certain items like repurposed computers can be sold to members of campus. This new system will be headed by Glenn Strause. The new location for this new program was still to be determined as of the date Mr Strause was interviewed.

Any organic or inorganic waste that is not recyclable or is not a hazardous item is hauled for disposal. The same company that hauls this waste also picks up the single stream recycling. The waste from Lehigh does not go into a landfill but is taken to COVANTA, a Waste-to-Energy incinerator located in Conshohocken, PA.

### Materials Flow Diagram:

From the information gathered we have designed the food flow on campus included below:

Figure 7: Materials Flow

### Materials Usage

Because of all the multiple ways materials come onto campus, there is no information on the total input of materials. This is true even if we try to look at a particular material stream such as for example, paper. Regular copy paper that is used by Lehigh printers can be tallied since it is purchased from a single vendor through the LVAIC consortium. But there are other types of paper used like letter head paper and specialty printing paper for poster printing. These come from a different source.

There are also challenges in quantifying paper waste. Most paper is recycled as part of the Single Stream recycling program. This means it is gathered with recyclable plastic and cans. The university is given totals of recyclables but not by type of recyclable. In addition, it is impossible to quantify how much paper that is purchased by Lehigh U. is recycled on campus since some of the paper that ends up in the Single Stream recycling was brought onto campus from other sources (i.e. students or visitors).

On the other hand, Lehigh tries to maintain a detailed record of the recycling streams collected such as ink cartridges, oil filters and scrap metal among others. This helps compare the progress from one year to the next.

## Water

### Description

All the potable water that is used for drinking, cooking, etc. at Lehigh’s three campuses is from the City of Bethlehem. It is transported in 60-inch concrete pipes. To provide water on other side of ridge, the city pumps water at night to an elevated storage for use during the day.  This storage tank is on Mountaintop. Facilities and Grounds oversee the water system on all three campuses.

A distinctive feature at Lehigh University Asa Packard campus are its five outdoor water fountains located at Farrington Square, Packard Laboratory, Chandler-Ullmann Hall, STEPS and the Alumni Memorial Bldg. These fountains are feed from potable water.

Water is recycled when the fountain is in operation and water is refilled as needed since there is some loss due to evaporation.  During the late Fall and winter months the fountains are drained. Lehigh also has two swimming pools on campus. One is for competition and another for practice.  The water is monitored weekly or daily and is only drained for repairs.

Potable water is also used for irrigating the landscape at Packard Campus when needed.   This is mostly when new plants have been planted and need watering till the plant is established. Plants on campus must be a native species of low maintenance and if possible perennial. This helps reduce the need of using potable water for landscaping and reduces maintenance costs.

Landscape watering for Goodman Campus is done via wells. In addition, well water is used for irrigation for all the athletic and practice fields, and putting greens at the Golf Practice center. Well water is also used for the bathrooms at the Golf Practice center but it is not approved as drinking water. There are signs posted over the sinks informing users not to drink the water. Facilities indicated that, from their own tests they believe that the water could be certified as potable but it is a cumbersome process and the expected use would not justify the cost.

There are about half a dozen wells for this purpose. The irrigation systems for fields and grounds uses a SMART system. It measures rainfall and ground moisture to determine if irrigation is needed.  It is a spray system (not drip). Given that it is a large area of grass that is being maintained this is the most effective way to water the fields.

There are a couple rain gardens on campus that help manage stormwater. There is a small one at the side entrance of the STEPS building. There others at the tennis and baseball fields. In addition, according to Facilities, there are porous walkways for the softball field and Williams Hall. These are strategies to manage and reduce runoff water that would otherwise end in the Storm Sewer System.

Finally, STEPS has a couple unique features related to its stormwater management. The first one is comprised of two underground tanks. These tanks are approximately 5ft diameter x 60ft and are located south of Maginnes Hall, under the lawn. These tanks collect rain runoff, stores it and allowing for a slow release to the ground. The second feature is the green roof located on the south section of STEPS (along W. Packer Avenue). There is a smaller section over the entrance overhang that is located closest to Maginnes.

### Water Flow Diagram:

From the information gathered we have designed the food flow on campus as shown in  Figure 8

Figure 8: Water Flow

### Water Usage

All potable water used on campus comes from the City of Bethlehem Water Department. Every building on campus has a water meter (data in Appendix D).  This information allows us to quantify the total input of City Water for Lehigh University and analyze the consumption of water by campus and by function per building. For the output though, there are no measurements.  Facilities assumes that the output into the sanitary sewer system is the same as the input. This is an approximation since there will be some output in the form of evaporation in Heating and Cooling.

To our knowledge, there is no record of the precipitation on campus nor what percentage ends up being runoff. There appears to be a weather station at Mountaintop campus but we do not know if it keeps record of rainfall. It should be possible to estimate how much precipitation fell during 2017 on each campus from meteorological data. An estimation of percentage that becomes runoff due to impervious surfaces such as roofs and roads could be found. To our knowledge, there is no survey of runoff from landscaped surfaces. It should be noted though that Facilities is not aware of any problem due to runoff from vegetation areas.

There is also output of water from the non-paved surfaces due to evaporation, transpiration of vegetation and what is filtered into the ground water supply. It may be possible to calculate what these outputs are in average, but that is beyond the scope of this project.

#### Water Sankey Diagrams

Since we have the measurements from the water meters for all buildings on campus, we build a Sankey Diagram for total water consumption (Figure 9), the largest consumer of water by function on campus is Housing (41%). This is not surprising since showers and laundry are water intensive activities. Second set of buildings are Academics (25%) with Administration not far behind (20%).

Figure 9: Sankey Diagram for Total Water Consumption

If we look at the Sankey Diagram for Housing water consumption (Figure 10). Given the total consumption per building and comparing to the total consumption campuswide we find that the two largest consumers are Trembley Park (11.4% of Housing) and Saucon Village (10.4% of Housing). These are both residential complexes with various units. Saucon Village housing complex is formed by five houses: Diamond, Gipson, Severs, Hartman and More.

It is notable that the third largest water consumer is Alpha Sigma Phi (9.1% of Housing). This consumption is significant because it is of the same order of magnitude of Saucon Village. In addition, the water consumption of this fraternity is more than the residential Halls: Brodhead House (6.6% of Housing) and McClintic-Marchall House (6.6% of Housing).

We do not know how many people occupy any of these buildings so it may be possible that they all have comparable water consumption per capita. This is an analysis that would be important to do. Housing numbers vary per academic year but it is likely that Residential Services would have the totals for 2017. If consumption per capita is calculated for each Housing building on campus, this would help in identifying buildings that may have had plumbing issues or if the increased consumption is due to behavior. Facilities may have records in there were plumbing issues that were addressed during the 2017 calendar year. If there are any outlier buildings with higher than average water consumption per capita, then education efforts can be targeted to the occupants of these buildings.

Figure 10: Sankey Diagram for Housing Water Consumption

We also look at the distribution in water consumption for Academic buildings (Figure 11). It is interesting that the largest water consumer is the Sherman Fairchild Laboratory (4.8 %), combining Iacocca North and South buildings (4.7%) and finally the third building is STEPS with (3.5%).

Figure 11: Sankey Diagram for Academic Water Consumption

Again, we do not know what is the occupancy or average occupancy of these buildings. Occupants would have a significant impact on the total water consumption for each building. In these buildings we have permanent occupants (faculty and staff) and variable occupants (students and visitors). The challenge in determining an approximate per capita water consumption for academic buildings is the variation, in number and type, of these populations by semester and by day.

We summarize some of the results in this section in the following table:

| Category | Building or Function | Qualifier |
| --- | --- | --- |
| Most water by function | Residential Housing | 41% of total |
| Top Residential Housing | Trembley Park | 11.4% of Housing |
| Top Academic Building | Sherman Fairchild Lab | 19% of Academic |

Table 6: 2017 Water Consumption Top Consumers Summary Table

# Recommendations and Future Work

- Study the feasibility and economics of installing a composting system or biodigestor to capture nutrients from the food, reduce water usage (if used instead of the Enviropure® System) and serve as an educational tool. There exist some commercial automated versions of both types of systems that are installed at locations such as, Bergen Community College, NJ; Honesdale High School, PA; Purchase College, SUNY; Princeton University, NJ; Columbia University, NY; etc.

- Study the LCA and carbon footprint of implementing an automatic composter such as the Rocket Composter from Food Waste Experts (or other similar system) versus using the Enviropure ® system.

There are plans of installing an Enviropure ® system in the University Center when it is remodeled. It is clear from interviews, that Dining Services prefers using this system because it reduces contact of personnel with food and it streamlines the disposal of food waste. The reduced contact reduces health risks to the food handling personnel. It is also clear from the electricity per square foot data, that Rathbone has a significant electricity consumption. A water consumption for dining is also high.  There is only data for the last four months for 2017 but if we take the average of these four months (417 kgal) and calculate the estimated consumption over 10.5 months we get 4,374kgals of potable water used which is more than Mountaintop’s Central Heating and Refrigeration water consumption and over 700 kgal more than STEPS water consumption.

- Analyze water consumption per building by population (number of students scheduled, staff and faculty offices). A similar analysis was done by Abdelalim et. al for Carleton University, Canada. This would give a better picture of the efficiency of each building.

Due to how water is used, an analysis based on the square footage of the buildings would not be a useful calculation.

- Analyze energy consumption per building by population and gross square footage. This would give a better picture of the energy efficiency of each building.

Heating and electricity is used throughout each building but some buildings may have a higher population density which may contribute to a higher total consumption.

- After doing the density analysis suggested in recommendation 3 and 4, identify what are the larger sources of water and energy consumption for highest consuming buildings.

- Porous walkways and porous pavement could be used more widely. This may reduce water output to the Storm Sewer System and would also reduce icing during the winter. Further research is needed to determine where this technology would be most applicable.

- Expand Food Recovery Network reach. Need more volunteers.

- Use a social media platform to spread the word of “available” food on campus after events (pilot presently being tested)

- Use collected rain water at STEPS as grey water instead of present slow release to Storm Sewer System.

- Follow up on effectiveness new Surplus Program.

# References

Abdelalim, A., O'Brien, W., & Shi, Z. (2015). Visualization of energy and water consumption and GHG emissions: A case study of a Canadian University Campus. Energy and Buildings, 334-352.

Energy Information Administration. (2018, April). eia.gov. Retrieved from https://www.eia.gov/tools/faqs/faq.php?id=73&t=11,

Enviropure Systems. (2013). Biomix Slides.

Gay, E. T. (2017, March). Undestanding Campus Metabolism: Transformative Sustainability at Furman University. Bachellor of Science Thesis. Furman University Department of Earth and Environmental Sciences.

Lehigh University. (2017). Lehigh University’s Campus Sustainability Plan 2020. Retrieved from Sustainability at Lehigh: https://sustainability.lehigh.edu/sites/sustainability.lehigh.edu/files/Campus%20Sustainability%20Plan%202020_0.pdf

Lehigh University. (2018, April). Data Tracking Master Sheet.

Lehigh University. (2018, April). Lehigh University. Retrieved from Lehigh University Campus Map: https://www.lehigh.edu/campusmap/

Lopes Silva, D. A., de Oliveira, J. A., Saavedra, Y. M., Ometto, A. R., Rieradevall i Pons, J., & Gabarrell Durany, X. (2014). Combined MFA adn LCA approach to evaluate the metobolism of service polygons: A case study on a university campus. Resources, Conservation and Recycling, 157-168.

LVAIC. (2017, 12 2). LVAIC. Retrieved from About: http://lvaic.org/about/

US Department of Energy. (2014, October 10). Energy Efficiency & Renewable Energy. Retrieved from Alternative Fuels Data Center Web site: https://www.afdc.energy.gov/fuels/fuel_comparison_chart.pdf

# Acknowledgements

The gathering of the information presented would not have been possible without the collaboration from:

- Jane Altemose, Director of Purchasing

- Robert Bruneio, Manager of Transportation & Parking Services

- Daniel Burke, Concessions Director

- Gary Falasca, Director Custodial Grounds & Athletic Buildings

- Jason Gall, Associate Director of Athletic

- David Joseph, Executive Dir. Student Auxiliary Services

- Don Pasda, Director Building and Utility Systems

- Steve Schlegel, Energy Conservation Engineer

- Lauren Sleeger, Director of Rathbone

- Doug Spengel, Associate Director, Utilities and Energy Management

- Glenn Strause, Surplus Program Manager

- Katharine Targett, Sustainability Program Manager

My participation in the Energy Systems Engineering program would not have been possible without the unwavering support of my husband, Robert Cohen and the encouragement from my children Joshua, Ana and Carina.

# Appendix A

## 2017 Electricity and Gas Data by Campus and by Building

|  | MWh |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Asa Packer | 51207.31 |  |  |  |  |
| Goodman | 4041.253 |  |  |  |  |
| Mountaintop | 13745.87 |  |  |  |  |
| Three-Campus Total | 68994.43 |  |  |  |  |

| Facility Name | Building Number | Campus | Purpose | Meter Type | CY17 Total (kWh) |
| --- | --- | --- | --- | --- | --- |
| STEPS | 9A | Asa Packer | Academic | Submeter | 5,581,450 |
| Packard Laboratory | 19 | Asa Packer | Academic | Submeter | 4,184,620 |
| Mudd | 6 | Asa Packer | Academic | Submeter | 1,943,838 |
| Whitaker Lab | 5 | Asa Packer | Academic | Submeter | 1,921,278 |
| ZAC (minus garage) | 49 | Asa Packer | Academic | Submeter | 1,579,375 |
| Rauch Business Ctr | 37 | Asa Packer | Academic | Submeter | 1,550,309 |
| Sinclair Opto | 7 | Asa Packer | Academic | Submeter | 1,062,708 |
| Williams Hall | 31 | Asa Packer | Academic | Submeter | 799,939 |
| Maginnes Hall | 9 | Asa Packer | Academic | Submeter | 782,013 |
| Mohler Lab | 200 | Asa Packer | Academic | PPL | 770,000 |
| Sinclair Lab | 7 | Asa Packer | Academic | Submeter | 746,541 |
| Sherman Fairchild Lab | 16A | Asa Packer | Academic | Submeter | 714,013 |
| Physics (ex Fairchild) | 16 | Asa Packer | Academic | Submeter | 559,176 |
| Chandler-Ullmann Hall | 17 | Asa Packer | Academic | Submeter | 463,200 |
| RBC Chiller | 37 | Asa Packer | Academic | Submeter | 450,173 |
| Wilbur Powerhouse | 12 | Asa Packer | Academic | Submeter | 314,747 |
| Fritz Laboratory | 13 | Asa Packer | Academic | Submeter | 272,416 |
| Coppee Hall | 33 | Asa Packer | Academic | Submeter | 250,502 |
| Christmas-Saucon Hall | 14 | Asa Packer | Academic | Submeter | 205,026 |
| Neville Hall | 6A | Asa Packer | Academic | Submeter | 193,610 |
| Drown Hall | 35 | Asa Packer | Academic | Submeter | 113,501 |
| Coxe Lab | 32 | Asa Packer | Academic | Submeter | 87,499 |
| Price Hall (sub Grace) | 40 | Asa Packer | Academic | Submeter | 28,135 |
| Philosophy Building | 15 | Asa Packer | Academic | Submeter | 12,311 |
| CH&R 1500T Chiller | 10 | Asa Packer | Admin | Submeter | 2,357,343 |
| CH&R Chilled Water | 10 | Asa Packer | Admin | Submeter | 1,065,204 |
| CH&R 750T Chiller | 10 | Asa Packer | Admin | Submeter | 477,477 |
| Alumni Memorial Building | 27 | Asa Packer | Admin | Submeter | 424,483 |
| Police | 47 | Asa Packer | Admin | Submeter | 262,541 |
| CH&R | 10 | Asa Packer | Admin | Submeter | 254,445 |
| Johnson Hall | 36 | Asa Packer | Admin | Submeter | 196,715 |
| Human Resources | 188/189 | Asa Packer | Admin | PPL | 122,522 |
| Research & Sponsored Programs | 23B | Asa Packer | Admin | PPL | 85,830 |
| Distance Education & Summer Sessions | 187 | Asa Packer | Admin | PPL | 35,722 |
| Post Office (sub FSq) | 202 | Asa Packer | Admin | Submeter | 33,613 |
| Facilities Services and Campus Planning | 3/3A | Asa Packer | Admin | PPL | 31,409 |
| Risk Management Office/Internal Audit | 197 | Asa Packer | Admin | PPL | 26,602 |
| Access Control Building | 42 | Asa Packer | Admin | Submeter | 25,743 |
| Facilities Services and Campus Planning | 3/3A | Asa Packer | Admin | PPL | 25,279 |
| Zoellner Arts Center Administration Building | 4 | Asa Packer | Admin | PPL | 21,729 |
| Parking Office | 199 | Asa Packer | Admin | PPL | 20,595 |
| Environmental Health and Safety | 23A | Asa Packer | Admin | PPL | 17,463 |
| 416 E. Fith St Rectory | 316 | Asa Packer | Admin | PPL | 14,139 |
| Academic Diversity/Outreach | 198 | Asa Packer | Admin | PPL | 13,213 |
| Humanities Center | 224 | Asa Packer | Admin | PPL | 8,605 |
| 219 Warren Square | 195 | Asa Packer | Admin | PPL | 7,747 |
| Taylor Gym | 38 | Asa Packer | Athletics | Submeter | 935,567 |
| Grace Hall (minus 3) | 39 | Asa Packer | Athletics | Submeter | 774,749 |
| Grace Chiller (NOT a sub) | 39 | Asa Packer | Athletics | Submeter | 271,986 |
| Grace CC (sub Grace) | 39 | Asa Packer | Athletics | Submeter | 18,000 |
| Rathbone Dining - 82% | 63 | Asa Packer | Dining | Submeter | 1,296,766 |
| UC Dining - 60% | 29 | Asa Packer | Dining | Submeter | 1,103,550 |
| Lamberton Hall | 34 | Asa Packer | Dining | Submeter | 448,544 |
| Brodhead Dining | 201 | Asa Packer | Dining | Submeter | 130,677 |
| Linderman Library | 30 | Asa Packer | Library | Submeter | 1,488,063 |
| E.W. Fairchild-Martindale Computer Ctr | 8 | Asa Packer | Library | Submeter | 1,445,825 |
| E.W. Fairchild-Martindale Library | 8 | Asa Packer | Library | Submeter | 965,061 |
| E.W. Fairchild-Martindale Library | 8 | Asa Packer | Library | Submeter | 593,891 |
| UC - 40% | 29 | Asa Packer | Student Life | Submeter | 761,260 |
| Packer Memorial Church | 18 | Asa Packer | Student Life | Submeter | 71,896 |
| Dialogue Ctr (sub Grace) | 41 | Asa Packer | Student Life | Submeter | 14,384 |
| 215 Summit St | 196A | Asa Packer | Housing | PPL | 20,882 |
| Alpha Chi Omega 93A | 93 | Asa Packer | Housing | Submeter | 133,225 |
| Alpha Gamma Delta 95A | 95 | Asa Packer | Housing | Submeter | 87,838 |
| Alpha Omicron Pi 107A | 107 | Asa Packer | Housing | Submeter | 134,581 |
| Alpha Phi 98 | 98 | Asa Packer | Housing | Submeter | 80,979 |
| Alpha Tau Omega 99 | 99 | Asa Packer | Housing | Submeter | 90,838 |
| Brodhead (excluding dining) | 201 | Asa Packer | Housing | Submeter | 928,504 |
| Centennial I Buildings | 56-62 | Asa Packer | Housing | Submeter | 358,431 |
| Centennial II Buildings | 50-55 | Asa Packer | Housing | Submeter | 457,879 |
| Chi Phi 105 | 105 | Asa Packer | Housing | Submeter | 78,815 |
| Chi Psi 84 | 84 | Asa Packer | Housing | Submeter | 111,629 |
| Delta Chi 86 | 86 | Asa Packer | Housing | Submeter | 75,967 |
| Delta Upsilon 81 | 81 | Asa Packer | Housing | Submeter | 302,209 |
| Dravo House | 65 | Asa Packer | Housing | Submeter | 274,520 |
| Drinker House | 64 | Asa Packer | Housing | Submeter | 149,093 |
| Farrington Square | 202 | Asa Packer | Housing | Submeter | 1,210,623 |
| Gamma Phi Beta 100 | 100 | Asa Packer | Housing | Submeter | 82,264 |
| House 104 | 104 | Asa Packer | Housing | Submeter | 68,806 |
| Kappa Alpha 85 | 85 | Asa Packer | Housing | Submeter | 104,048 |
| Kappa Alpha Theta 106A | 106 | Asa Packer | Housing | Submeter | 65,050 |
| Kappa Delta 82 | 82 | Asa Packer | Housing | Submeter | 254,270 |
| Kappa Sigma 87 | 87 | Asa Packer | Housing | Submeter | 314,691 |
| M&M Chiller 1 | 67 | Asa Packer | Housing | Submeter | 42,103 |
| M&M Meter 1 (OLD) | 67 | Asa Packer | Housing | Submeter | 203,533 |
| M&M Meter 2 (NEW) | 67 | Asa Packer | Housing | Submeter | 136,488 |
| Packer House | 20 | Asa Packer | Housing | PPL | 149,680 |
| Phi Delta Theta 97 | 97 | Asa Packer | Housing | Submeter | 88,372 |
| Phi Kappa Theta 88 | 88 | Asa Packer | Housing | Submeter | 86,299 |
| Phi Sigma Kappa 90 | 90 | Asa Packer | Housing | Submeter | 84,536 |
| Pi Beta Phi 83A | 83 | Asa Packer | Housing | Submeter | 101,361 |
| Pi Kappa Alpha 89 | 89 | Asa Packer | Housing | Submeter | 87,437 |
| Psi Upsilon 80 | 80 | Asa Packer | Housing | Submeter | 68,705 |
| Rathbone - 18% | 63 | Asa Packer | Admin | Submeter | 284,656 |
| Richards House | 66 | Asa Packer | Housing | Submeter | 146,600 |
| Sayre Park Village A-D |  | Asa Packer | Housing | Submeter |  |
| Sayre Park Village A | 109A | Asa Packer | Housing | Submeter | 135,609 |
| Sayre Park Village B | 109B | Asa Packer | Housing | Submeter | 155,685 |
| Sayre Park Village C | 109C | Asa Packer | Housing | Submeter | 198,810 |
| Sayre Park Village D | 109D | Asa Packer | Housing | Submeter | 113,024 |
| Sigma Chi 102 | 102 | Asa Packer | Housing | Submeter | 74,700 |
| Sigma Phi Epsilon 92 | 92 | Asa Packer | Housing | Submeter | 90,728 |
| Taylor House | 68 | Asa Packer | Housing | Submeter | 480,455 |
| Theta Chi 91 | 91 | Asa Packer | Housing | Submeter | 81,246 |
| Theta Xi 96 | 96 | Asa Packer | Housing | Submeter | 69,417 |
| Trembley Park A - G |  | Asa Packer | Housing | Submeter |  |
| Trembley Park A - 71 | 71 | Asa Packer | Housing | Submeter | 155,477 |
| Trembley Park B - 72 | 72 | Asa Packer | Housing | Submeter | 141,319 |
| Trembley Park C - 73 | 73 | Asa Packer | Housing | Submeter | 113,348 |
| Trembley Park D - 74 | 74 | Asa Packer | Housing | Submeter | 120,965 |
| Trembley Park E - 75 | 75 | Asa Packer | Housing | Submeter | 185,601 |
| Trembley Park F - 76 | 76 | Asa Packer | Housing | Submeter | 149,604 |
| Trembley Park G - 77 | 77 | Asa Packer | Housing | Submeter | 133,147 |
| Umoja House 101 | 101 | Asa Packer | Housing | Submeter | 143,782 |
| Warren Square A-F |  | Asa Packer | Housing | PPL |  |
| Warren Square A | 196 | Asa Packer | Housing | PPL | 43,656 |
| Warren Square B | 192 | Asa Packer | Housing | PPL | 22,354 |
| Warren Square C | 24 | Asa Packer | Housing | PPL | 49,713 |
| Warren Square D - Eco House | 193 | Asa Packer | Housing | PPL | 45,609 |
| Warren Square E | 25 | Asa Packer | Housing | PPL | 48,679 |
| Warren Square F | 21 | Asa Packer | Housing | PPL | 49,133 |
| West Packer House | 22 | Asa Packer | Housing | PPL | 10,549 |
| Zeta Tau Alpha 94 | 94 | Asa Packer | Housing | Submeter | 102,136 |
| 416 East 5th St Rear Garage | 316 | Asa Packer | Parking | PPL | 6,567 |
| Parking Lot Lights |  |  | Parking | PPL |  |
| Parking Lot Lights | None | Asa Packer | Parking | PPL | 2,850 |
| Parking Lot Lights | None | Asa Packer | Parking | PPL | 933 |
| Parking Lot Lights | None | Asa Packer | Parking | PPL | 171 |
| Parking Lot Lights | 92 | Asa Packer | Parking | PPL | 1,861 |
| Parking Lot Lights | None | Asa Packer | Parking | PPL | 607 |
| Alumni Parking Garage | 26A | Asa Packer | Parking | Submeter | 169,036 |
| Farrington Sq Garage | 2 | Asa Packer | Parking | Submeter | 219,916 |
| ZAC Garage #48 | 48 | Asa Packer | Parking | Submeter | 226,560 |
| Johnny's (sub FSq) | 202 | Asa Packer | Retail | Submeter | 174,894 |
| The Cup (sub FSq) | 202 | Asa Packer | Retail | Submeter | 109,239 |
| Service Building | 1 | Asa Packer | Vacant | PPL | 80,000 |
| Brickman Building | 127 | Goodman | Admin | Submeter | 35,175 |
| Former Ben Franklin | 125 | Goodman | Admin | PPL | 328,800 |
| Transportation Building | 126 | Goodman | Admin | PPL | 66,623 |
| Baseball | 129 | Goodman | Athletics | Submeter | 14,014 |
| Cundey Varsity House | 121 | Goodman | Athletics | Submeter | 675,506 |
| Golf Training Center | 149 | Goodman | Athletics | PPL | 16,383 |
| Goodman Stadium | 150 | Goodman | Athletics | PPL | 436,200 |
| Rauch Field House | 123 | Goodman | Athletics | Submeter | 403,134 |
| Stabler A&CC | 124 | Goodman | Athletics | Submeter | 1,254,616 |
| Tennis Courts | 128 | Goodman | Athletics | Submeter | 148,726 |
| Track Building | 115a | Goodman | Athletics | Submeter | 9,481 |
| Ulrich Flds+Scrbrd | 151 | Goodman | Athletics | Submeter | 326,749 |
| Saucon Village Houses |  | Goodman | Housing | PPL |  |
| Saucon Village Diamond House |  | Goodman | Housing | PPL |  |
| Saucon Village Diamond House | 143 | Goodman | Housing | PPL | 19,350 |
| Saucon Village Diamond House | 143 | Goodman | Housing | PPL | 20,632 |
| Saucon Village Diamond House | 143 | Goodman | Housing | PPL | 17,650 |
| Saucon Village Gipson House |  | Goodman | Housing | PPL |  |
| Saucon Village Gipson House | 140 | Goodman | Housing | PPL | 15,634 |
| Saucon Village Gipson House | 140 | Goodman | Housing | PPL | 2,322 |
| Saucon Village Gipson House | 140 | Goodman | Housing | PPL | 16,742 |
| Saucon Village Gipson House | 140 | Goodman | Housing | PPL | 21,555 |
| Saucon Village Gipson House | 140 | Goodman | Housing | PPL | 13,746 |
| Saucon Village Hartman House |  | Goodman | Housing | PPL |  |
| Saucon Village Hartman House | 137 | Goodman | Housing | PPL | 9,244 |
| Saucon Village Hartman House | 137 | Goodman | Housing | PPL | 7,847 |
| Saucon Village Hartman House | 137 | Goodman | Housing | PPL | 15,500 |
| Saucon Village Hartman House | 137 | Goodman | Housing | PPL | 25,005 |
| Saucon Village Hartman House | 137 | Goodman | Housing | PPL | 14,800 |
| Saucon Village More House |  | Goodman | Housing | PPL |  |
| Saucon Village More House | 134 | Goodman | Housing | PPL | 18,179 |
| Saucon Village More House | 134 | Goodman | Housing | PPL | 14,683 |
| Saucon Village More House | 134 | Goodman | Housing | PPL | 18,183 |
| Saucon Village Severs House |  | Goodman | Housing | PPL |  |
| Saucon Village Severs House | 131 | Goodman | Housing | PPL | 26,440 |
| Saucon Village Severs House | 131 | Goodman | Housing | PPL | 25,071 |
| Saucon Village Severs House | 131 | Goodman | Housing | PPL | 21,622 |
| Parking Lot Lights | None | Goodman | Parking | PPL | 599 |
| Benzak House | None | Goodman | Vacant | PPL | 1,043 |
| Building A-Iacocca Hall | 111 | Mountaintop | Academic | Submeter | 4,592,723 |
| Building C-Mountaintop Initiative & Data X | 113 | Mountaintop | Academic | Submeter | 752,655 |
| Building H -Imbt Laboratories | 117 | Mountaintop | Academic | Submeter | 2,184,847 |
| Building J- Mailing & Printing | 118 | Mountaintop | Admin | Submeter | 465,560 |
| 1515 Mountain Drive North (WLTV) | 225 | Mountaintop | Admin | PPL | 223,440 |
| Building B- Warehouse | 112 | Mountaintop | Admin | Submeter | 118,961 |
| Building D - Central Heating & Refrigeration | 114 | Mountaintop | Admin | Submeter | 312,677 |
| Building D-CW | 114 | Mountaintop | Admin | Submeter | 1,891,971 |
| Building F-Jordan Hall | 115 | Mountaintop | Admin | Submeter | 141,054 |
| Gatehouse | 119 | Mountaintop | Admin | Submeter | 22,488 |
| Sayre Field Salt Shed | 110 | Mountaintop | Admin | PPL | 3,582 |
| Ben Franklin TechVentures | 116 | Mountaintop | Non Lehigh | Submeter | 3,035,911 |
| Meter With Outlets on Pole/Rd to Bethlehem Star | None | Mountaintop | RE: Vacant | PPL | - |
| Boat House | None | Off | Athletics | PPL | 1,239 |

# Appendix B

## Dining Options at Lehigh University

### Resident Dining :

| Name | Contact/Responsible Person |
| --- | --- |
| - Rathbone Hall (63 University Dr) Two Star Green Restaurant | Lauren Sleeger Director (610)758-4169 Chef Joseph (Joe) Kornafel: jok207@lehigh.edu |
| - Brodhead Dining Hall (in Brodhead Residential Hall) | Lauren Sleeger Director (610)758-4169 Chef Joe Kornafel: jok207@lehigh.edu |
| - Cort @ Lower U.C. (1st floor U. C.) | Paul Rinaldi Director (610)758-5169 Chef Ross Wilson |

### Lehigh Concessions:

Director: Dan Burke (610)758-4991

| Name | Contact/Responsible Person |
| --- | --- |
| - “Lehigh Smokehouse’’ | Made at Rathbone |
| - Zoellner: candy and soda | Treni Pion, Assistant Director of Catering (610)758-4168 |
| - Hot Dog Cart: | Dan Burke, Concessions Director (610) 758-4991 |
| - Luigi’s Italian Ice Cart | Dan Burke, Concessions Director (610) 758-4991 |
| - Stabler Arena: | Dan Burke, Concessions Director (610) 758-4991 |
| - Goodman Stadium: | Dan Burke, Concessions Director (610) 758-4991 |
| - Nutty Bavarian | Dan Burke, Concessions Director (610) 758-4991 |

### Lehigh Retail Dining:

Director: Jason Lenig (610) 758-4170

| Name | Contact/Responsible Person |
| --- | --- |
| - Baker’s Junction (2nd fl UC) | Jason Lenig, Director of Retail Dining (610) 758-4170 |
| - Global Café (2nd fl William’s Hall) | Lauren Sleeger (610)758-4169 Chef Tim Leger (til205@lehigh.edu) |
| - Pandinis (2nd fl UC) | Jason Lenig, Director of Retail Dining (610) 758-4170 |
| - Iacocca Café (Iacocca Hall, Mountaintop Campus) | Treni Pion, Assistant Director of Catering (610)758-4168 |

| - Upper U.C. (2nd fl UC) Houses 6 specialty shops |  |
| --- | --- |
| - Flame | Jason Lenig, Director of Retail Dining (610) 758-4170 |
| - Salsa Rico | Jason Lenig, Director of Retail Dining (610) 758-4170 |
| - Simply to Go | Jason Lenig, Director of Retail Dining (610) 758-4170 |
| - Mein Bowl | Jason Lenig, Director of Retail Dining (610) 758-4170 |
| - Cyclone Salads | Jason Lenig, Director of Retail Dining (610) 758-4170 |
| - Hissho Sushi | Jason Lenig, Director of Retail Dining (610) 758-4170 |

| Continue Retail Dining |  |
| --- | --- |
| - The Hawk’s Nest (Lamberton Hall Student Center) | Director: Oumou Fall (610) 758-6415 |
| - Common Grounds (2nd fl Rauch Business Center) | Lauren Sleeger (610)758-4169 Chef Tim Leger (til205@lehigh.edu) |
| - Fud Truk : fresh cut fries from local PA farms (info from website) | Dan Burke, Concessions Director (610) 758-4991 |
| - Lucy’s Café (Lower Level Linderman Library) | Jason Lenig, Director of Retail Dining (610) 758-4170 |
| - 1865 House of Pi (Mobile pizza cart) | Dan Burke, Concessions Director (610) 758-4991 |
| - E.W.F.M. Lounge (Peet’s Coffee and Tea) (Lower Level Fairchild Martindale Library) | Jason Lenig, Director of Retail Dining (610) 758-4170 |

### Asa Packer Dining Room:

Director: Claudine Sidney

### Catering:

Director: Claudine Sidney (610)758-6225

# Appendix C

## Present Recycling and Disposal of Materials

| Abbreviations: | FS&CP - Facilities Services & Campus Planning |  |
| --- | --- | --- |
|  | EH&S - Environmental Health & Safety |  |
|  | P&M - Printing & Mailing |  |
| List of Waste/Recycling Contractors | Services | Lehigh Contact |
| ABM | Collect indoor trash/single stream recycling for all three campuses. Also collects e-waste and takes it to Building J. Recycles white goods. Removes refrigerant and arranges for pickup by a vendor. | Facilities Services & Campus Planning |
| ACM Associates, Inc | Asbestos Removal & remediation | Facilities Services & Campus Planning |
| AERC | Batteries collected by FS&CP & ABM including batteries from electronics removed by Printing & Mailing Services. Tubes, ballasts and lamps changed by ABM. Library microfilm removal project. | Facilities Services & Campus Planning |
| An e-waste contractor | picks up electronics for proper disposal. Batteries are separated by Printing & Mailing Services | Facilities Services & Campus Planning |
| ASA's Attic | Department's Surplus Property Items. This program will fall under a new Surplus management system beginning Spring 2018 | Purchasing |
| Brightview Industries | Ground Maintenance for all 3 campuses: leaf removal, snow removal. Materials collected taken to leaf composting area off College Drive. Some composting reused in planting beds. Collect outdoor trash/single stream recycling. | Facilities Services & Campus Planning |
| Buckman | Scrap metal from FS&CP. | Facilities Services & Campus Planning |
| Call2Recycle | Collected by EH&S who then arranges for proper disposal | Environmental Health & Safety |
| Clean Harbors | Hazardous waste: chemical waste (collected at Seeley-Mudd Room 184, Iacoca Hall Room C44 STEPS Room 082), paint, batteries collected by EH&S | Environmental Health & Safety |
| Contractor for particular project | Construction & Demolition (concrete, asphalt, metal, wires, piping) recycling and disposal |  |
| Cougle's Recycling | Paper trimmings & printed waste and outdated printed materials | Printing & Mailing |
| Curtis Bay | Biohazardous Waste (Health Center, Iacocca Hall, STEPS) | Facilities Services & Campus Planning |
| Filta-Fry | Removes waste vegetable oil from all dining including concessions). Oil is purified and most of it converted to biodiesel | Dining Services |
| Funding Factory | Laser Printer Ink Cartridges recycling | Printing & Mailing |
| Furniture Bank of Central Ohio | recycled/donated furniture & mattresses. | Residential Services |
| Great South Side Sale | Student Move-Out (some items not included) | Community Service Office |
| H. Binderman & Son Scrap Metal Yard | Transportation Services takes its scrap metal to location in Hellertown for recycling. | Transportation Serv. |
| Interstate Batteries | Transportation Services car batteries recycled (also purchased from same company) | Transportation Serv. |
| Plastic Film | Taken to Giant for recycling. Turned into alternative lumber, decking and furniture | Sustainability Office |
| Safe Harbors | Ink from printing presses | Printing & Mailing |
| Safety Clean | Used motor oil and antifreeze. microfilm from Library removal project. | Transportation Serv. |
| Service Tire | Recycles used tires | Transportation Serv. |
| Sodexo | removes trash/single stream recycling for dining areas | Dining Services |
| Sustainable Waste Solutions | Trash/single stream recycling hauler from-onsite compactors. | Facilities Services & Campus Planning |
| Terracycle | Writing utensils and their packaging. | Brenda Bachman |
| Titan Mobile Shredding | Shreds confidential information and used in paving roads | Printing & Mailing |
| Veolia, Inc. | Radioactive waste | Environmental Health & Safety |

# Appendix D

## 2017 Water Data by Campus and by Building

|  | kgal |
| --- | --- |
| Asa Packer Campus | 98,180 |
| Goodman Campus | 2,584 |
| Mountaintop Campus | 2,675 |
| Three-Campus Total | 103,438 |

| Facility Name | Building No. | Campus | Function | CY17 Total (kgal) |
| --- | --- | --- | --- | --- |
| Sherman Fairchild Laboratory | 16A | Asa Packer | Academic | 4,969 |
| STEPS | 9A | Asa Packer | Academic | 3,610 |
| Mudd Building | 6 | Asa Packer | Academic | 2,377 |
| Whitaker Laboratory | 5 | Asa Packer | Academic | 1,575 |
| Rauch Business Center | 37 | Asa Packer | Academic | 1,085 |
| Packard Laboratory | 19 | Asa Packer | Academic | 1,082 |
| Zoellner Arts Center | 49 | Asa Packer | Academic | 940 |
| Fritz Laboratory | 13 | Asa Packer | Academic | 669 |
| Sinclair Laboratory | 7 | Asa Packer | Academic | 595 |
| Maginnes Hall | 9 | Asa Packer | Academic | 444 |
| Mohler Laboratory | 200 | Asa Packer | Academic | 426 |
| Williams Hall | 31 | Asa Packer | Academic | 346 |
| Sherman Fairchild Cntr - Physics | 16 | Asa Packer | Academic | 293 |
| Chandler-Ullmann Hall | 17 | Asa Packer | Academic | 272 |
| Neville Hall | 6A | Asa Packer | Academic | 176 |
| Christmas Saucon Hall | 14/14a | Asa Packer | Academic | 166 |
| Philosophy Building | 15 | Asa Packer | Academic | 139 |
| Drown Hall | 35 | Asa Packer | Academic | 138 |
| Wilbur Hall | 12 | Asa Packer | Academic | 127 |
| Johnson Hall | 36 | Asa Packer | Academic | 115 |
| Coppee Hall | 33 | Asa Packer | Academic | 57 |
| Distance Education | 189 | Asa Packer | Academic | 29 |
| Humanities Center | 224 | Asa Packer | Academic | 13 |
| 215 Summit St | None | Asa Packer | Academic | 8 |
| CH&R | 10 | Asa Packer | Admin | 6,460 |
| CH&R (Packer) | 10 | Asa Packer | Admin | 4,192 |
| Rathbone Hall (Bldg) | 63 | Asa Packer | Admin | 2,790 |
| Alumni Memorial Building | 27 | Asa Packer | Admin | 1,375 |
| Alumni Memorial Building | 27 | Asa Packer | Admin | 863 |
| 526 Brodhead Ave #23 | 23 | Asa Packer | Admin | 71 |
| Budget Office | 187 | Asa Packer | Admin | 71 |
| 522 Brodhead Ave #23 | 23 | Asa Packer | Admin | 69 |
| 516 Brodhead Ave #23 | 23 | Asa Packer | Admin | 68 |
| Zoellner Arts Center Administration Building | 4 | Asa Packer | Admin | 49 |
| Environmental Health and Safety | 023A | Asa Packer | Admin | 34 |
| Facilities Services/Planning | 3 | Asa Packer | Admin | 19 |
| Facilities Services and Campus Planning | 3a | Asa Packer | Admin | 18 |
| 622 Brodhead Ave #199 | 199 | Asa Packer | Admin | 8 |
| 616 Brodhead Ave #197 | 197 | Asa Packer | Admin | 7 |
| 618 Brodhead Ave | 198 | Asa Packer | Admin | 7 |
| Police Station | 47 | Asa Packer | Admin | 6 |
| Taylor Gym | 38 | Asa Packer | Athletics | 1,064 |
| Grace Hall | 39 | Asa Packer | Athletics | 447 |
| University Center (Dining) | 29 | Asa Packer | Dining | 3,351 |
| Rathbone Hall (Dining) | 63 | Asa Packer | Dining | 1,666 |
| Lamberton Hall | 34 | Asa Packer | Dining | 306 |
| Trembley Park | 71-77 | Asa Packer | Housing | 4,850 |
| Alpha Sigma Phi | 106 | Asa Packer | Housing | 3,875 |
| Brodhead House | 201 | Asa Packer | Housing | 2,822 |
| M&M House | 67 | Asa Packer | Housing | 2,818 |
| Centennial II | 50-55 | Asa Packer | Housing | 2,419 |
| Farrington Square | 202 | Asa Packer | Housing | 1,967 |
| Dravo House | 65 | Asa Packer | Housing | 1,871 |
| Centennial I | 58 | Asa Packer | Housing | 1,791 |
| Taylor House | 68 | Asa Packer | Housing | 1,457 |
| Drinker House | 64 | Asa Packer | Housing | 1,409 |
| Richards House | 66 | Asa Packer | Housing | 1,382 |
| Alpha Phi | 98 | Asa Packer | Housing | 685 |
| Beta Theta Pi | 83 | Asa Packer | Housing | 654 |
| Sigma Phi Ep. | 92 | Asa Packer | Housing | 584 |
| Gamma Phi | 100 | Asa Packer | Housing | 579 |
| Delta Upsilon | 81 | Asa Packer | Housing | 571 |
| Alpha Gamma Delta | 95 | Asa Packer | Housing | 514 |
| Alpha Chi Rho | 93 | Asa Packer | Housing | 447 |
| Alpha Omicron Pi | 107 | Asa Packer | Housing | 426 |
| Sayre Park Village C | 109c | Asa Packer | Housing | 417 |
| Delta Tau Delta | 89 | Asa Packer | Housing | 412 |
| Sigma Alpha Mu | 94 | Asa Packer | Housing | 412 |
| Kappa Sigma | 87 | Asa Packer | Housing | 407 |
| Kappa Alpha | 85 | Asa Packer | Housing | 407 |
| Sayre Park Village A | 109a | Asa Packer | Housing | 382 |
| Theta Xi | 96 | Asa Packer | Housing | 355 |
| Sayre Park Village B | 109b | Asa Packer | Housing | 345 |
| Theta Chi | 91 | Asa Packer | Housing | 342 |
| Chi Psi | 84 | Asa Packer | Housing | 299 |
| Sigma Chi | 102 | Asa Packer | Housing | 299 |
| Delta Phi | 86 | Asa Packer | Housing | 280 |
| Chi Phi | 105 | Asa Packer | Housing | 280 |
| 230 W Packer Ave Delta Chi Annex | 22 | Asa Packer | Housing | 279 |
| Phi Sigma Kappa | 90 | Asa Packer | Housing | 258 |
| Psi Upsilon | 80 | Asa Packer | Housing | 239 |
| Alpha Tau Omega | 99 | Asa Packer | Housing | 226 |
| Umoja House | 101 | Asa Packer | Housing | 184 |
| 220 Warren Square #196 | 196 | Asa Packer | Housing | 180 |
| Packer House | 20 | Asa Packer | Housing | 148 |
| 532-534 Brodhead Ave #24 | 24 | Asa Packer | Housing | 143 |
| 227 Warren Square | 25 | Asa Packer | Housing | 130 |
| Phi Gamma Delta #104 | 104 | Asa Packer | Housing | 130 |
| 233 W Packer Ave Delta Chi | 21 | Asa Packer | Housing | 104 |
| 212 Warren Square B | 192 | Asa Packer | Housing | 70 |
| President's House | 28 | Asa Packer | Housing | 62 |
| 222 Summit St #193 | 193 | Asa Packer | Housing | 57 |
| Barnett House | 194 | Asa Packer | Housing | 43 |
| Lambda Chi Alpha | 97 | Asa Packer | Housing | 30 |
| Phi Kappa Theta | 88 | Asa Packer | Housing | 29 |
| Sayre Park Village D | 109d | Asa Packer | Housing | 19 |
| Farrington Square | 202 | Asa Packer | Housing | 10 |
| Warren Square F | 21 | Asa Packer | Housing | 1 |
| Fairchild Martindale Library | 8 | Asa Packer | Library | 1,980 |
| Linderman Library | 30 | Asa Packer | Library | 542 |
| 416 East 5th St Rear Garage | 316 | Asa Packer | Parking | 59 |
| Alumni Garage | 26A | Asa Packer | Parking | 21 |
| Farrington Square Parking Garage | 202 | Asa Packer | Parking | 15 |
| 2 Farrington Sq The CUP | 202 | Asa Packer | Retail | 257 |
| 4 Farrington Sq Jazzmans Coffee | 202 | Asa Packer | Retail | 17 |
| University Center (Bldg) | 29 | Asa Packer | Student Life | 1,245 |
| Dialogue Ctr (sub Grace) | 41 | Asa Packer | Student Life | 23 |
| Kappa Delta | 82 | Asa Packer | to be demo |  |
| Ben Franklin Building #125 | 125 | Goodman | Admin | 286 |
| Transportation Services | 126 | Goodman | Admin | 45 |
| Varsity House (Bldg) | 121 | Goodman | Athletics | 996 |
| Goodman Stadium | 150 | Goodman | Athletics | 796 |
| Stabler Arena | 124 | Goodman | Athletics | 492 |
| Varsity House (Laundry) | 121 | Goodman | Athletics | 313 |
| Rauch Field House | 123 | Goodman | Athletics | 269 |
| Ulrich Fields Complex | 151 | Goodman | Athletics | 119 |
| Goodman Drive Softball Field | Field O? | Goodman | Athletics | 65 |
| Field Shop | 127 | Goodman | Athletics | 49 |
| Tennis Center | 128 | Goodman | Athletics | 47 |
| Mulvihill Golf Center | 149 | Goodman | Athletics | 8 |
| Goodman Campus Fields (Bldg C?) | 130 | Goodman | Athletics | 4 |
| Goodman Campus Legends Park | Field O? | Goodman | Athletics | 2 |
| Saucon Village Diamond House | 143 | Goodman | Housing | 1,184 |
| Saucon Village Gipson House | 140 | Goodman | Housing | 878 |
| Saucon Village Severs House | 131 | Goodman | Housing | 864 |
| Saucon Village Hartman House | 137 | Goodman | Housing | 770 |
| Saucon Village More House | 134 | Goodman | Housing | 706 |
| 1918 Creek Rd | None | Goodman | tenant | 15 |
| A Bldg South-Iacocca Hall | 111 | Mountaintop | Academic | 3,740 |
| Building H -Imbt Laboratories | 117 | Mountaintop | Academic | 1,609 |
| A Bldg North -Iacocca Hall | 111 | Mountaintop | Academic | 1,120 |
| Building C-Mountaintop Initiative & Data X | 113 | Mountaintop | Academic | 71 |
| Building D-Central Heating & Refrigeration | 114 | Mountaintop | Admin | 4,182 |
| Building J- Mailing & Printing | 118 | Mountaintop | Admin | 143 |
| Building F -Jordan Hall | 115 | Mountaintop | Admin | 56 |
| Building B-Warehouse | 112 | Mountaintop | Admin | 9 |
| Gatehouse-MTC | 119 | Mountaintop | Admin | 6 |
| Warehouse Off-campus | 225 | off-campus | Admin | 6 |

# Appendix E

## Food Ordering Example