Bedrock Earthscapes, LLC

Exhibit A – Native Area Stewardship

The Roosevelt University Schaumburg (Robin) campus listed below is covered by this Agreement.

Roosevelt University

1400 N. Roosevelt Boulevard

Schaumburg, IL  60173						Approximately 9 acres of native plantings

1.	Areas for native plant stewardship:

Low-profile prairie

Approximately 6.5 acres of areas across the north and west of the property are created low-profile prairie.  The area will receive monthly stewardship to ensure successful continued establishment and improvement.

Native grass parking lot islands

Approximately 1.5 acres of the long 18’ and 28’ wide parking lot islands have been converted to native grasses.  The large parking lot islands were converted using a native grass seed mix containing little blue stem, side oats grama and native forbs for additional color.  The small end islands were planted using 2” prairie dropseed native grass plugs (approximately 2800 plugs).

Ornamental native plant beds

The areas listed below were converted to ornamental native plant beds using one gallon native plants.  These beds are in areas of high visibility to add color and texture to the landscape. They will be maintained as part of this agreement.

- A circular bed surrounding the base of the Torch Tower in the main driveway circle

- Three parking lot islands bordering the walking path through the west parking lot to the west prairie..

No-mow cool season grass meadows

Approximately 1 acre will be maintained as no-mow cool season grass meadows.  This includes the open lawn at the southwest corner of the building and the small turf area south of the south building entry parking lot.  These areas will be maintained weed-free, fertilized once a year and mowed once a year.

2.	ONGOING STEWARDSHIP

- Low-profile Prairie Areas and Native-grass Parking Lot Islands

- Annual mowing will be done in the early Spring (March or April) to cut down the previous year’s herbaceous growth thereby allowing more light to reach the ground to stimulate native seed germination, and to retard non-desirable invasive woody plant growth.  Mowing will be done to a height of approximately 6” to protect the raised crowns of desirable native plants.  Mowing of the walking path through the native areas will be done once per month during the growing season.

- Stewardship visits will be made during April, May, June, July, August, September and October.  The purpose of these visits will be a) to control unwanted non-native woody and herbaceous plants through cutting, physical removal and/or use of appropriate herbicides, and b) to ensure continued improvement in the health, quality and beauty of the native areas.

- Native plant seed will be collected from this area in the Fall for dispersal into open areas and areas of low plant diversity.

- Native ornamental bed areas

- These bed areas will be maintained during weekly landscape maintenance visits to control weeds.  Application of compost to these bed areas is not included in this base agreement but is available as a separate service if needed.  As plants in these beds mature, plant density should negate the need for traditional bed mulching practices.

- Prescription fire is one tool in managing and restoring native areas.  It is used to clear debris, recycle nutrients, create favorable conditions for native seed germination, control invasive plants and to stimulate native plant and animal species.  Native plant areas are now established, and annual controlled burns in restoration areas should be carried out as practical and as weather permits as an additional service.  The size, location and condition of the areas to be burned will all be considered.  Proper site preparation, fire breaks, burn plans and notification of appropriate local authorities are needed and will be completed prior to controlled burns.

- Watering of native or created native areas is not included in this agreement.  BEDROCK EARTHSCAPES will offer watering services during the first season of any new plantings to help ensure proper establishment.

Bedrock Earthscapes, LLC

Exhibit B - Landscape Maintenance Services

1.	DESCRIPTION OF SITE:

Cool season turf, shrubs, trees and traditional landscape areas on the Roosevelt University Schaumburg (Robin) campus listed below is covered by this Agreement.

Roosevelt University

1400 N. Roosevelt Boulevard

Schaumburg, IL  60173							Approximately 4 acres.

2.	LIST OF LANDSCAPE MAINTENANCE SERVICES:

A/N = As Necessary to maintain a proper appearance.

N/I = Not Included in the base agreement. We would be pleased to submit a separate proposal for this service.

Number of Occurrences

Services	per year

A.	Services in the month of April will consist of the following:

1)		Cleanup accumulated winter debris, dead plant material and leaves

from turf and bed areas.	1

2)		Clean accumulated winter organic debris from parking lot curb line

gutters.	1

3)		Mow all turf areas (lawn and no-mow) to approximately 2.5".	1

4)		Clean grass clippings from walks and paved areas after mowing.	1

5)		Blow off winter debris from the perimeter sidewalks.	1

B.	Services to be performed during the months of May, June, July, August, September and October will be as follows:

1)		Mow established lawn turf, including the large court yard, to approximately 3".	25

2)		Blow grass clippings from walks and paved areas after mowing.	25

3)		Trim along walks, buildings, plantings, signs, and other obstructions.	25

4)		Line trim grass adjacent to the walks, driveways, and roadways.	2

5)		Fertilize turf with a balanced turf fertilizer based upon soil test results,									including 1# of nitrogen per application. (May & September)	2

Fertilize no-mow areas in September	1

6)		Apply weed control to turf, no-mow and native grass areas to control

broadleaf weed species.  Spot treatments to be made as needed.	                         2

7)		Core aeration of turf areas in the Fall.	N/I

8)		Prune shrubs by cutting the current season’s growth as needed.	A/N

9)		Trim formal hedges by cutting the current season’s growth.	2

12)		Prune and shape ornamental landscape trees under 15' of suckers,

low hanging branches, and dead or diseased branches.	A/N

13)		Apply Round-Up weed inhibitor to bed areas and sidewalk and parking lot

cracks to control weed growth.	2

14)		Hand weed groundcover and planting beds to keep

a weed-free appearance.			A/N

15)		Edge turf adjacent to beds.		1

- 16)	Maintain gravel beds in a neat and weed-free manner.	A/N

17)	   Cover existing mulched beds with hardwood mulch to maintain

a 2.5” mulch cover.	(will be quoted separately upon request)	N/I

18)		Install annual flowers.	(will be quoted separately upon request)	N/I

19)		Mow the walking path through the native prairie areas monthly.	7

20)		Spray out and/or hand weed the sand volleyball court monthly	7

21)		All court yards (A/B, main, E, F, G) will be line trimmed and blown off monthly.	7

C.	Services in the month of November will consist of the following:

1)		Mow turf to approximately 2 ¼".	1

2)		Clean grass clippings from walks and paved areas after mowing.	1

3)		Fall cleanup of leaves and debris from turf and bed areas.	2

3.	MOWING OF GRASS

A.	Mowers shall be kept sharp and in good condition in order to cut the grass blades cleanly.  Lawn turf shall be cut evenly and the direction of mowing shall be alternated as needed to minimize ruts and matting.  No-mow areas will be mowed once in the Spring to help maintain a uniform appearance.  Appropriate mowing equipment and patterns will be employed to permit recycling of clippings on the lawn.  Heavy clipping accumulation will be picked up if necessary.

B.	Mowing height in the summer shall be set at 3" so as to encourage a deeper root system.  A deep-rooted lawn is more efficient in obtaining water and nutrients and is more drought tolerant and weed resistant.

C.	Mowing height for the last cutting shall be lowered to 2 1/4".  This is done in preparation of winter and will discourage turf diseases that can form if tall grass is matted down by snow.

4.	TURF FERTILIZATION AND WEED CONTROL

A.		A healthy turf will be promoted through the most effective use of all available turf cultural practices including mowing, irrigation, overseeding, aeration and fertilization.  Your turf fertility program will be based on soil test results, and is designed to promote a dense turf with a uniform growth pattern throughout the season without excessive growth.  It can be customized based upon your quality expectations.  Our desire is to not only promote a healthy turf, but to also promote the greater benefits of a healthy soil balance.  We believe that this is an environmentally sound approach to fertilization that will provide our customers with the best immediate and long term results.  Generally, we will use balanced slow release fertilizers at the rate of approximately 1 pound of nitrogen per thousand square feet per application.

B.	 A healthy turf is the best weed control, and BEDROCK EARTHSCAPES will attempt to limit the use of chemicals for both post-emergent and pre-emergent weed control.  Blanket treatments of broadleaf weed control will be limited to two times per year, with additional spot treatments being used as needed to contain broadleaf weeds in turf areas.

C.  BEDROCK EARTHSCAPES shall only use chemicals which are non-restricted, approved by the Environmental Protection Agency and are considered to be the safest and most effective materials for the specific task.

D.		All pesticides shall be applied by a licensed applicator or a licensed operator under supervision of a licensed applicator in accordance with local, state and federal regulations.

5.	PRUNING and BED CARE

- The pruning and trimming described in SECTION 2 shall be done at the proper time as required by the plant species.  Dead or damaged branches will be removed; however, this does not include removal of severely damaged or dead trees and shrubs.  Pruning, rather than shearing, will be used where ever possible to develop the natural form of the plant and to create the effect intended by the landscape architect.

- BEDROCK EARTHSCAPES will attempt to limit the use of chemicals for weed control in bed areas.  Blanket treatments of pre-emergent weed control will be limited to known problem areas.  Hand weeding and chemical spot treatments will be used as needed to control weed growth in bed areas.

6.	WATERING

- Watering of turf areas, bushes, trees and other plant material is not included in this Agreement and shall be responsibility of the Owner.  BEDROCK EARTHSCAPES will give recommendations for watering so as to promote a healthy landscape, and work with the owner on automatic irrigation system cycles and durations to minimize potable water use and runoff.  Watering services are available and may be contracted separately, if requested.

7.	INSECT AND DISEASE CONTROL

- BEDROCK EARTHSCAPES will make an effort to monitor problems with insects and diseases throughout the growing season.  If pesticide applications are necessary in order to control insects and/or diseases, BEDROCK EARTHSCAPES will notify Owner of the recommended treatment and the cost of treatment, which shall be in addition to the compensation specified in Section 6 of the Landscape Services Agreement.

8.	POLICING GROUNDS and DEBRIS

- Papers and other debris will be picked up prior to each mowing, but routine debris pick up will be the responsibility of the Owner.  BEDROCK EARTHSCAPES will use the Owner's disposal containers for trash and debris picked up on site.

- As part of our commitment to retain and recycle all organic waste on-site, a compost site will be established on the OWNER’s property to compost all site generated organic debris.  BEDROCK EARTHSCAPES will use finished compost on site for soil enhancement.

9.	QUALITY CONTROL

A.		BEDROCK EARTHSCAPES shall make every effort to perform services so as to avoid inconvenience to Owner.  Weekly services will be performed on the same day(s) each week.  However, in the event of holidays, rain or other factors, BEDROCK EARTHSCAPES may need to reschedule to another day.  This will usually be the next day.

B.	    BEDROCK EARTHSCAPES has set a high standard of quality and professionalism.  This maintenance program will be carefully performed in order to achieve the maximum visual and environmental impact.  BEDROCK EARTHSCAPES has included a visual inspection of your site as part of our weekly service visit.  Additionally, we will attempt to meet with your designee during each service visit, and provide e-mail follow-up that documents our visit.

- BEDROCK EARTHSCAPES requests the opportunity to perform documented, on-site inspections with you at least twice a year. This is to jointly review your property in detail, and to ensure that BEDROCK EARTHSCAPES is meeting your expectations of quality landscape service.

Thank you for your business, and the opportunity to be of service.  We look forward to working with you as we provide these services.  If you have any questions, comments or requests, now or at any time, please feel free to contact us.

Bedrock Earthscapes, LLC

630-461-1159

Enriching life through improving our environment.