Great Salt Lake Wetlands

2012 LiDAR Mapping Project

Box Elder, Weber, Davis, Salt Lake and Tooele

Counties, Utah

COMPLETION REPORT

# Table of Contents

Table of Contents	1

SUMMARY OVERVIEW	2

Executive Summary	2

Contractor	2

Scope Overview	2

Specifications for Deliverables	3

LiDAR Deliverables	3

Grid Model Deliverables	3

Miscellaneous Deliverables	3

Project Area Extents and Project Tile Index	3

LiDAR DATA REPORT	4

Pre-Flight Planning	4

Control	4

Final Planning – Procedures and Activities	5

Planning	5

Summary of Supporting Documents	5

Data Processing Procedures Report	6

Data Storage	6

Navigation System	6

LiDAR System	6

LiDAR QUALITY CONTROL REPORT	9

Methodology	9

Relative Accuracy	9

Within Swath Overlap Accuracy	9

Fundamental Vertical Accuracy	10

Conclusions	11

FLIGHT REPORT	12

GROUND CONTROL REPORT	13

Introduction	13

Ground Control Survey	13

Data Collection	13

Data Processing and Analysis	13

APPENDIX A – Index Maps and Area Boundaries	16

APPENDIX B – Flight Plan Maps	21

APPENDIX C – Raw Data File Listing	22

# SUMMARY OVERVIEW

## Executive Summary

This project encompasses five areas in five Utah counties and encompasses about 1147 square miles shown below. Data was collected on September and October, 2011.

| Study Area | County | Size (mi²) |
| --- | --- | --- |
| Great Salt Lake Wetlands in three flight blocks labeled “North”, “Middle”, and “South | Salt Lake, Davis, Weber, Box Elder | 834 |
| Tooele | Tooele | 251 |
| Lower Bear River | Box Elder | 62 |
| Total |  | 1147 |

## Contractor

This project was completed under contract UGS110817 between Utah Automated Geographic Reference Center (Utah AGRC) and Utah State University (USU) LASSI Service Center.

Primary technical point of contact information:

Robert T. Pack, Ph.D., P.E

robert.pack@usu.edu

Utah State University

LASSI Service Center

4110 Old Main Hill

Logan, UT 84322-4110

PH 1-435-797-7049

## Scope Overview

Our responsibilities included:

- Flight planning;

- Identification of ground control to be applied as airborne GNSS base stations and for DEM processing;

- Aerial data acquisition;

- Collection of GNSS base station data during flight;

- Collection of GNSS RTK ground data for application in DEM accuracy testing;

- Processing, calibration and classification of LiDAR returns;

- Output of data deliverables including metadata;

- Compilation of Project Completion Report, including Flight, Data Processing and LiDAR DEM Accuracy reporting in compliance with National Standards for Spatial Data Accuracy (NSSDA) guidelines.

## Specifications for Deliverables

The required accuracy and file formats for each delivery was as follows:

### LiDAR Deliverables

Grid Projection:	UTM Zone 12N
Horizontal Datum:	NAD83(CORS96)
Vertical Datum:	NAVD88 using GEOID09
Tile Size:	2000 m X 2000 M
Average Post Spacing:	0.85 m
Average Data Density:	1.37 sh/m2
File Formats:	*.las (v. 1.2)
Classified Datasets:	ASPRS/LAS Default Classes

### Grid Model Deliverables

File Format:	IMG (.img)
Grid Projection:	UTM Zone 12N
Horizontal Datum:	NAV83(CORS96)
Vertical Datum:	NAVD88 using GEOID09
Tile Size:	2000 m X 2000 m
Cell Size:	1.00m

### Miscellaneous Deliverables

Breakpoint Files:	LAS 1.2 (.las) on specific code
Metadata Files:	FGDC compliant XML file. (.xml)
Project Tile Index:	Portable Document Format (.pdf)
Completion Report:	Portable Document Format (.pdf)

LiDAR data acquisition was performed using a Riegl LMS Q560 airborne laser sensor system capable of up to a maximum 200 kHz pulse repetition rate and collection of full waveform returns.

## Project Area Extents and Project Tile Index

The tile layout and project extents for the five areas surveyed are provided in Appendix A. The number of tiles is summarized in Table 1.

Table 1. Project areas.

| Area | Number of Tiles |
| --- | --- |
| Bear River | 40 |
| Great Salt Lake (GSL) North | 463 |
| Great Salt Lake (GSL) Middle | 83 |
| Great Salt Lake (GSL) South | 250 |
| Tooele | 98 |

Tiles were designed on a 2000 m by 2000 m grid and were automatically generated.

# LiDAR DATA REPORT

## Pre-Flight Planning

Appendix B provides a map showing flightline layout and target locations for the five subject areas. Table 3 provides the pre-flight mission parameters used for the project.

Table 3. Pre-flight mission parameters.

## Control

The area surrounding the study area was searched for candidate vertical control monuments over which the GNSS ground station could be placed. The goal was to tie to A- or B-order vertical control, while at the same time, be within 10 km of the study area. A total of 8 ground control stations were used for this project.

The benchmarks were selected on the basis of (1) vertical control accuracy, (2) accessibility, (3) security for targets and the GPS base station.  Five GPS base stations were established. Benchmarks on the north shore (B 94), in Weber County (H 23) and Tooele (H 51) were occupied for several days each.  This enabled the calculation of strong static GPS solutions which have been compared with the published vertical coordinates.  Moreover, each of these GPS stations were active during lidar flights thereby enabling differential GPS corrections.

At each of the stations, 5-foot diameter white circular targets were established, an example of which is shown in Figure 1 for station 314RM in Davis County.  The surface of each target was leveled using a five foot long construction level.  The target height was then determined using an automatic level. This was done using a back-sight to the monument and a fore-sight to the table surface (see Figure 1). The accuracy of the target height relative to the monument was consistently within about 1 cm. All eight targets were scanned by the lidar in at least one flightline.

The GPS base stations were set up directly over the given monument and the height to the antenna measured within 1 mm.  This was used to compare calculated coordinates with published coordinates. In order to make proper comparisons, the heights measured at previous dates needed to be adjusted according to observed HTDP point velocities published by NGS for nearby CORS stations. These points were thereby brought up to date.

Figure 1. Example of lidar target along with equipment used to level its height relative to a nearby benchmark. This is benchmark 314RM in Davis County.

## Final Planning – Procedures and Activities

### Planning

Weather forecasts and project schedule identified an aerial acquisition window during the months of September and October 2011. Prior to each acquisition campaign, the following was completed:

- Brief flight crew and ground support personnel on project requirements

- Investigate PDOP forecast for location (Flights to be conducted with PDOP below 3.0)

- Decision to mobilize Bob Pack to site to set up targets and GNSS base stations.

- Complete a reconnaissance of the project area was conducted to report on ground conditions.

It was planned such that each time the aircraft was mobilized out of Logan, Utah each of the five areas could be completed during a contiguous block of days.

### Summary of Supporting Documents

- CV NGS DATASHEETS.htm– NGS Data Sheets NGS benchmarks used

- PDOP Plots subdirectory – contains PDOP forecasts for periods of data acquisition.

(The above listed documentation is provided in softcopy format only.)

## Data Processing Procedures Report

### Data Storage

After each flight, all raw navigation data, raw LiDAR data, raw image data, coverage data, and flight logs were off-loaded to a computer and an additional backup storage copy created.

### Navigation System

The airborne GNSS data were processed from the five base station locations using GrafNet software from NovAtel.  Data was also collected from nearby International GPS Service for Geodynamics (IGS) stations for the periods of the flight. Airborne GNSS data was processed based on the ITRF05 Ellipsoid model.

The computed trajectories and the base station coordinates were used in the processing of the IMU data using Inertial Explorer from Waypoint. A smoothed trajectory was produced with error estimates based on the separation between trajectories processed forward and backward in time. The trajectory files were then transformed to the NAD83(CORS96) and NAVD88(GEOID09) project datum and the UTM Zone 12N projection for use in the LiDAR processing.

### LiDAR System

LiDAR waveform files were analyzed using RiAnalyze software to discriminate data points. These points are output in the internal coordinate system of the LiDAR scanner. Each data point is assigned an echo value so it can be used in point classification work. RiProcess then uses the trajectory files created from the raw navigation data to generate XYZ points in a world coordinate system. A boresight calibration and strip (single scan line) adjustment was performed in RiProcess to improve data accuracy. This project’s data were processed in strip form, meaning each flight line was processed independently.  Processing the lines individually provides the data analyst with the ability to quality control (QC) the overlap between lines. To assess trajectory integrity, individual flight strips were then checked against adjacent strips to ensure good matching in the dataset.

The low gradient terrain within the study area results in highly visible manifestations of errors within overlap regions. For example, on some of the shoreline slopes a gradual 10 cm drop in elevation can occur over a distance of 1000 m.  Hence a 1 cm contour interval would be 100 m wide and a 2 cm vertical error would result in a 200 m shift in a contour location.  It was therefore necessary to develop custom strip overlap adjustment methods that would not only optimize the lidar system calibration but also correct GPS/IMU navigation errors manifested within individual strips.

A method has been implemented that corrects for aircraft roll and aircraft altitude error detected by analyzing elevation differences in all overlapping strips simultaneously. Figure 2 shows an example color-coded map of overlapping regions where blue equals a -10 cm difference, cyan a -5 cm difference, green 0 cm, yellow +5 cm, and red +10 cm.  Figure 3 shows the same series of strips after adjustment. Because the center of the overlap zone is where adjacent strips are mosaicked via a mosaic line, it is important that these lines are consistently green. As shown in Figure 3 this is the case for all strips which results in smooth contouring across the entire project. This wouldn’t have been the case using traditional methods that ignore within-strip errors associated with the GPS/IMU system.

Figure 2. Overlap data prior to within-strip correction, colored by elevation difference (blue = -10 cm, cyan = -5 cm, green = 0 cm, yellow = +5cm, red = +10 cm).

Mosaic Lines

Figure 3. Overlap data after the within-strip correction, colored by elevation difference (blue = -10 cm, cyan = -5 cm, green = 0 cm, yellow = +5cm, red = +10 cm).

Each flightline (strip) was then brought into TerraScan (by Terrasolid) in the project datum and coordinate system. These flightlines were then combined and several classification routines, customized for the given terrain and vegetation, were then run to classify the points into standard ASPRS/LAS default classifications.

Significant effort was given to the creation of automated routines that would detect the dozens of river banks and hundreds of lake shorelines within the subject areas. The routine then automatically creates polylines that then serve as breaklines for hydro-flattening.  For this work, custom tools were developed using LAS-tools, a set of routines developed by Martin Isenburg (out of Germany), and custom Matlab scripts developed in-house. These breaklines, consisting of a series of closely spaced points were then added to the point cloud LAS files with a unique classification code. When combined in a LAS file with original lidar points, the quality of the hydro-flattening can immediately be exploited as a triangulated irregular network (TIN) in any LAS viewer or GIS system (such as ArcGIS).

Using the point classifications and breakline points, digital elevation models (DEMs) of the bare earth and digital surface models (DSMs) of all points were generated for each tile and carefully checked for data quality assurance.

# LiDAR QUALITY CONTROL REPORT

## Methodology

The QC check was intended to ensure that data would meet contractual standards set in FEMA (2003, Section A.8) and USGS NGP Guidelines v.13 (2010). Table 4 provides a summary of their standards for root mean squared error in the z (height) direction (RMSEz):

Table 4. Standards for RMSEz used in this project.

| RMSEz | Condition | Source |
| --- | --- | --- |
| 7.0 cm | Relative accuracy within individual swaths | USGS |
| 10.0 cm | Within swath overlap regions | USGS |
| 12.5 cm | Fundamental vertical accuracy (in the clear) | USGS |
| 18.5 cm | Under all major vegetation categories in flat areas | FEMA |
| 37.0 cm | Under all major vegetation categories in hilly areas | FEMA |

### Relative Accuracy

Relative DEM accuracy was checked for the two typical terrain types within this project using RTK GPS surveys. Table 5 shows the results for these areas. The results show a relative accuracy of 3.8 cm within the typical rolling sagebrush terrain of the GSL shoreline. This is similar to the 2.8 cm relative accuracy achieve on the flat sagebrush areas of Cedar Valley (sister project). A relative accuracy of 3.6 cm is achieved in a subdivision in Hooper. These results are well under the 7.0 cm specification required by the contract.

Table 5. Relative accuracy checks.

### Within Swath Overlap Accuracy

Table 6 shows the mean and RMSEz difference between all DEM cells within overlapping regions. These statistics were calculated by custom Matlab scripts in USU’s custom adjustment software. Table 6 shows that systematic shifts within a given overlap region are less than 1 cm. The RMSEz between overlapping surfaces is consistently between 2.3 and 6.3 cm  These results are within the required 10 cm specification.

Table 6. Mean and RMSEz difference between DEM cells within overlapping regions.

### Fundamental Vertical Accuracy

It was proposed and accepted by AGRC that a series of 5’ diameter LiDAR targets be used as a spot checks for fundamental vertical accuracy relative to a selection of know brass bench marks distributed around the subject area. The strategy was to place these targets prior to the flights and measure their height using the lidar results such that they could be compared to independently leveled heights measured in the field relative to the brass bench marks.  Table 7 shows the results of this work for bench marks occupied by long GPS static observations associated with the lidar collection. The results indicate an average fundamental vertical accuracy of 8.1 cm for the three targets relative to the published bench mark elevations.  It should be noted that the average difference between the GPS static measurements and the published elevations is 7.1 cm.  Given the GPS residuals are in the order a less than 2 cm, it is possible some of the vertical error is associated with the published coordinates. Nevertheless, these results indicate the fundamental vertical accuracy is well within specifications required for this project.

Table 7. Fundamental vertical accuracy as determined at four lidar target locations with strong vertical control.

Five targets were also placed near benchmarks that were not occupied by our static GPS surveys. These differences were found to average 17.8 cm as shown in Table 8. The largest differences are associated with RTK surveyed benchmarks maintained by Salt Lake and Weber Counties. These county surveys focused on horizontal control and did not have clearly stated vertical accuracies. A 14.7 cm discrepancy was discovered with an old 1953 benchmark given a Class II vertical accuracy by NGS. However, only a 7.4 cm difference was found with a NGS Class I vertical benchmark found in the Bear River area. Given the 8 cm accuracy results in the previous table that are cross-checked with multi-day static GPS work, it is deemed unlikely that the main source of error is associated with the lidar survey. It is also possible that since their dates of publication, some of these points may have been subject to movement associated with settlement of the silts or construction disturbances. The investigation of the vertical accuracy of these published coordinates is beyond the scope of this contract. Nevertheless, these result suggest that adjustments of the lidar data by up to 26 cm (10 inches) may be necessary in order to match local datums based on weak vertical control.

Table 8. Vertical accuracy as determined relative to benchmarks with various vertical accuracies.

Horizontal positional accuracy was not formally tested in this project and was not a specification of this contract.

## Conclusions

Given results given above, the following can be concluded:

- There is a tested < 4 cm RMSEz relative accuracy,

- There is a tested < 7 cm RMSEz overlap accuracy, and

- There is a tested < 8 cm RMSEz fundamental vertical accuracy.

# FLIGHT REPORT

USU’s Cessna 208B Skywagon remote sensing aircraft, N4630F, based out of Logan, Utah was utilized on this project.  This aircraft was mobilized out of Logan Municipal Airport, Utah.  The actual local flight times and duration of flights were controlled by weather, fuel consumption of the aircraft on the commute from Logan, Utah, and safety of flight operations around Hill Air Force Base and the Salt Lake International Airport. This limited our flexibility in planning for times when the GNSS constellation was most favorable thereby producing the highest number of satellites visible in the best geometric configuration relative to the GNSS receivers onboard the aircraft as well as at the base station on the ground.

Ordinarily two flights were performed per day, weather permitting.  Flights originated from Logan, Utah each morning with a refueling stop at a local field at mid-day. Flight durations varied between 3 and 4 hours.  At the beginning or end of most days, a calibration flight pattern was flown over the USU campus. This enabled the improvement of IMU to Lidar alignment which has a tendency to drift in virtually every lidar system.

Because of limitations associated with flying around Salt Lake International Airport, the GSL South block was flown at night. This involved two flights between midnight and 6:00am on October 13 & 14, 2011 and a flight between 2:00am and 6:00am on October 18, 2011. During these periods, virtually no interference with air traffic was encountered.

The flight dates are summarized by Table 9.

Table 9. Summary of flight dates.

| Block | Dates |
| --- | --- |
| Bear River | 28 September 2011 |
| GSL North | 27, 29, 30 September & 3, 4, 10 October 2011 |
| GSL Middle | 10-12 October 2011 |
| GSL South | 13, 14, 18 October 2011 |
| Tooele | 18 October 2011 |

Navigation File(s):

A listing GPS base station files and raw flightline (LiDAR) files is given in Appendix C.

# GROUND CONTROL REPORT

## Introduction

A LiDAR survey was conducted for the purposes of developing a high-accuracy digital terrain model (DTM) of the Great Salt Lake Wetlands project area.  In support of this work, ground control was established near the project area. This report summarizes the results.

## Ground Control Survey

Table 10 provides a list of coordinates for each of the 9 bench marks used in this study. The benchmarks listed with a bold font were used as static GPS stations and were occupied during the lidar flights. Stations identified with an asterisk were used as base stations for RTK surveys subsequent to the flights.

Table 10. List of benchmarks used in the five subject areas.

## Data Collection

Using physical descriptions of benchmark locations, each of the 9 stations were occupied, some used for static GPS observations, some used for RTK data collections and all of which were used for lidar target analysis. The static observations were made with a NovAtel dual-frequency GPS receiver. RTK measurements were made with a Topcon GR-5 GNSS (including GLONASS) base/rover pair.

## Data Processing and Analysis

Processing steps performed at each benchmark include ellipsoid to orthometric height conversion, horizontal time-dependent processing of point velocities for epoch adjustment, and target leveling relative to the benchmarks.  Static GPS solutions are disclosed for those points occupied and lidar shot elevations have been compiled for each of the targets. A summary of these processing results is given in Tables 11 and 12.

Table 11. Ground control computations.

Table 12. Ground control computations (continued).

# APPENDIX A – Index Maps and Area Boundaries

# APPENDIX B – Flight Plan Maps

N

Flight line layout and target locations for the Great Salt Lake Wetlands

N

# APPENDIX C – Raw Data File Listing

BEAR RIVER BLOCK:

Flown 09/28/2011
Navigation File(s):

Remote_20110928_01.log

Remote_20110928_02.log

Base Station File(s):

BaseStation_20110928.pdc

Raw Flightline (LIDAR) Files:

110928_163228.sdf

110928_163543.sdf

110928_163943.sdf

110928_164406.sdf

110928_164903.sdf

110928_165319.sdf

110928_165852.sdf

110928_170513.sdf

110928_171216.sdf

110928_171959.sdf

110928_172621.sdf

110928_173252.sdf

GSL NORTH BLOCK:

Flown 09/27 - 09/30, 10/03 – 10/04, 10/10

Navigation File(s):

Remote_GSL_20110927.log

Remote_GSL_20110927_02.log

Remote_GSL_20110928_02.log

Remote_GSL_20110929_01.log

Remote_GSL_20110929_02.log

Remote_GSL_20110930_01.log

Remote_GSL_20110930_02.log

Remote_20111003_01.log

Remote_20111003_02.log

Remote_20111004.log

Remote_20111010_01.log

Base Station File(s):

BaseStation_20110927and28.pdc

Base_GSL_20110929_01.pdc

Base_GSL_20110929_02.pdc

Base_GSL_20110930_01.pdc

Base_GSL_20110930_02.pdc

Base_GSL_20111004_01.pdc

Base_GSL_20111004_02.pdc

Base_GSL_20111010_01.pdc

Raw Flightline (LIDAR) Files:

110927_163856.sdf

110927_164125.sdf

110927_164408.sdf

110927_164523.sdf

110927_164952.sdf

110927_165538.sdf

110927_170112.sdf

110927_170658.sdf

110927_171233.sdf

110927_171834.sdf

110927_172420.sdf

110927_173029.sdf

110927_173600.sdf

110927_174158.sdf

110927_174740.sdf

110927_175426.sdf

110927_180008.sdf

110927_180606.sdf

110927_181136.sdf

110927_181729.sdf

110927_182248.sdf

110927_182843.sdf

110927_183423.sdf

110927_184027.sdf

110927_184610.sdf

110927_185208.sdf

110927_185751.sdf

110927_190414.sdf

110927_190947.sdf

110927_191608.sdf

110927_192210.sdf

110927_192851.sdf

110927_193445.sdf

110927_194137.sdf

110927_194748.sdf

110927_195455.sdf

110927_200127.sdf

110927_214153.sdf

110927_214420.sdf

110927_215123.sdf

110927_215849.sdf

110927_220623.sdf

110927_221350.sdf

110927_222115.sdf

110927_222837.sdf

110927_223629.sdf

110927_224428.sdf

110927_225210.sdf

110927_230001.sdf

110927_230909.sdf

110927_231159.sdf

110928_174124.sdf

110928_174458.sdf

110928_175008.sdf

110928_175737.sdf

110928_195133.sdf

110928_195906.sdf

110928_200630.sdf

110928_201335.sdf

110928_202103.sdf

110928_202828.sdf

110928_203525.sdf

110928_204222.sdf

110928_204918.sdf

110928_205553.sdf

110928_210222.sdf

110928_210830.sdf

110928_211417.sdf

110928_212010.sdf

110928_212528.sdf

110928_213052.sdf

110928_213601.sdf

110928_214057.sdf

110928_214538.sdf

110928_215016.sdf

110928_215446.sdf

110928_215846.sdf

110928_220244.sdf

110928_220628.sdf

110928_221117.sdf

110928_221807.sdf

110928_222502.sdf

110928_223142.sdf

110928_223835.sdf

110928_224531.sdf

110929_161016.sdf (calibration)

110929_161248.sdf (calibration)

110929_161639.sdf (calibration)

110929_162747.sdf

110929_163444.sdf

110929_164129.sdf

110929_164826.sdf

110929_165515.sdf

110929_170207.sdf

110929_170912.sdf

110929_171624.sdf

110929_172306.sdf

110929_173019.sdf

110929_173719.sdf

110929_174415.sdf

110929_175114.sdf

110929_175812.sdf

110929_180503.sdf

110929_181153.sdf

110929_181853.sdf

110929_182542.sdf

110929_183252.sdf

110929_183939.sdf

110929_184632.sdf

110929_185345.sdf

110929_204033.sdf

110929_204750.sdf

110929_205439.sdf

110929_210119.sdf

110929_210833.sdf

110929_211533.sdf

110929_212224.sdf

110929_212921.sdf

110929_213614.sdf

110929_214319.sdf

110929_215013.sdf

110929_215714.sdf

110929_220409.sdf

110929_221119.sdf

110929_221812.sdf

110929_222517.sdf

110929_223217.sdf

110929_223915.sdf

110929_224602.sdf

110929_225325.sdf

110929_225958.sdf

110929_230658.sdf

110929_230904.sdf

110930_160418.sdf (calibration)

110930_160612.sdf (calibration)

110930_160837.sdf (calibration)

110930_162123.sdf

110930_162627.sdf

110930_163335.sdf

110930_164031.sdf

110930_164804.sdf

110930_165518.sdf

110930_170237.sdf

110930_170940.sdf

110930_171655.sdf

110930_172347.sdf

110930_173109.sdf

110930_173806.sdf

110930_174535.sdf

110930_175238.sdf

110930_175944.sdf

110930_180640.sdf

110930_181359.sdf

110930_182136.sdf

110930_182830.sdf

110930_202633.sdf

110930_202856.sdf

110930_203144.sdf

110930_203429.sdf

110930_203747.sdf

110930_204121.sdf

110930_204520.sdf

110930_204907.sdf

110930_205316.sdf

110930_205724.sdf

110930_210207.sdf

110930_210653.sdf

110930_211142.sdf

110930_211648.sdf

110930_212206.sdf

110930_212749.sdf

111003_171841.sdf

111003_172110.sdf

111003_172503.sdf

111003_173133.sdf

111003_173910.sdf

111003_174606.sdf

111003_175325.sdf

111003_180006.sdf

111003_180703.sdf

111003_181345.sdf

111003_182008.sdf

111003_182648.sdf

111003_183340.sdf

111003_184017.sdf

111003_184706.sdf

111003_185332.sdf

111003_185948.sdf

111003_190634.sdf

111003_191313.sdf

111003_191959.sdf

111003_192550.sdf

111003_193154.sdf

111003_193759.sdf

111003_194437.sdf

111003_195141.sdf

111003_195842.sdf

111003_215102.sdf

111003_215807.sdf

111003_220521.sdf

111003_221130.sdf

111003_221734.sdf

111003_222350.sdf

111003_223102.sdf

111003_223301.sdf

111003_223950.sdf

111003_224710.sdf

111003_225348.sdf

111003_230025.sdf

111003_230653.sdf

111003_231433.sdf

111003_232158.sdf

111003_233007.sdf

111003_233635.sdf

111003_234305.sdf

111004_192336.sdf

111004_192540.sdf

111004_195605.sdf

111010_181334.sdf (calibration)

111010_181543.sdf (calibration)

111010_181805.sdf (calibration)

111010_183812.sdf

111010_184606.sdf

111010_185324.sdf

111010_190032.sdf

111010_190749.sdf

111010_191503.sdf

111010_192253.sdf

111010_193027.sdf

111010_193803.sdf

111010_194535.sdf

111010_195257.sdf

111010_195950.sdf

111010_200659.sdf

111010_201346.sdf

111010_202017.sdf

GSL MIDDLE BLOCK:

Flown 10/10 – 10/12
Navigation File:

Remote_20111010_02.log

Remote_20111012_01.log

Base Station File:

Base_GSL_20111010_02.pdc

Base_GSL_20111011_01.pdc

Base_GSL_20111012.pdc

Raw Flightline (LIDAR) Files:

111010_220413.sdf

111010_220620.sdf

111010_220841.sdf

111010_221137.sdf

111010_221453.sdf

111010_221756.sdf

111010_222108.sdf

111010_222454.sdf

111010_222840.sdf

111010_223233.sdf

111010_223718.sdf

111010_224213.sdf

111010_224759.sdf

111010_225356.sdf

111010_225938.sdf

111010_230539.sdf

111010_231139.sdf

111010_231802.sdf

111012_172203.sdf (Calibration)

111012_172341.sdf (Calibration)

111012_172616.sdf (Calibration)

111012_174454.sdf

111012_175144.sdf

111012_175827.sdf

111012_180533.sdf

111012_181223.sdf

111012_181921.sdf

111012_182607.sdf

111012_183317.sdf

111012_184011.sdf

111012_184718.sdf

111012_185356.sdf

111012_190101.sdf

111012_190734.sdf

111012_191434.sdf

111012_192104.sdf

111012_192800.sdf

111012_193434.sdf

111012_194129.sdf

111012_195100.sdf

GSL SOUTH BLOCK:

Flown 10/13 – 10/14, 10/18
Navigation File:

Remote_20111013_01.log

Remote_20111013_02.log

Remote_20111014_01.log

Remore_20111014_02.log

Remote_GSL_South_20111018_01

Base Station File:

00052851.pdc

00052861.pdc

00052871.pdc

00052881.pdc

00052901.pdc

Raw Flightline (LIDAR) Files:

111013_060816.sdf

111013_061515.sdf

111013_061730.sdf

111013_062014.sdf

111013_062304.sdf

111013_062617.sdf

111013_063405.sdf

111013_064138.sdf

111013_064924.sdf

111013_065718.sdf

111013_070508.sdf

111013_071243.sdf

111013_072024.sdf

111013_072800.sdf

111013_073543.sdf

111013_074301.sdf

111013_075030.sdf

111013_075747.sdf

111013_080523.sdf

111013_081245.sdf

111013_082023.sdf

111013_082745.sdf

111013_083518.sdf

111013_084246.sdf

111013_100810.sdf

111013_101529.sdf

111013_102237.sdf

111013_102936.sdf

111013_103659.sdf

111013_104416.sdf

111013_105138.sdf

111013_105844.sdf

111013_110531.sdf

111013_111304.sdf

111013_111936.sdf

111013_112608.sdf

111013_113227.sdf

111013_113846.sdf

111013_114455.sdf

111013_115108.sdf

111013_115649.sdf

111013_120217.sdf

111013_120800.sdf

111014_054202.sdf (calibration)

111014_054344.sdf (calibration)

111014_054550.sdf (calibration)

111014_061040.sdf

111014_061554.sdf

111014_062050.sdf

111014_062607.sdf

111014_063122.sdf

111014_063623.sdf

111014_064121.sdf

111014_064608.sdf

111014_065118.sdf

111014_065550.sdf

111014_070039.sdf

111014_070516.sdf

111014_071022.sdf

111014_071753.sdf

111014_072107.sdf

111014_072426.sdf

111014_072804.sdf

111014_073207.sdf

111014_073628.sdf

111014_074103.sdf

111014_074608.sdf

111014_075138.sdf

111014_075708.sdf

111014_080254.sdf

111014_080849.sdf

111014_081449.sdf

111014_082027.sdf

111014_082624.sdf

111014_083227.sdf

111014_094357.sdf

111014_095004.sdf

111014_095643.sdf

111014_100332.sdf

111014_101035.sdf

111014_101702.sdf

111014_102404.sdf

111014_103105.sdf

111014_103757.sdf

111014_104443.sdf

111014_105158.sdf

111014_105902.sdf

111014_110603.sdf

111014_111310.sdf

111014_112009.sdf

111014_112652.sdf

111014_113332.sdf

111014_114032.sdf

111014_114727.sdf

111014_115429.sdf

111014_120202.sdf

111018_083123.sdf (calibration)

111018_083301.sdf (calibration)

111018_083511.sdf (calibration)

111018_090012.sdf

111018_090707.sdf

111018_091402.sdf

111018_092053.sdf

111018_092813.sdf

111018_093514.sdf

111018_094229.sdf

111018_094925.sdf

111018_095610.sdf

111018_100309.sdf

111018_101015.sdf

111018_101739.sdf

111018_102509.sdf

111018_103159.sdf

111018_103912.sdf

111018_104444.sdf

111018_105001.sdf

111018_105409.sdf

111018_105711.sdf

111018_110028.sdf

GSL TOOLE BLOCK:

Flown 10/18 
Navigation File:

Remote_GSL_20111018_02.log

Remote_GSL_20111018_03.log

Base Station File:

00052901.pdc

Raw Flightline (LIDAR) Files:

111018_162607.sdf

111018_162924.sdf

111018_163515.sdf

111018_164153.sdf

111018_164648.sdf

111018_165105.sdf

111018_165606.sdf

111018_170029.sdf

111018_170524.sdf

111018_171002.sdf

111018_171442.sdf

111018_171824.sdf

111018_172207.sdf

111018_172553.sdf

111018_173007.sdf

111018_173352.sdf

111018_173800.sdf

111018_174142.sdf

111018_174408.sdf

111018_174806.sdf

111018_175213.sdf

111018_175646.sdf

111018_180113.sdf

111018_180526.sdf

111018_181012.sdf

111018_181440.sdf

111018_181914.sdf

111018_182357.sdf

111018_182845.sdf

111018_183313.sdf

111018_183750.sdf

111018_184220.sdf

111018_184719.sdf

111018_185208.sdf

111018_185644.sdf

111018_190130.sdf

111018_204911.sdf

111018_205408.sdf

111018_205901.sdf

111018_210355.sdf

111018_210851.sdf

111018_211342.sdf

111018_211813.sdf

111018_212307.sdf

111018_212723.sdf

111018_213138.sdf

111018_213549.sdf

111018_213955.sdf

111018_214343.sdf

111018_214713.sdf

111018_215029.sdf

111018_215345.sdf

111018_215658.sdf

111018_220149.sdf

111018_220347.sdf

111018_220702.sdf

111018_221104.sdf

111018_221529.sdf

111018_221930.sdf

111018_222314.sdf

111018_222636.sdf