Cedar Valley, Hurricane Fault, Ogden Valley, and Lowry Water

LiDAR Acquisition

Iron, Emery, Sanpete, Washington,

and Weber Counties, Utah

COMPLETION REPORT

# Table of Contents

Contents

Table of Contents	2

SUMMARY OVERVIEW	4

Executive Summary	4

Contractor	4

Scope Overview	4

Specifications for Deliverables	5

LiDAR Deliverables	5

Grid Model Deliverables	5

Miscellaneous Deliverables	5

Project Area Extents and Project Tile Index	5

LiDAR DATA REPORT	6

Pre-Flight Planning	6

Control	6

Final Planning – Procedures and Activities	7

Planning	7

Summary of Supporting Documents	8

Data Processing Procedures Report	8

Data Storage	8

Navigation System	8

LiDAR System	8

LiDAR QUALITY CONTROL REPORT	11

Methodology	11

Relative Accuracy	11

Within Swath Overlap Accuracy	11

Fundamental Vertical Accuracy	12

Conclusions	13

FLIGHT REPORT	14

GROUND CONTROL REPORT	15

Introduction	15

Data Collection	15

Data Processing and Analysis	15

APPENDIX A – Index Maps and Area Boundaries	18

APPENDIX B – Flight Plan Maps	22

APPENDIX C – Raw Data File Listing	24

Cedar Valley Project: File Listing	24

Ogden Valley Project: File Listing	26

Hurricane Project: File Listing	27

Lowry Project: File Listing	27

# SUMMARY OVERVIEW

## Executive Summary

This project encompasses four areas in five Utah counties and encompasses about 617 square miles shown below. Data was collected in September and October, 2011.

| Study Area | County | Size (mi²) |
| --- | --- | --- |
| Cedar Valley & Hurricane Fault | Iron | 499 |
| Lowry Water | Sanpete and Emery | 59 |
| Ogden Valley | Weber | 59 |
| Total |  | 617 |

## Contractor

This project was completed under contract number UGS110915 between Utah Automated Geographic Reference Center (Utah AGRC) and Utah State University (USU) LASSI Service Center .

Primary technical point of contact information:

Robert T. Pack, Ph.D., P.E

robert.pack@usu.edu

Utah State University

LASSI Service Center

4110 Old Main Hill

Logan, UT 84322-4110

PH 1-435-797-7049

## Scope Overview

Our responsibilities included:

- Flight planning;

- Identification of ground control to be applied as airborne GNSS base stations and for DEM processing;

- Aerial data acquisition;

- Collection of GNSS base station data during flight;

- Collection of GNSS RTK ground data for application in DEM accuracy testing;

- Processing, calibration and classification of LiDAR returns;

- Output of data deliverables including metadata;

- Compilation of Project Completion Report, including Flight, Data Processing and LiDAR DEM Accuracy reporting in compliance with National Standards for Spatial Data Accuracy (NSSDA) guidelines.

## Specifications for Deliverables

The required accuracy and file formats for each delivery was as follows:

### LiDAR Deliverables

Grid Projection:	UTM Zone 12N
Horizontal Datum:	NAD83(CORS96)
Vertical Datum:	NAVD88 using GEOID09
Tile Size:	2000 m X 2000 M
Average Post Spacing:	0.85 m
Average Data Density:	1.37 sh/m2
File Formats:	*.las (v. 1.2)
Classified Datasets:	ASPRS/LAS Default Classes

### Grid Model Deliverables

File Format:	IMG (.img)
Grid Projection:	UTM Zone 12N
Horizontal Datum:	NAV83(CORS96)
Vertical Datum:	NAVD88 using GEOID09
Tile Size:	2000 m X 2000 m
Cell Size:	1.00m

### Miscellaneous Deliverables

Breakpoint Files:	LAS 1.2 (.las) on specific code
Metadata Files:	FGDC compliant XML file. (.xml)
Project Tile Index:	Portable Document Format (.pdf)
Completion Report:	Portable Document Format (.pdf)

LiDAR data acquisition was performed using a Riegl LMS Q560 airborne laser sensor system capable of up to a maximum 200 kHz pulse repetition rate and collection of full waveform returns.

## Project Area Extents and Project Tile Index

The tile layout and project extents for the five areas surveyed are provided in Appendix A. The number of tiles is summarized in Table 1.

Table 1. Project areas.

| Area | Number of Tiles |
| --- | --- |
| Cedar Valley | 385 |
| Hurricane Fault | 61 |
| Ogden Valley | 65 |
| Lowry Water | 59 |

Tiles were designed on a 2000 m by 2000 m grid and were automatically generated.

# LiDAR DATA REPORT

## Pre-Flight Planning

Table 2 provides a list of flight block areas and associated flightline statistics:

Table 2. Flight block areas.

| Area | Flight AGL (m) | Spacing (m) | Number of Lines | Line Kilometers |
| --- | --- | --- | --- | --- |
| Cedar Valley | 750 | 460 | 201 | 2396 |
| Hurricane Fault | 750 | 460 | 6 | 116 |
| Ogden Valley | 750 | 460 | 44 | 281 |
| Lowry Water | 750 | 460 | 22 | 334 |

Table 3 provides the pre-flight mission parameters for the aircraft and laser scanner:

Table 3. Pre-flight mission parameters.

The flight plans for the subject areas are found in Appendix B.

## Control

The area surrounding the study area was searched for candidate vertical control monuments over which the GNSS ground station could be placed. The goal was to tie to A- or B-order vertical control, while at the same time, be within 10 km of the study area. Table 4 provides a list of ground control stations used for this project.

The benchmarks were selected on the basis of (1) vertical accuracy, (2) accessibility, and (3) security for targets and the GPS base station. Five GPS base stations were established. One station in Ogden Valley (WOLF) and Lowry Water (SPRING) were occupied for most of a day each. Q 376 and H 28 GPS stations in Cedar Valley were occupied about 2 days each and C364 on Hurricane Fault for most of one day. This enabled the calculation of strong static GPS solutions which have been compared with the published vertical coordinates.

At each of the stations, 5-foot diameter white circular targets were established, an example of which is shown in Figure 1 for station Y 351.  The surface of each target was leveled using a five foot long construction level.  The target height was then determined using an automatic level. This was done using a back-sight to the monument and a fore-sight to the table surface (see Figure 1). The accuracy of the target height relative to the monument was consistently within about 1 cm. All ten targets were scanned by the lidar in at least one flightline.

The GPS base stations were set up directly over the given monument (with the exception of monument C 364 near LaVerkin, Utah) and the height to the antenna measured within 1 mm.  In the case of Station C 364, the monument was at the base of a cliff in an area of poor GPS satellite visibility. A steel pin was therefore established for the GPS station and the pin’s height was accurately measured relative to the control point using an automatic level. This was used to compare calculated coordinates with published coordinates. In order to make proper comparisons, the heights measured at previous dates needed to be adjusted according to observed HTDP point velocities published by NGS for nearby CORS stations. These points were thereby brought up to date.

Figure 1. Example of lidar target along with equipment used to level its height relative to a nearby benchmark.

## Final Planning – Procedures and Activities

### Planning

Weather forecasts and project schedule identified an aerial acquisition window during the months of September and October 2011. Prior to each acquisition campaign, the following was completed:

- Brief flight crew and ground support personnel on project requirements

- Investigate PDOP forecast for location (Flights to be conducted with PDOP below 3.0)

- Decision to mobilize Bob Pack to site to set up targets and GNSS base stations.

- Complete a reconnaissance of the project area was conducted to report on ground conditions.

It was planned such that each time the aircraft was mobilized out of Logan, Utah each of the four areas could be completed during a single block of days.  The exception to this rule was made for the Lowry Water area where after the first flight, it was determined that two canyons required supplemental data. As second flight was therefore planned for October 24, 2011.  Fortunately, the areas requiring the reflight were snow free on that date.

### Summary of Supporting Documents

- CV NGS DATASHEETS.htm– NGS Data Sheets NGS benchmarks used

- PDOP Plots subdirectory – contains PDOP forecasts for periods of data acquisition.

(The above listed documentation is provided in softcopy format only.)

## Data Processing Procedures Report

### Data Storage

After each flight, all raw navigation data, raw LiDAR data, raw image data, coverage data, and flight logs were off-loaded to a computer and an additional backup storage copy created.

### Navigation System

The airborne GNSS data were processed from the five base station locations using GrafNet software from NovAtel.  Data was also collected from nearby International GPS Service for Geodynamics (IGS) stations for the periods of the flight. Airborne GNSS data was processed based on the ITRF05 Ellipsoid model.

The computed trajectories and the base station coordinates were used in the processing of the IMU data using Inertial Explorer from Waypoint. A smoothed trajectory was produced with error estimates based on the separation between trajectories processed forward and backward in time. The trajectory files were then transformed to the NAD83(CORS96) and NAVD88(GEOID09) project datum and the UTM Zone 12N projection for use in the LiDAR processing.

### LiDAR System

LiDAR waveform files were analyzed using RiAnalyze software to discriminate data points. These points are output in the internal coordinate system of the LiDAR scanner. Each data point is assigned an echo value so it can be used in point classification work. RiProcess then uses the trajectory files created from the raw navigation data to generate XYZ points in a world coordinate system. A boresight calibration and strip (single scan line) adjustment was performed in RiProcess to improve data accuracy. This project’s data were processed in strip form, meaning each flight line was processed independently.  Processing the lines individually provides the data analyst with the ability to quality control (QC) the overlap between lines. To assess trajectory integrity, individual flight strips were then checked against adjacent strips to ensure good matching in the dataset.

A custom strip overlap adjustment method has been developed that not only optimize the lidar system calibration but also correct GPS/IMU navigation errors manifested within individual strips. This method corrects for aircraft roll and aircraft altitude error detected by analyzing elevation differences in all overlapping strips simultaneously. Figure 2 shows an example color-coded map of overlapping regions where blue equals a -10 cm difference, cyan a -5 cm difference, green 0 cm, yellow +5 cm, and red +10 cm.  Figure 3 shows the same series of strips after adjustment. Because the center of the overlap zone is where adjacent strips are mosaicked via a mosaic line, it is important that these lines are consistently green. As shown in Figure 3 this is the case for all strips which results in smooth contouring across the entire project. This wouldn’t have been the case using traditional methods that ignore within-strip errors associated with the GPS/IMU system.

Figure 2. Overlap data prior to within-strip correction, colored by elevation difference (blue = -10 cm, cyan = -5 cm, green = 0 cm, yellow = +5cm, red = +10 cm).

Mosaic Lines

Figure 3. Overlap data after the within-strip correction, colored by elevation difference (blue = -10 cm, cyan = -5 cm, green = 0 cm, yellow = +5cm, red = +10 cm).

Each flightline (strip) was then brought into TerraScan (by Terrasolid) in the project datum and coordinate system. These flightlines were then combined and several classification routines, customized for the given terrain and vegetation, were then run to classify the points into standard ASPRS/LAS default classifications.

Significant effort was given to the creation of automated routines that would detect the dozens of river banks and hundreds of lake shorelines within the subject areas. The routine then automatically creates polylines that then serve as breaklines for hydro-flattening.  For this work, custom tools were developed using LAS-tools, a set of routines developed by Martin Isenburg (out of Germany), and custom Matlab scripts developed in-house. These breaklines, consisting of a series of closely spaced points were then added to the point cloud LAS files with a unique classification code. When combined in a LAS file with original lidar points, the quality of the hydro-flattening can immediately be exploited as a triangulated irregular network (TIN) in any LAS viewer or GIS system (such as ArcGIS).

Using the point classifications and breakline points, digital elevation models (DEMs) of the bare earth and digital surface models (DSMs) of all points were generated for each tile and carefully checked for data quality assurance.

# LiDAR QUALITY CONTROL REPORT

## Methodology

The QC check was intended to ensure that data would meet contractual standards set in FEMA (2003, Section A.8) and USGS NGP Guidelines v.13 (2010). Following is a summary of their standards for RMSEz:

| RMSEz | Condition | Source |
| --- | --- | --- |
| 7.0 cm | Relative accuracy within individual swaths | USGS |
| 10.0 cm | Within swath overlap regions | USGS |
| 12.5 cm | Fundamental vertical accuracy (in the clear) | USGS |
| 18.5 cm | Under all major vegetation categories in flat areas | FEMA |
| 37.0 cm | Under all major vegetation categories in hilly areas | FEMA |

### Relative Accuracy

Relative DEM accuracy was checked for the four typical terrain types within this project using RTK GPS surveys. Table 5 shows the results for these areas. The results show a relative accuracy of 2 cm within the Cedar City Airport runway and tarmac area. This is well under the 7.0 cm specification required by the contract. Within relatively flat sagebrush terrain, a relative accuracy of 2.8 cm was achieved and in bouldery hillsides and areas treed by cedars, a relative accuracy of 8 to 9 cm was measured. This is again within the required specifications.

Table 5. Relative accuracy checks.

### Within Swath Overlap Accuracy

Table 6 shows the mean and RMSEz difference between all DEM cells within overlapping regions. These statistics were calculated by custom Matlab scripts in USU’s custom adjustment software. Table 6 shows that systematic shifts within a given overlap region are less than 1 cm. In areas of sparse vegetation, the RMSEz between overlapping surfaces is about 7.2 cm and in the forested Ogden Valley area, the RMSEz is 14.7 cm. This relatively high value is likely caused by difference in DEM interpolation under the forest canopy. This interpolation can vary due to occlusion patterns that depend on scan angle. In open areas, the RMSEz is much less than the average shown. These results are within the required specifications.

Table 6. Mean and RMSEz difference between DEM cells within overlapping regions.

### Fundamental Vertical Accuracy

It was proposed and accepted by AGRC that a series of 5’ diameter LiDAR targets be used as a spot checks for fundamental vertical accuracy relative to a selection of know brass bench marks distributed around the subject area. The strategy was to place these targets prior to the flights and measure their height using the lidar results such that they could be compared to independently leveled heights measured in the field relative to the brass bench marks.  Table 7 shows the results of this work for bench marks occupied by long GPS static observations associated with the lidar collection. The results indicate an average fundamental vertical accuracy of 4.7 cm for the four targets relative to the published bench mark elevations.  It should be noted that the average difference between the GPS static measurements and the published elevations is 3.5 cm. These results indicate the fundamental vertical accuracy is well within specifications required for this project.

Table 7. Fundamental vertical accuracy as determined at four lidar target locations with strong vertical control.

Six targets were also placed near benchmarks that were not occupied by our static GPS surveys. These differences were found to average 4.9 cm as shown in Table 8. These result suggest that adjustments of the lidar data by up to 6 cm may be necessary in order to match local datums in the various areas surveyed.

Table 8. Vertical accuracy as determined relative to benchmarks with various vertical accuracies.

Horizontal positional accuracy was not formally tested in this project and was not a specification of this contract.

## Conclusions

Given results given above, the following can be concluded:

- There is a tested < 3 cm RMSEz relative accuracy,

- There is a tested < 8 cm RMSEz overlap accuracy except in forested areas where the accuracy is <15 cm due to interpolation differences caused by occlusions, and

- There is a tested < 5 cm RMSEz fundamental vertical accuracy.

# FLIGHT REPORT

USU’s Cessna 208B Skywagon remote sensing aircraft, N4630F, based out of Logan, Utah was utilized on this project.  This aircraft was mobilized out of Logan Municipal Airport, Utah. The actual local flight times and duration of flights were controlled by weather, fuel consumption of the aircraft on the commute from Logan, Utah, and safety of flight operations in this mountainous region. This limited our flexibility in planning for times when the GNSS constellation was most favorable thereby producing the highest number of satellites visible in the best geometric configuration relative to the GNSS receivers onboard the aircraft as well as at the base station on the ground.

Ordinarily two flights were performed per day, weather permitting.  Flights originated from Logan, Utah for Cedar Valley and the two Lowry Water flight. Some gaps in the data associated with the first Lowry Water flight in September were noted.  A second flight was therefore planned and completed when enroute to Cedar Valley on October 24, 2011. Flights originated from Cedar City for the Cedar Valley and Hurricane Fault areas. Flight durations varied between 3 and 4 hours.  At the beginning or end of most days, a calibration flight pattern was flown over either the USU campus or a part of Cedar City. This enabled the improvement of IMU to Lidar alignment which has a tendency to drift in virtually every lidar system.

Because of limitations associated with weather and impending snow, the upland areas of Cedar Valley were flown at night on October 24 & 25, 2011.

The flight dates are summarized in Table 9.

| Block | Dates |
| --- | --- |
| Ogden Valley | 23 September 2011 |
| Lowry Water | 26 September 2011 |
| Lowry Reflight | 24 October 2011 |
| Cedar Valley | 24-27 October 2011 |
| Hurricane Fault | 26 October 2011 |

Navigation File(s):

A listing of GPS base station files and raw flightline (LiDAR) files is given in Appendix C.

# GROUND CONTROL REPORT

## Introduction

Ground control was in and near the project area in support of the lidar work. This report summarizes the results.

Table 10 provides a list of coordinates for each of the 11 bench marks used in this study. The benchmarks listed with a bold font were used as static GPS stations and were occupied during the lidar flights. Stations identified with an asterisk were used as base stations for RTK surveys subsequent to the flights.

Table 10. List of benchmarks used in the five subject areas.

## Data Collection

Using physical descriptions of benchmark locations, each of the 11 stations were occupied, some used for static GPS observations, some used for RTK data collections and all of which were used for lidar target analysis. The static observations were made with a NovAtel dual-frequency GPS receiver. RTK measurements were made with a Topcon GR-5 GNSS (including GLONASS) base/rover pair.

## Data Processing and Analysis

Processing steps performed at each benchmark include ellipsoid to orthometric height conversion, horizontal time-dependent processing of point velocities for epoch adjustment, and target leveling relative to the benchmarks.  Static GPS solutions are disclosed for those points occupied and lidar shot elevations have been compiled for each of the targets. A summary of these processing results is given in Tables 11 and 12.

Table 11. Ground control computations.

Table 12. Ground control computations (continued).

# APPENDIX A – Index Maps and Area Boundaries

Lowry Water

# APPENDIX B – Flight Plan Maps

Cedar Valley

N

Hurricane Fault

N

Ogden Valley

N

Lowry Water (includes reflight area)

N

# APPENDIX C – Raw Data File Listing

Cedar Valley Project: File Listing
Flown:  10/24/2011-10/27/2011

Navigation File(s):

Remote_CedarVAlley_20111024_01

Remote_CedarValley_20111024_02

Remote_CedarValley_20111025_01

Remote_CedarValley_20111025_02

Remote_CedarValley_20111026

Remote_CedarValley_20111027_01

Remote_CedarValley_20111027_02

Base Station File(s):

00052950.pdc

00052951.pdc

00052960.pdc

00052961.pdc

00052980.pdc

00052981.pdc

00052950.pdc

00052951.pdc

00052960.pdc

00052961.pdc

00052990.pdc

00052991.pdc

00052992.pdc

00052993.pdc

Raw Flightline (LIDAR) File(s):

111024_204440.sdf

111024_204950.sdf

111024_205551.sdf

111024_210058.sdf

111024_210427.sdf

111024_210802.sdf

111024_211347.sdf

111024_211822.sdf

111024_212319.sdf

111024_212828.sdf

111024_213439.sdf

111024_213625.sdf

111024_214503.sdf

111024_215019.sdf

111024_215835.sdf

111024_220218.sdf

111024_220611.sdf

111024_220933.sdf

111024_221309.sdf

111024_221613.sdf

111024_221854.sdf

111024_222103.sdf

111024_222824.sdf

111024_223102.sdf

111024_224101.sdf

111024_224626.sdf

111024_225201.sdf

111024_225746.sdf

111024_230404.sdf

111025_011508.sdf

111025_012212.sdf

111025_012811.sdf

111025_013449.sdf

111025_014015.sdf

111025_014538.sdf

111025_015005.sdf

111025_015453.sdf

111025_015933.sdf

111025_020350.sdf

111025_020809.sdf

111025_021240.sdf

111025_022132.sdf

111025_022330.sdf

111025_022614.sdf

111025_022846.sdf

111025_023145.sdf

111025_023435.sdf

111025_023730.sdf

111025_161936.sdf

111025_162316.sdf

111025_162652.sdf

111025_163032.sdf

111025_163357.sdf

111025_163743.sdf

111025_164108.sdf

111025_164420.sdf

111025_164728.sdf

111025_165041.sdf

111025_165705.sdf

111025_170016.sdf

111025_170331.sdf

111025_170725.sdf

111025_171125.sdf

111025_171558.sdf

111025_171903.sdf

111025_172020.sdf

111025_172539.sdf

111025_173212.sdf

111025_173731.sdf

111025_174635.sdf

111025_175031.sdf

111025_175458.sdf

111025_180011.sdf

111025_180520.sdf

111025_181011.sdf

111025_181445.sdf

111025_181920.sdf

111025_182348.sdf

111025_182741.sdf

111025_183024.sdf

111025_183300.sdf

111025_184216.sdf

111025_184655.sdf

111025_185129.sdf

111025_185626.sdf

111025_190057.sdf

111025_190453.sdf

111025_190825.sdf

111025_211820.sdf

111025_211955.sdf

111025_212141.sdf

111025_213537.sdf

111025_214022.sdf

111025_214553.sdf

111025_215032.sdf

111025_215619.sdf

111025_220059.sdf

111025_220635.sdf

111025_221100.sdf

111025_221625.sdf

111025_222111.sdf

111025_222628.sdf

111025_223108.sdf

111025_223628.sdf

111025_224936.sdf

111025_225426.sdf

111025_230138.sdf

111025_230727.sdf

111025_231427.sdf

111025_232009.sdf

111025_232658.sdf

111025_233255.sdf

111025_233939.sdf

111025_234546.sdf

111026_000324.sdf

111026_000941.sdf

111026_001541.sdf

111026_002156.sdf

111026_002742.sdf

111026_003403.sdf

111026_003940.sdf

111026_004514.sdf

111026_005041.sdf

111026_005612.sdf

111026_010135.sdf

111026_010654.sdf

111026_011211.sdf

111026_214818.sdf

111026_215015.sdf

111026_215240.sdf

111026_215556.sdf

111026_215926.sdf

111026_220330.sdf

111026_220725.sdf

111026_221120.sdf

111026_221945.sdf

111026_222351.sdf

111026_222811.sdf

111026_223252.sdf

111026_223801.sdf

111026_225551.sdf

111026_230102.sdf

111026_230601.sdf

111026_231118.sdf

111026_231628.sdf

111026_232141.sdf

111026_232700.sdf

111026_233253.sdf

111026_233851.sdf

111026_234532.sdf

111026_235137.sdf

111026_235848.sdf

111027_000529.sdf

111027_001339.sdf

111027_001512.sdf

111027_001707.sdf

111027_001951.sdf

111027_002117.sdf

111027_002535.sdf

111027_002802.sdf

111027_003507.sdf

111027_004242.sdf

111027_005020.sdf

111027_005825.sdf

111027_010557.sdf

111027_011408.sdf

111027_012130.sdf

111027_012959.sdf

111027_013805.sdf

111027_170640.sdf

111027_170832.sdf

111027_171055.sdf

111027_171343.sdf

111027_172249.sdf

111027_173105.sdf

111027_173912.sdf

111027_174718.sdf

111027_175547.sdf

111027_180401.sdf

111027_181230.sdf

111027_182033.sdf

111027_182833.sdf

111027_183620.sdf

111027_184402.sdf

111027_185123.sdf

111027_185846.sdf

111027_190559.sdf

111027_191357.sdf

111027_192051.sdf

111027_192748.sdf

111027_193412.sdf

111027_194026.sdf

111027_194643.sdf

111027_195246.sdf

111027_195846.sdf

111027_200453.sdf

111027_201104.sdf

111027_201706.sdf

111027_202308.sdf

111027_202924.sdf

111027_203554.sdf

111027_204242.sdf

111027_204911.sdf

111027_205611.sdf

111027_210232.sdf

111027_210940.sdf

111027_221650.sdf

111027_222613.sdf

111027_223056.sdf

111027_223517.sdf

111027_223930.sdf

111027_224319.sdf

111027_224708.sdf

111027_225035.sdf

111027_225412.sdf

111027_225717.sdf

111027_230020.sdf

111027_230720.sdf

111027_231231.sdf

111027_231818.sdf

111027_232331.sdf

111027_232644.sdf

111027_232945.sdf

111027_233408.sdf

111027_234145.sdf

111027_234836.sdf

111027_235400.sdf

111027_235841.sdf

111028_000351.sdf

111028_000946.sdf

111028_001325.sdf

111028_001715.sdf

111028_002055.sdf

111028_002455.sdf

111028_002815.sdf

111028_003155.sdf

111028_003445.sdf

111028_003758.sdf

111028_003951.sdf

111028_004616.sdf

111028_004841.sdf

111028_005314.sdf

111028_005834.sdf

111028_010407.sdf

111028_010953.sdf

111028_011537.sdf

111028_012138.sdf

111028_012747.sdf

111028_013401.sdf

111028_013925.sdf

111028_014541.sdf

111028_015116.sdf

111028_015556.sdf

111028_020030.sdf

111028_020449.sdf

111028_020913.sdf

111028_021326.sdf

111028_021804.sdf

111028_022155.sdf

Ogden Valley Project: File Listing
Flown:  9/23/2011

Navigation File(s):

Remote_OgdenValley_20110923_01

Remote_20110923_02

Base Station File(s):

BaseStation_20110923.pdc

Raw Flightline (LIDAR) File(s):

110923_162924.sdf

110923_163033.sdf

110923_163251.sdf

110923_163836.sdf

110923_164128.sdf

110923_164455.sdf

110923_164900.sdf

110923_165224.sdf

110923_165700.sdf

110923_165947.sdf

110923_170538.sdf

110923_171154.sdf

110923_171813.sdf

110923_172445.sdf

110923_173127.sdf

110923_173731.sdf

110923_174354.sdf

110923_174926.sdf

110923_175417.sdf

110923_175910.sdf

110923_180348.sdf

110923_180809.sdf

110923_181436.sdf

110923_181843.sdf

110923_182213.sdf

110923_182508.sdf

110923_182806.sdf

110923_183015.sdf

110923_183233.sdf

110923_184404.sdf

110923_184611.sdf

110923_184912.sdf

110923_211901.sdf

110923_212303.sdf

110923_212628.sdf

110923_213014.sdf

110923_213352.sdf

110923_213745.sdf

110923_214735.sdf

110923_215053.sdf

110923_215353.sdf

110923_215700.sdf

110923_215948.sdf

110923_220158.sdf

110923_220414.sdf

110923_220646.sdf

110923_220910.sdf

110923_221158.sdf

110923_221443.sdf

110923_221749.sdf

Hurricane Project: File Listing
Flown:  10/26/2011

Navigation File(s):

Remote_HurricaneFault_20111026

Base Station File(s):

00052990.pdc

00052991.pdc

00052992.pdc

Raw Flightline (LIDAR) File(s):

111026_183901.sdf

111026_184840.sdf

111026_185709.sdf

111026_190343.sdf

111026_191721.sdf

111026_193443.sdf

111026_194315.sdf

111026_195240.sdf

Lowry Project: File Listing
Flown:  9/26/2011 & 10/24/2011

Navigation File(s):

Remote_20110926_01

Remote_20110926_02

Remote_20111024

Base Station File(s):

00052691.pdc

Raw Flightline (LIDAR) File(s):

110926_174831.sdf

110926_175949.sdf

110926_180218.sdf

110926_180716.sdf

110926_181101.sdf

110926_181632.sdf

110926_182126.sdf

110926_182657.sdf

110926_183222.sdf

110926_183750.sdf

110926_184315.sdf

110926_184821.sdf

110926_185332.sdf

110926_185846.sdf

110926_190355.sdf

110926_190912.sdf

110926_191421.sdf

110926_191935.sdf

110926_192435.sdf

110926_192947.sdf

110926_193500.sdf

110926_194205.sdf

110926_194744.sdf

110926_195235.sdf

110926_195741.sdf

110926_200139.sdf

110926_200459.sdf

110926_200727.sdf

110926_224632.sdf

110926_224831.sdf

110926_225115.sdf

111024_175151.sdf

111024_175528.sdf

111024_175734.sdf

111024_180001.sdf

111024_180227.sdf

111024_180456.sdf

111024_180822.sdf

111024_181153.sdf

111024_181601.sdf

111024_181931.sdf