Elevation QA Review Report

Project:  Date:

Reviewer:    Total Square Miles Reviewed:

Elevation Type:     Format:       Grid Spacing:    Tile size:

Projection:       Zone:       Datum:         Units:

Licensing:           Metadata:       Min Elev:     Max Elev:

Materials Received:

Vertical Accuracy Test Performed:           Test Point Source:            RMSE:

Vertical Accuracy Test Notes:

QA Review Summary:

Accuracy test compiled from first order or better horizontal and  third order or better vertical  from the NGS control point database. 2 Meter Data

| Vertical Accuracy Statistic Worksheet |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| elevations units in meters |  |  |  |  |  |  |  |
| pt number |  |  |  | z (independent) | z (test) | diff in z | (diff in z)2 |
| 1 |  |  |  | 1373.679 | 1373.78 | -0.10 | 0.01 |
| 2 |  |  |  | 1520.112 | 1520.08 | 0.03 | 0.00 |
| 3 |  |  |  | 1374.118 | 1374.02 | 0.10 | 0.01 |
| 4 |  |  |  | 1798.002 | 1797.74 | 0.26 | 0.07 |
| 5 |  |  |  | 1542.528 | 1542.53 | 0.00 | 0.00 |
| 6 |  |  |  | 1499.575 | 1499.56 | 0.02 | 0.00 |
| 7 |  |  |  | 1590.018 | 1590.10 | -0.08 | 0.01 |
| 8 |  |  |  | 2093.795 | 2093.24 | 0.56 |  |
| 9 |  |  |  | 1656.139 | 1656.43 | -0.29 | 0.09 |
| 10 |  |  |  | 2641.032 | 2641.65 | -0.61 |  |
| 11 |  |  |  | 1571.438 | 1571.41 | 0.03 | 0.00 |
| 12 |  |  |  | 1458.711 | 1458.63 | 0.09 | 0.01 |
| 13 |  |  |  | 1287.596 | 1287.60 | 0.00 | 0.00 |
| 14 |  |  |  | 1386.357 | 1386.28 | 0.08 | 0.01 |
| 15 |  |  |  | 2227.387 | 2227.37 | 0.02 | 0.00 |
| 16 |  |  |  | 2144.433 | 2141.39 | 3.04 |  |
|  |  |  |  |  |  | sum | 0.20 |
| OutLier: a control point located in an area that does not reflect |  |  |  |  |  | average | 0.01 |
| the true ground elevation, i.e. bridges, etc. |  |  |  |  |  | RMSE | 0.11 |
|  |  |  |  |  |  | NSSDA | 0.22 |

Salt Lake City_UT_2 Meter.xls

| Item # | Deliverable type | Area | X1 | Y1 | Tile Number | Sample File |
| --- | --- | --- | --- | --- | --- | --- |
| 501 | Lidar | Utah 2 Meter | 431714.4 | 4525479.9 | 12TVL300240,320240.ASC | 12TVL300240,320240.ASC_TIN ARTIFACTS.jpg |

Utah_2 Meter data received

12TVL300240,320240.ASC_TIN ARTIFACTS