Horizon’s, Inc.

# LiDAR Acquisition/Processing/Quality Control

13582 Utah 1 Meter Area

#### LiDAR Acquisition

LiDAR Acquisition.  LiDAR acquisition for this project was performed by  in a  equipped with an LH System  LiDAR system including an inertial measuring unit (IMU) and a dual frequency GPS receiver.  Acquisition was accomplished between 11/16/2006 and 12/7/2006. Maps identifying the flight lines and LiDAR collection log sheets containing information regarding flights, times, conditions, etc are included in the Appendix.  The LiDAR flight specifications follow:

| ### Field of View | 20 Degrees |
| --- | --- |
| Altitude | 3500 feet AMT |
| Scan Rate | 56 Hz |
| Pulse Rate | 70000 kHz |
| Airspeed | 140 knots |
| Average Post Spacing | 1.0 meters |
| Average Swath Width | 1427 feet |
| Intensity Mode | 4+3 |
| Total Flight Lines: | 85 |
| Number of Lifts | 4 |

# GPS Data Collection

GPS Data Collection.  A GPS receiver was in constant operation over a  control point during flight acquisition.  The base station identification and location are listed in the following table. Base station information is also included in the Appendix.

| # Station | # Location |
| --- | --- |
| # LO0622 | # 40 37 05.78236 # 111 59 47.19312 |

# GPS base station

# GPS Data Processing

GPS Data Processing. All GPS phase data was post processed with continuous kinematic survey techniques using “On the Fly” (OTF) integer ambiguity resolution.  The GPS data was processed with forward and reverse processing algorithms.  The results from each process, using the data collected at the airport, were combined to yield a single fixed integer phase differential solution of the aircraft trajectory.  Plots of altitude and the forward and reverse GPS solution residuals are included in the Appendix. The differences between the forward to reverse solution within the project area were within project specifications (<10cm) in both the horizontal and vertical components, indicating a valid and accurate solution.

# IMU Data Processing

IMU Data Processing. An IMU was used to record precise changes in position and orientation of the LIDAR scanner at a rate of 200 Hz.  All IMU data was processed post flight with a filter to integrate inertial measurements and precise phase differential GPS positions.  The resulting solution contains geodetic position, omega, phi, kappa, and time for subsequent merging with the laser ranging information.LiDAR Data Processing

LIDAR Data Preprocess

Flight Line Data Acquisition/Quality Control Check.  LiDAR data and the IMU files were processed together using LIDAR processing software.  The data set for each flight line was checked for project area coverage, data gaps between overlapping flight lines, and tension/compression areas (areas where data points are more or less dense than the average project specified post spacing). Based on this check it appears the entire project area is covered without gaps. LiDAR Coverage Check and Line Overlap Check Diagrams are included in Appendix.

Boresighting Process.  Pre-processing of LiDAR corrects for rotational, atmospheric, and elevation differences that are inherent to LiDAR data sets. This process is called boresighting. LiDAR data was collected for bi- and cross-directional flight lines over the project area. Using an iterative process that involves analyzing raster difference calculations the Omega, Phi, Kappa angle corrections of the LiDAR instrument were determined. The corrections were applied to the LiDAR data set for the project area.

Vertical Accuracy Check. Extensive comparisons were made of vertical and horizontal positional differences between points common to two or more LiDAR flight lines. This was done for the airport bore-sight testing area and the project area. All flight lines (airport and project) were within project specifications for vertical accuracy.

LiDAR Intensity Check.  An intensity raster for each flight line was generated.  The raster was checked and verified that intensity was recorded for each LiDAR point.

Project Coordinate System.  LiDAR preprocessing software outputs data to its corresponding UTM zone in meters and a GRS80 ellipsoidal height.  LiDAR data was transformed to a project coordinate system of   NAD83 NAVD88 Meters UTM zone 12 Geoid 03 .

Vertical Bias Correction.  LiDAR has a consistent vertical offset.  LiDAR ground points were compared to independently surveyed and positioned ground control points in the project area.  Based on the results of these comparisons, the LiDAR data was vertically biased down to the ground.

Project Ground Control Check. Comparisons between on-site ground survey control points and LiDAR data yielded the following results:

1 Meter Area

| ### Vertical Accuracy (RMSE) | 0.086 |
| --- | --- |
| Standard Deviation | 0.090 |
| Mean Difference | 0.000 |
| Number of Points | 11 |

LiDAR Data Surfacing Process.

Raw LiDAR Data Set.  LiDAR data in overlap areas of project flight lines was removed and data from all swaths was merged into a single data set.  The data set was trimmed to the digital project boundary including an additional buffer zone (buffer zone assures adequate contour generation from the DEM).  Resulting data set is the Raw LiDAR data. The Raw LiDAR data set was processed through a minimum block mean algorithm and points were classified as either bare earth or non-bare earth.  User developed “macros” that factor mean terrain angle and height from the ground, were used to determine bare earth point classification.

LiDAR Surfacing Process.  The surfacing process is a 2D-edit procedure that ensures the accuracy of the automated feature classification. Editors used a combination of imagery, intensity of the LiDAR reflection and tin-editing software to assess points.  The resulting data set is 2D Surfaced Bare Earth. The LiDAR data is filtered using a quadric error metric to remove redundant points. This method leaves points where there is a change in the slope of surfaces (road ditches) and eliminates points from evenly sloped terrain (flat field) where the points do not affect the TIN LiDAR data. The resulting data set is 2D Surfaced/Filtered Bare Earth

Appendix Items.

Flight Map 1 Meter Area

LiDAR Collection Logsheet

1 Meter Area Lift 18 Page 1

Diagram – GPS Solution Residuals

F and R combined 1 Meter Area Lift 18

F and R RMS 1 Meter Area Lift 18

Height 1 Meter Area Lift 18

PDOP 1 Meter Area Lift 18

Diagram – Aircraft Trajectory Plot

Trajectory 1 Meter Area Lift 18

Diagram – LiDAR Coverage Check for Project Lines

LiDAR Coverage Check 1 Meter Area Lift 18

LiDAR Collection Logsheet

1 Meter Area Lift 20 Page 1

1 Meter Area Lift 20 Page 2

Diagram – GPS Solution Residuals

F and R combined 1 Meter Area Lift 20

F and R RMS 1 Meter Area Lift 20

Height 1 Meter Area Lift 20

PDOP 1 Meter Area Lift 20

Diagram – Aircraft Trajectory Plot

Trajectory 1 Meter Area Lift 20

Diagram – LiDAR Coverage Check for Project Lines

LiDAR Coverage Check 1 Meter Area Lift 20

LiDAR Collection Logsheet

1 Meter Area Lift 21 Page 1

Diagram – GPS Solution Residuals

F and R combined 1 Meter Area Lift 21

F and R RMS 1 Meter Area Lift 21

Height 1 Meter Area Lift 21

Diagram – LiDAR Coverage Check for Project Lines

LiDAR Coverage Check 1 Meter Area Lift 21

LiDAR Collection Logsheet

1 Meter Area Lift 25 Page 1

Diagram – GPS Solution Residuals

F and R combined 1 Meter Area Lift 25

F and R RMS 1 Meter Area Lift 25

Height 1 Meter Area Lift 25

PDOP 1 Meter Area Lift 25

Diagram – Aircraft Trajectory Plot

Trajectory 1 Meter Area Lift 25

Diagram – LiDAR Coverage Check for Project Lines

LiDAR Coverage Check 1 Meter Area Lift 25